3GPP TSG-RAN WG3 Meeting #126	R3-247785

Orlando, USA, 18th – 22nd Nov, 2024

Agenda Item:	11.2

Source:	Lenovo (Moderator)

Title:	SoD of CB AI based Slicing

Document for:	Approval

# 1	Introduction

CB: # AIRAN1_Slicing

- Check the open issues above

- Provide TPs to capture the agreements

(moderator - Lenovo)

Summary of offline disc R3-247785

# 2	Summary for chairman notes

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

Agree TP for 38.423 BLCR: R3-247823

# 3	Discussion

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE? Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node. WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. Turn WA to agreement: UE performance per Slice is reported from target node to source node.

Issue#1:

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message.

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE?

| R3-247442 | (TP to BLCR 38.423) Discussion on AIML based network slicing (Lenovo) | other |
| --- | --- | --- |

For chair note:

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

Issue#2:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

Turn WA to agreement: UE performance per Slice is reported from target node to source node.

To align companies’ understanding:

- Aspect#1: How UE performance per slice granularity can be discussed/decided by RAN3, but has some dependency on how UE performance per UE granularity is calculated which was discussed in Rel-18 correction and an LS to SA5 is triggered, e.g.,

  - Or if it is an average over time among all packets via all DRBs within the same slice/UE, e.g., R3-247412

- AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps

  - Or if it is an average of DRB level metric within the same slice/UE, e.g., R3-247412

- AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps

E///, Huawei, ZTE: SA5 has defined and supported the per slice per UE performance. RAN3 does not need to further discuss.

Nokia, CATT: it is unclear from SA5 spec, how per slice per UE performance is calculated. What’s supported in SA5 spec now is per slice, but not per slice per UE.

For Chair Note:

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

- Aspect#2: Issue raised by some companies is that: transferring UE performance per slice level may cause problem for the source gNB to correctly understand if UE performance is improved/degraded after handover, no matter how it is calculated according to Aspect#1. Same issue exists in Rel18 with UE level performance transfer.

  - Proponent companies propose to support per QoS flow (group) level UE performance transfer (measured per DRB eventually) to resolve the issue.

  - Opponent companies either don’t believe this is a big issue, or don’t believe per QoS flow (group) level UE performance measurement can resolve the issue neither.

Example given in R3-247469

| Definition flavour and sub-flavour | Before handover | After handover (no data on DRB1) |
| --- | --- | --- |
| Simple average, excluding DRBs with no data when averaging | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = N/A DRB2: average delay = 100ms overall average delay = 100ms |
| Simple average, regarding the value as 0 for DRBs with no data | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = 0ms DRB2: average delay = 100ms overall average delay = 50ms |
| Average weighted by number of packets (assume the number of packets of the two DRBs are the same before handover) | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = N/A DRB2: average delay = 100ms overall average delay = 100ms |