3GPP TSG-RAN WG3 Meeting #126	R3-247790

Orlando, USA, 18th–22nd November, 2024

Agenda Item:	11.4

Source:	Ericsson (Moderator)

Title:	SoD on CB: # AIRAN5_ContinousMDT

Document for:	Approval

# 1	Introduction

This SoD is to discuss the following CB:

CB: # AIRAN5_ContinuousMDT

- Check the solutions on the table taken the above principle into account

(moderator - E///)

Summary of offline disc R3-247790

# 1	Summary for chairman notes

Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

Identification of requirements and description of problem statement, followed by LS drafting to be continued.

# 2	Discussion

## 2.1 	Continuous MDT Initiation

During the online discussions on Continuous MDT the following agreement was taken:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

And the following FFS was captured:

Compare the solutions based on this principle.

To provide a complete list of agreements, the following should be recalled from the agreements made at RAN3-125bis:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Further, the two problems that need to be resolved for Continuous MDT are:

-	Problem A (measurement continuity): how to ensure that the same UE is selected for collection of MDT measurements during the same RRC state, across different RRC states and across RRC_Connected mobility.

-	Problem B (trace correlation): how to ensure that the TCE receivingves the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

The agreement above implies that solutions for continuous MDT should:

- Reuse the Management Based MDT framework

- Rely on the OAM to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT

- Be network based and avoid impacts on legacy UEs.

Considering the Continuous MDT solutions presented at this meeting and excluding solutions with UE impact, the following list of solution classes can be derived:

- Solution 1: Based on RAN-OAM interaction.

  - The OAM configures Management Based MDT to the gNBs taking part in Continuous MDT. The configuration includes specific identifiers (e.g. Trace Record) dedicated to the Continuous MDT process, namely the NG-RAN deduces from such identifier that the Management Based configuration is for Continuous MDT.  The RAN configures parameters such as the Trace Record Session Reference, which are reassigned to the UE (in full or in part) across RRC States and RRC_Connected mobility, namely such parameters are assigned/reassigned to the UE throughput the duration of the Continuous MDT session.

  - Measurement continuity is ensured by

    - Including in the Handover signalling an indication that the UE has to be selected for continuous MDT, e.g. the TR/TRSR assigned at source to the Continuous MDT process for the UE

    - Checking the TR/TRSR in the Logged MDT reports signalled by the UE at RRC_Idel/Inactive to RRC_Connected transitions. TR/TRSR reveal that the UE should be selected for Continuous MDT

  - Trace Correlation is achieved by:

    - Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace.

- Solution 2: Based on OAM-RAN-AMF interaction.

  - The OAM configures Management Based MDT at a gNB that selects the UE for Continuous MDT. When UE transits to RRC_Idle, a context related to the MDT configuration is stored by the gNB serving the UE to the Core Network. Upon transition from RRC_Idle to RRC_Connected or after successful handover, the AMF indicates to the second gNB serving the UE that the MDT configuration used for the UE, including an indication that the UE should be selected for management-based immediate MDT. In case of handover, the source gNB sends the MDT context related to the data collection to the target gNB during handover preparation.

  - Measurement continuity is ensured by

    - Receiving at the handover target gNB the MDT configuration previously used for Continuous MDT for the UE (signalled from the source gNB to the target gNB)

    - Receiving at the new serving gNB the MDT configuration previously used for Continuous MDT for the UE (signalled from the AMF to the serving gNB)

  - Trace Correlation is achieved by:

    - Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the Core Network and maintained in the Core Network for the duration of the UE context.

    - The AMF assigned identifier is signalled to the serving gNB as part of the MDT configuration used for continuous MDT. The serving gNB includes such identifier in the Trace Records signalled to the TCE.

- -	Solution 3: Based on OAM-RAN-AMF interaction.

  - Introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE. Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE.
The gNB serving a Continuous MDT UE sends parts of the MDT configuration to the AMF, which stores such information. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration

  - Measurement continuity is ensured by

    - The serving gNB receiving from the AMF information concerning the Continuous MDT configuration for the UE (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR allocated by the old serving gNB) which were previously stored in the AMF/5GC by the old serving gNB, so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

    - Source gNB provides the target gNB with information concerning the Continuous MDT configuration (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB). The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

  - Trace Correlation is achieved by:

    - “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE

- Solution 4: Based on RAN-OAM interaction (Trace Correlation without RAN3 Impact).

Measurement continuity is ensured by similar way proposed by solution 2, e.g., OAM/AMF assign an identifier and stored in CN.

For MDT report:

Immediate MDT report: gNB sends the immediate MDT Records to TCE, including the TR1, TRSR1 of immediate MDT(legacy behavior), and the assigned TR2,TRSR2 for logged MDT when UE goes to idle.

Logged MDT report: gNB sends the received MDT Record to the TCE, including the TR2, TRSR2 of the logged MDT(legacy behavior), and the assigned TR1,TRSR3 for immediate MDT UE will execute during connected state.

  - Measurement continuity is ensured by

    - Similar with solution 2.

  - Trace Correlation is achieved by:

    - Correlation two MDT Record by the “assigned TRx,TRSRx”  in this MDT report.

One first observation about the solutions above is that all the solutions deeply touch on MDT functional aspects, for which SA5 is responsible.

It is proposed to:

- Attempt down filtering of the solutions in RAN3

- Draft an LS to SA5 and ask for feedback on the best solution to follow

QC: we should capture requirements for OAM rather than solutions

HW: Shall the solution discussion be held in RAN3?

QC: we should discuss which requirements to agree, but we shod not send a list of solutions

Nok: RAN3 should work on the solution and eventually consult with SA5

Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

Identification of requirements and description of problem statement, followed by LS drafting to be continued.

## 2.1.1	Initial discussions on solutions down filtering in RAN3

Criteria to evaluate solutions:

- Can the solution provide measurement collection continuity?

- Can the solution be achieved by reusing Management Based MDT? What impact does the solution have on the current Management Based MDT?

- What impact does the solution have on the network functions and nodes?

- What are the solution dependencies from support at network nodes and functions, e.g. which network nodes need to support the feature in order for the feature to work?

It is proposed to agree to the above criteria for an initial solution evaluation in RAN3.