3GPP TSG-RAN WG3 Meeting #126	R3-247803

US, Orlando, 18-22 November 2024

Agenda Item:	20.2

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on CB: # AIPHY

Document for:	Approval

# Introduction

CB: # AIPHY

- Compare the solutions above for training data collection from LMF to gNB

(moderator - E///)

# For the Chairman’s Notes

For case 3a:

WA: RAN3 follows the principle of positioning that LMF starts the NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated.

# Discussion

## 3.1 Data collection signalling

Some solutions have been discussed for data collection over NRPPa to support case 3a.

For example, Xiaomi’s R3-247320 propose re-use existing non-UE associated NRPPa message with LMF indicating data collection is available at first step. Fig.1 is taken from their contribution:

Figure 1 Option 2 configure part A and obtain part B in same message

In QC’s R3-247459, they describe one option where LMF proactively provides a list of available UEs/PRUs in a non-UE associated message. Fig.2 is taken from their contribution:

Figure 2: General data collection procedure for gNB-side model training in Case 3a.

The LMF may provide the list of available UEs/PRUs in an area of interest to the NG-RAN. This may be proactively pushed to the NG-RAN by an LMF (Option 1) or requested by the NG-RAN node on demand (Option 2). In both Options, this can be a new (non-UE associated) NRPPa message, which includes for each UE in the list an LMF assigned deferred Routing ID, which may be used later at Step 4. To enable an AMF to distinguish a Routing ID assigned by an LMF (used in this procedure) from a Routing ID assigned by the AMF (used as currently specified for NRPPa messages), the two types of Routing IDs could be selected from different ranges. If the UE is a stationary PRU, the location of the PRU may be provided in addition

Ericsson’s R3-247578 proposed to introduce Xn’s like data collection procedures over NRPPa (non-UE associated) with a set of class1 and class2 procedures.

Moderator proposes to harmonize the three solutions as below in fig 3:

Fig.3. baseline figure for data collection in case 3a

Proposal 1:

- The LMF proactively indicates that data collection for a list of UEs/TRPs is available via new non-UE associated NRPPa message.

- The gNB determines the data collection needed and sends a request to LMF via new non-UE associated NRPPa message.

- If not already configured, the gNB configures the UE with UL SRS to get part A information. FFS if time stamp or other information (e.g., SRS configuration) can be used to correlate part A and part B info.

CATT: LMF just sends the related labels of the UE served by the gNB?

Nokia: take step 2 as assumption based on information from LMF

E///: the principle of NRPPa transaction initiated by LMF is kept, and ok to capture step 2 as agreement

ZTE: can the LMF know that gNB hosts AI/ML mode. E///: yes

CEWIT: LMF can only provide UE location, not measurements

Hw: the LMF triggers, no LCS client in the gNB, gNB should not trigger anything, AI/ML should accommodate to positioning principle

QC: how to identify the UE from the LMF in step 1? Can data collection only happen when LMF receives data collection request from LMF?

Proposals:

For case 3a:

WA: RAN3 follows the principle of positioning that LMF starts the NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B.

FFS on the protocol used (NRPPa or NGAP). FFS how the UE is identified in the protocol.

## 3.2 Inference output signalling

It was agreed online that:

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

RAN1 agreement stipulates the following parameters:

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

We note these IEs are already present in the TRP Measurement result:

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore.

Proposal 2: RAN3 confirms that the following IEs in the TRP Measurement Result in TS 38.455 can be re-used as per legacy specification:

- Time Stamp IE

- Measurement Quality IE

- LoS/NLoS information IE

Common understanding that above IEs can be re-used in RAN3

It is proposed to discuss whether the specification needs to distinguish if the measurements are AI-ML based in the Measurement procedure:

- Option 1: dedicated IEs (e.g., AI/ML UL-RTOA) in the Measurement request and response message.

- Option 2: new flag, AI/ML Positioning measurement	ENUMERATED (true,…) in the TRP Measurement Result IE

Option 2 is considered more future proof if new AI/ML generated measurements are added and with less spec impacts.

Proposal 3: RAN agree with option 2

FFS if LMF should be aware of AI/ML measurements.

## 3.3 AI/ML Model placement

Considering the proposals from Nokia, CATT, China Telecom,

And to avoid new F1 impact for data collection and model inference output over F1

And that CU has more capacity than DU,

Proposal 4: It is proposed that gNB-CU hosts AI/ML model for positioning measurement inference in case 3a.

## 3.4 TP discussion

Stage 2 TP to TS 38.300 based on Nokia proposal:

# Annex -TP to TS 38.300 ## <<< start of changes >>> ## 16.20 Support of AI/ML for NG-RAN ### 16.20.1 General Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN. The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization. The objective of NG-RAN support for AI/ML assisted positioning is to enable NG-RAN to provide output of Model Inference, e.g., intermediate parameters to the LMF. ### 16.20.2 Principles Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results). NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements from UEs. ## <<< unmodified text skipped >>> ### 16.20.3 Data Collection and Reporting The following information can be configured to be reported by an NG-RAN node: - Predicted resource status information; - UE performance feedback; - Measured UE trajectory; - Energy Cost (EC). The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure. The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover. Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. ### 16.20.x Positioning procedures (name FFS) The following information can be configured to be reported by an NG-RAN node to an LMF: - Positioning intermediate parameters (e.g. LoS indicator, NLoS indicator, timing information). NRPPa procedures are used for the configuration and reporting of intermediate parameters. ## <<< end of changes >>>

Stage 3 TP to TS 38.455, based on CMCC TP (for Case 3b), addition of sample based measurement parameters to the MEASUREMENT REQUEST message:

| >>Sample-based Measurement Parameters | O |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Nt |  |  |  | Window size for measurement collection. |  |  |
| >>>Nt’ |  |  |  | Number of reported measurement samples. |  |  |
| >>>k |  |  |  | Interval between measurement samples. |  |  |

Proposal 5: discuss if above two TPs can be agreed

# Conclusion, Recommendations

# References