# 3GPP TSG-RAN WG4 Meeting # 112      													R4-2412001

# Maastricht, Netherlands, 19th-23th, Aug, 2024

Title: 	Expected changes due to introduction of new bands n87 and n88

Agenda item:	7.15.2

Source:	Nokia

Document for:	Approval

# 1	Background

RAN#104 approved the new WID on introduction of NR bands n87 and n88 [1]. This contribution discussed expected changes for UE specification 38.101-1 due to introduction of bands n87 and n88.

# 2 	Discussion

Expected changes are listed in Table 1 for both bands separately. As a reference LTE band 31 and 72 specifications are listed.

Proposal: Use table 1 as a framework for requirements on new NR bands n87 and n88.

Table 1: expected changes for UE specification 38.101-1 due to introduction of bands n87 and n88

| Requirement | B87 in 36.101 | n87 proposal | B88 in 36.101 | n88 proposal |
| --- | --- | --- | --- | --- |
| Operating bands | 410 MHz – 415 MHz 420 MH z– 425 MHz | Re-use LTE | 412 MHz – 417 MHz 422 MH z– 427 MHz | Re-use LTE |
| Channel bandwidths | 1.4, 3 and 5 MHz | 3 and 5 MHz | 1.4, 3 and 5 MHz | 3 and 5 MHz |
| EARFCN / NR-ARFCN | LTE band specific | Needs n87 specific | LTE band specific | Needs n88 specific |
| Sync-raster | Not in LTE | Needs n87 specific | Not in LTE | Needs n88 specific |
| TX–RX frequency separation | 10 MHz | 10 MHz | 10 MHz | 10 MHz |
| UE maximum output power | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 |
| MPR | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 |
| A-MPR | No A-MPR | Not needed | No A-MPR | Not needed |
| Output power dynamics | Not band specific | Not band specific | Not band specific | Not band specific |
| Transmit signal quality | Not band specific | Not band specific | Not band specific | Not band specific |
| Occupied bandwidth | Not band specific | Not band specific | Not band specific | Not band specific |
| Spectrum emission mask | Not band specific | Not band specific | Not band specific | Not band specific |
| ACLR | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. |
| General spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |
| Spurious emissions for UE co-existence | Is band specific | LTE requirement is a baseline | Is band specific | LTE requirement is a baseline |
| Transmit intermodulation | Not band specific | Not band specific | Not band specific | Not band specific |
| REFSENS power level | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR |
| REFSENS UL allocation | 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content | As an error not specified, maintenance CRs are provided into this meeting 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content |
| Max input level | Not band specific | Not band specific | Not band specific | Not band specific |
| ACS | Not band specific | Not band specific | Not band specific | Not band specific |
| IBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| OBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| NBB | Not band specific | Add n87 into NBB table | Not band specific | Add n88 into NBB table |
| Spurious response | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |
| Wideband intermodulation | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |
| Spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |

# 3	Conclusion

In this contribution we have discussed and proposed UE RF requirement framework for introduction of bands n87 and n88 in Table 1.

# 4	References

- [1] RP-241663 New WID on introduction of NR bands n87 and n88, RAN4#104, Nokia