3GPP TSG-RAN WG4 Meeting #112	R4-2412333

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.16.3

Source:	Intel Corporation

Title:	BS demodulation performance requirements for MMSE-IRC

Document for:	Discussion

# Introduction

A new WI on Demodulation performance requirements enhancements was approved in RAN #104 meeting [1]. The WI targets for improvements for 1) MMSE-IRC UE performance requirements for 8Rx, and 2) MMSE-IRC BS performance requirements. The justification and objectives related to BS MMSE-IRC performance requirements enhancements are provided below [1]:

# 3 Justification In Rel-19, to verify the UE and BS demodulation performance with advanced receiver and apply demodulation requirements to more scenarios of practical network, the following enhancements are identified: - … - BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in neighbouring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system. There is necessity to introduce BS performance requirements for MMSE-IRC advanced receiver under inter-cell interference. # 4 Objective ### 4.2 Objective of Performance part WI … - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution we provide initial views and proposals on MMSE-IRC based PUSCH performance requirements with inter-cell interference.

# Discussion

## Scenarios and interference profiles

The Rel-19 BS MMSE-IRC requirements are expected to be introduced under the following assumptions:

  - NR FR1

  - Homogeneous and heterogeneous deployments

  - FDD synchronous deployments

  - TDD synchronous deployments with aligned UL:DL configurations among cells

  - Slot-based transmission and aligned SCS among cells

The WID suggests reusing LTE interference profiles from TR 36.884 as a starting point, while not excluding the development of new interference profiles. Therefore, it is important to assess the extent to which LTE profiles are applicable for a realistic representation of NR environment.

The LTE interference profiles in TR 38.884 were previously derived for the following deployment scenarios with detailed assumptions provided in the corresponding technical report:

  - Scenario 1: Homogeneous deployment with macro cell only

  - Scenario 2: Heterogeneous deployment with co-channel low power node (LPN) within the macro cell coverage

Overall, the assumptions provided in the TR are relatively aligned with typical NR assumptions and, hence, profiles may serve as a reasonable approximation for the NR interference environment, especially given the similarities in FR1 LTE/NR deployments. However, several factors could result in distinct interference environments for NR:

- Deployment parameters and channel models: Minor differences in typical 3GPP deployment and channel model assumptions between LTE and NR could influence interference, though the impact is likely minimal.

- Channel bandwidth: LTE analysis was based on a 10MHz bandwidth, whereas NR's support of broader bandwidth could affect interference statistics due to UE TX power constraints.

- Traffic model: TR 36.884 assumes full buffer UL transmission, which may not reflect a realistic scenario. More recent LTE/NR analyses incorporate more realistic non-full-buffer traffic models, potentially altering interference statistics.

- UL precoding: The LTE requirements are defined under an assumption of 1Tx transmission, while 2Tx NR UE transmissions may have certain implications on the interference statistics.

- High power UEs: LTE interference profiles were based on a 23 dBm UE output power. The increasing population of high power UEs (HPUEs) in NR could affect overall interference levels.

Given these considerations, further analysis is recommended to determine the suitability of LTE interference profiles for NR and whether adjustments or new profiles are necessary. RAN4 is recommended to further discuss and agree on relevant assumptions for NR interference profiles analysis. The assumptions in TR 36.884 can be taken as a baseline and further modifications to address the aspects above can be considered.

Proposal #1:	Study suitability of LTE interference profiles for NR interference modelling and whether adjustments or new profiles are necessary.

## Interference modelling methodology

From 3GPP requirements perspective there are two generic methodologies for inter-cell interference modelling:

- DIP (Dominant Interfere Proportion): The DIP quantifies the proportion of interference power contributed by the dominant interferer(s) relative to the total interference power from all other cells and noise power. The approach was first introduced as a part of Rel-11 Advanced UE receivers SI and then used for a number of LTE items related to inter-cell interference handling, including advanced LTE BS receivers (Type A and Type B). In accordance with the methodology defined in TS 36.104 the interference is modelled as:

Each inter-cell interferer … is characterized by its associated dominant interferer proportion (DIP) value: DIPi=IoriN´ (i = 1,…, M) where Iori is the received energy from the i-th strongest inter-cell interferer involved in the requirement scenario and N´=j=1MIorj+N where N is the energy of the white noise source consistent with the definition provided in clause 8.1 and M is the total number of simultaneously transmitted inter-cell interferers involved in a given requirement scenario.

- INR (Interference to Noise Ratio): The INR methodology relies on explicit modelling of dominant interfering cell modelling with a fixed power level. The respective methodology was utilized to define requirements in a number of LTE items (CRS-IM, NAICS, etc) and also is used as the basis for NR UE MMSE-IRC requirements. In accordance with the methodology defined in TS 38.101-4 the interference is modelled as:

Each interfering cell involved in PDSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value: INRi=j=1NRXIor(i)(j)j=1NRXNoc(j) where Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc is the average power spectral density of a white noise source consistent with definition provided in section [4.4.3/4.5.3].

Both methodologies model the interference signals explicitly but differ in their representation of interference power and the process for defining requirements. For the DIP methodology the requirements are defined in terms of SINR @ X% of max throughput, with the simulations conducted across a SINR range and with interference power levels varying according to the actual SINR level. In contrast, the INR methodology adopts a more direct approach, defining requirements in terms of the useful signal's SNR @ X% of max throughput and performing simulations across a range of SNR values, while keeping interference power levels constant across the SNR range (while actual SINR will be a function of SNR). Both methods are effective for representing inter-cell interference conditions, but the INR methodology may be more suitable for scenarios with non-full-buffer traffic and provides more flexibility in requirements definition.

The interference profiles in TR 36.884 are specified using DIP methodology and are not directly translatable to INR metrics. If requirements are agreed to be based on the interference profiles from TR 36.884, then the DIP methodology should be retained. However, if new interference profiles are to be established, the INR methodology is recommended for consideration.

Proposal #2:	INR interference modelling methodology shall be used if new NR interference profiles are agreed to be defined. DIP interference modelling methodology shall be used if LTE interference profiles are reused.

## Reference receiver structure

In accordance with the WID the MMSE-IRC receiver shall be used for the PUSCH requirements definition. A detailed description of the standard reference MMSE-IRC receiver for LTE is available in TR 36.884 [2], which corresponds to the LTE BS Enhanced performance requirements Type A:

## 4.2 Baseline receiver: MMSE receiver The MMSE receiver weight matrix is expressed as follow: , where and denote the estimated channel matrix and noise power, respectively. P1 is the transmission power of UE scheduled by the serving cell and is equal to . ## 4.3 MMSE-IRC receiver The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

As it can be observed it is assumed that BS performs channel estimation for the useful signal on a per-RE level, while the interference is estimated on DMRS REs in the form of residual signals covariance matrix with a per PRB and per slot granularity. Given a high similarity of NR and LTE waveforms, the same principles can be applied for NR BS MMSE-IRC reference receiver.

It is assumed that the BS conducts channel estimation for the useful signal at the RE level. Interference is estimated on DMRS REs after subtraction of the useful signal as a residual signal average covariance matrix, with per PRB and per slot averaging granularity. Due to the high similarity between NR and LTE waveforms, these principles are also applicable to the NR BS MMSE-IRC reference receiver.

Proposal #3:	Reuse LTE MMSE-IRC reference receiver assumptions defined in TR 36.884.

## PUSCH requirements

The following framework of PUSCH demodulation requirements is suggested:

- Focus on SU-MIMO requirements. FFS if MU-MIMO shall be defined.

- The demodulation requirements can be based on baseline PUSCH requirements in TS 38.104 clauses 8.2.1/2:

| Transform precoding |  | Disabled FFS: Enabled |
| --- | --- | --- |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

- DMRS configuration: The parameters in TS 38.104 clause 8.2.1, 8.2.2 can be taken as a starting point and additional verification for interference-limited conditions is needed to confirm the setup.

| DM-RS | DM-RS configuration type | 1 |
| --- | --- | --- |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |

- Number of TX antennas: 1, 2, 4

- Number of RX antennas: 2, 4, 8

- Propagation conditions and correlation matrix: All or a subset of TDLB100-400 Low, TDLC300-100 Low, TDLA30-10 Low, TDLA30-10 Low

- FRC: QPSK/16QAM/64QAM

# Conclusion

In this paper we provide initial views on BS demodulation requirements for MMSE-IRC receivers and, in summary, make the following proposals:

Proposal #1:	Study suitability of LTE interference profiles for NR interference modelling and whether adjustments or new profiles are necessary.

Proposal #2:	INR interference modelling methodology shall be used if new NR interference profiles are agreed to be defined. DIP interference modelling methodology shall be used if LTE interference profiles are reused.

Proposal #3:	Reuse LTE MMSE-IRC reference receiver assumptions defined in TR 36.884.

# References

- RP-241297 “Revised WID on NR demodulation performance: Phase 5”, RAN #104, June 2024

- TR 36.884 “Performance requirements of MMSE-IRC receiver for LTE BS”