3GPP TSG-RAN WG4 Meeting # 112	R4-2413466

# Maastricht, Netherlands,19th-23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0614 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4 Corrections on 8Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon, Nokia, ZTE, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The applicability rule of 8Rx cases for 8Rx demodulation requirements are missing in 5.1.1.4. - Some test parameters for Rank 8 (8Tx) in common parameter table(Table 5.2-1) are missing - Table title for Table 5.2A.4.1-3 is missing - Some FRCs and requirements are still with [] - Other typos for test setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the applicability rules of 8Rx cases for 8Rx demodulation requirements in 5.1.1.4 - Add the test parameters for Rank 8(8Tx) in common parameter table(Table 5.2-1) - Add the title for Table 5.2A.4.1-3 - Remove the [] - Correct the typos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The applicability rules are still missing - The parameters for Rank 8 (8Tx) in common parameter table(Table 5.2-1) will still be missing - Table title for Table 5.2A.4.1-3 will still be missing - The FRCs and requirements will still be with [] - The typos will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.4, 5.2.4.1.1, 5.2.4.2.1, 5.2A.4.1, 6.1.1.2, A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 5.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-3) Clause 5.2.3.1.1 (Test 1-3) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-3) Clause 5.2.3.2.1 (Test 1-3) |  |
| PDSCH mapping type B (pdsch-MappingTypeB) | FR1 FDD | PDSCH | Clause 5.2.2.1.3 Clause 5.2.3.1.3 Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.3 Clause 5.2.3.2.3 Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Rate-matching around LTE CRS (rateMatchingLTE-CRS) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 Clause 5.2.3.1.4 | For UEs supporting “Alternative additional DMRS position for co-existence with LTE CRS”, if Test 1-2 is tested, the test coverage can be considered fulfilled without executing Test 1-1. Otherwise, only Test 1-1 is tested. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 Clause 5.2.3.2.4 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.1 (Tests 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.16 (Test 3-1, 3-2, 4-1, 4-2) Clause 5.2.2.1.19 (Test 1-1, 2-1) Clause 5.2.3.1.16 (Test 2-1) Clause 5.2.3.1.18 (Tests 1-1, 2-1) Clause 5.2.4.1.1 (Tests 3-1, 4-1, 5-1) | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Test 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.2.17 (Test 3-1, 3-2, 4-1, 4-2) Clause 5.2.2.2.19 (Test 1-1) Clause 5.2.2.2.20 (Tests 1-1, 1-2,2-1,2-2) Clause 5.2.3.2.17 (Test 2-1) Clause 5.2.3.2.18 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1, 1-2,2-1,2-2) Clause 5.2.4.2.1 (Tests 3-1, 4-1, 5-1) |  |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.1.2 Clause 5.2.4.1.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1) | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.2.2 Clause 5.2.4.2.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1) |  |
| Support number of active TCI states per BWP per CC, including control and data (maxNumberActiveTCI-PerBWP) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) | The requirements apply only when maxNumberActiveTCI-PerBWP is other than n1. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) |  |
| Support for maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) Clause 5.2.2.1.11 Clause 5.2.2.1.12 Clause 5.2.2.1.13 Clause 5.2.2.1.14 Clause 5.2.3.1.11 Clause 5.2.3.1.12 Clause 5.2.3.1.13 Clause 5.2.3.1.14 | The requirements apply only when maxSimultaneousResourceSetsPerCC ≥ 2 |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) Clause 5.2.2.2.11 Clause 5.2.2.2.12 Clause 5.2.2.2.13 Clause 5.2.2.2.14 Clause 5.2.3.2.11 Clause 5.2.3.2.12 Clause 5.2.3.2.13 Clause 5.2.3.2.14 |  |

5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

Table 5.2-1: Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12] For number of Tx>2, set “codebookMode” to 1 as defined in section 5.2.2.2.1 of TS 38.214 [12] |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 3 for 2 CSI-RS ports, 5 for 4 CSI-RS ports and 6 for 8 CSI-RS ports |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'No CDM' for 1 transmit antenna 'FD-CDM2' for 2, 4 and 8 transmit antenna |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests {1000-1007} for Rank 8 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3,Rank 4 and Rank 8 tests |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  |  | Not configured |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1:Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. For number of Tx>2 and Rank=1 or 2, Set “codebookMode” to 1 as defined in section 5.2.2.2.1 of TS 38.214 [12] |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |

5.2.4	8RX requirements

5.2.4.1	FDD

5.2.4.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.4.1.1-3 - Table 5.2.4.1.1-7, with the addition of test parameters in Table 5.2.4.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.4.1.1-1.

Table 5.2.4.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 2-1, 3-1, 4-1, 5-1 |

Table 5.2.4.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.4.1.1-3: Minimum performance for Rank 2 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |

Table 5.2.4.1.1-4: Minimum performance for Rank 2 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.7 |

Table 5.2.4.1.1-5: Minimum performance for Rank 4 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-3.7 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 4x8, ULA Low | 70 | 12.8 |

Table 5.2.4.1.1-6: Minimum performance for Rank 4 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-3.7 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 4x8, ULA Low | 70 | 15.8 |

Table 5.2.4.1.1-7: Minimum performance for Rank 8

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.1-3.8 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.5 |

5.2.4.2	TDD

5.2.4.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.4.2.1-3 - Table 5.2.4.2.1-7, with the addition of test parameters in Table 5.2.4.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.4.2.1-1.

Table 5.2.4.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 2-1, 3-1, 4-1, 5-1 |

Table 5.2.4.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.4.2.1-3: Minimum performance for Rank 2 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |

Table 5.2.4.2.1-4: Minimum performance for Rank 2 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.8 |

Table 5.2.4.2.1-5: Minimum performance for Rank 4 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.2-3.7 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 12.9 |

Table 5.2.4.2.1-6: Minimum performance for Rank 4 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.2-3.7 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 16.0 |

Table 5.2.4.2.1-7: Minimum performance for Rank 8

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.2-3.8 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |

5.2A.4	8RX requirements

5.2A.4.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.4.1-7 or Table 5.2A.4.1-8 (dependent on Baseline or Simplified SU-MIMO 8Rx Receiver), the requirements are based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.4.1-1 ~ Table 5.2A.4.1-6, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.4.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.9 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |

Table 5.2A.4.1-2: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx  Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |

Table 5.2A.4.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-20.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 15 | R.PDSCH.1-20.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.1-20.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 25 | R.PDSCH.1-20.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 30 | R.PDSCH.1-21.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 35 | R.PDSCH.1-21.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 40 | R.PDSCH.1-21.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.9 |
| 45 | R.PDSCH.1-21.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 50 | R.PDSCH.1-21.5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.5 |

Table 5.2A.4.1-4: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.3 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |

Table 5.2A.4.1-5: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |

Table 5.2A.4.1-6: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-34.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 10 | R.PDSCH.2-34.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 15 | R.PDSCH.2-34.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.2-34.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 25 | R.PDSCH.2-34.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 30 | R.PDSCH.2-34.6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 40 | R.PDSCH.2-3.7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.1 |
| 50 | R.PDSCH.2-35.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.0 |
| 60 | R.PDSCH.2-35.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 80 | R.PDSCH.2-35.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 90 | R.PDSCH.2-35.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 100 | R.PDSCH.2- 35.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.8 |

Table 5.2A.4.1-7: Minimum performance for multiple CA configurations, Baseline Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-1, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-1, 5.2A.4.1-3, 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied for both CCs. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

Table 5.2A.4.1-8: Minimum performance for multiple CA configurations, Simplified Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-2, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-2, 5.2A.4.1-3, 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-2 and 5.2A.4.1-5 shall be applied for both CCs. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

<Unchanged skipped>

6	CSI reporting requirements (Conducted requirements)

6.1	General

This clause includes conducted requirements for the reporting of channel state information (CSI).

6.1.1	Applicability of requirements

<Unchanged skipped>

6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2,4 or 8RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |
| UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx | CQI | Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 |
|  | PMI | All tests in Clause 6.3.3(Note 1) |
|  | RI | All tests in Clause 6.4.3(Note 1) |
| UE supports 2Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 |
|  | PMI | All tests in Clause 6.3.2(Note 2) |
|  | RI | All tests in Clause 6.4.2(Note 2) |
| UE supports only 8Rx | CQI | All tests in Clause 6.2.4 |
|  | PMI | N/A |
|  | RI | N/A |
| Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |

Table A.3.2.1.1-22: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-22.1 FDD | R.PDSCH.1-22.2 FDD | R.PDSCH.1-22.3 FDD | R.PDSCH.1-22.4 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.50 | 0.50 | 0.50 | 0.50 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 19968 | 63528 | 83976 | 106576 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 3 | 8 | 10 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 37800 | 119448 | 160272 | 201096 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 29600 | 125136 | 167904 | 210672 |
| Max. Throughput averaged over 2 frames | Mbps | 18.970 | 60.352 | 79.777 | 101.247 |