# 3GPP TSG- RAN WG4 Meeting #112	R4-2413561

# Maastricht, Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | 0044 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.115-1 for NCR conformance testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This specification is for NCR, not for the IAB. Repeated references for TS 38.101-1 and citation errors in References. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct wording error and remove duplicate references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - This specification is for NCR, not for the IAB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.2 , 6.20.4.2, 7.1.2.2.4, 7.1.3.1.2, 7.1.3.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2412796. |  |  |  |  |  |  |  |  |

< START OF CHANGE>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.106: "NR; Repeater Radio Transmission and Reception"

[3]	3GPP TS 38.115-2: "NR; Repeater conformance testing, Part 2: Radiated conformance testing"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[6]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[7]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing, Part 1: Conducted conformance testing"

[8]	3GPP TS 38.211: "NR; Physical channels and modulation"

[9]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[11]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”

[12]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters"

[13]	Federal Communications Commission: "Title 47 of the Code of Federal Regulations (CFR) "

[14]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[15]	3GPP TR 25.942: "RF system scenarios"

[16]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[17]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[18]	IEC 60 721: "Classification of environmental conditions"

[19]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[20]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[21]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[22]	Void

[23]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[24]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

< NEXT OF CHANGE>

6.18.2	Minimum requirement

The minimum requirement for MT connectors of  local area NCR type 1-C and NCR type 1-H is defined in TS 38.101-1 [9], clause 7.7.

< NEXT OF CHANGE>

6.20.4.2	Procedure

The minimum requirement is applied to all connectors under test,

For NCR type 1-H where there may be multiple TAB connectors they may be tested one at a time or multiple TAB connectors may be tested in parallel. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the connector under test to measurement equipment .

2)	For NCR-MT, set the measurement equipment parameters as specified in table 6.20.5.1-1.

3)	For NCR-MT, measure the spurious emissions over each frequency range described in table 6.20.5.1-1.

In addition, for a multi-band connector, the following steps shall apply:

4)	For multi-band connector and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

< NEXT OF CHANGE>

##### 7.1.2.2.4	Method of test

7.1.2.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

7.1.2.2.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors  for diversity reception via a combining network as shown in annex D.9 or D.11.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex F and the test parameters in table 7.1.2.2.4.2-1.

Table 7.1.2.2.4.2-1: Test parameters for testing PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active BWP index |  |  | 1 |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| PDSCH DMRS configuration | Antenna ports indexes |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PT-RS configuration |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: Point A coincides with minimum guard band as specified in TS 38.101-1 [9] for tested channel bandwidth and subcarrier spacing. |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 7.1.2.2.5.1-1 is achieved at the NCR-MT input.

6)	For each of the reference channels in table 7.1.2.2.5.1-1 measure the throughput and the 1st transmission BLER.

< NEXT OF CHANGE>

7.1.3.1.2	Common test parameters

Parameters specified in Table 7.1.3.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 7.1.3.1.2-1: Test parameters for CSI test cases

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Duplex mode |  |  | TDD |
| PTRS epre-Ratio |  |  | N/A |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | DMRS ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 {1000,1001,1002} for Rank3 {1000,1001,1002,1003} for Rank4 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  | N/A |
|  | Time density (LPT-RS) |  | N/A |
|  | Resource Element Offset |  | N/A |
| NZP CSI-RS for CSI acquisition | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. NOTE 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [9] for tested channel bandwidth and subcarrier spacing. |  |  |  |

< End OF CHANGE>