3GPP TSG-RAN4 Meeting #112	R4-2413928

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4711 | rev | 1 | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Perf) CR to TS 38.133 measurement accuracy related corrections for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The CSI-RS based L1-RSRP measurement core requirement and test cases has been introduced for NTN, however, the corresponding measurement accuracy requirement is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce CSI-RS based L1-RSRP accuracy requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN spec will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.19C.2(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised of R4-2411749 |  |  |  |  |  |  |  |  |

# <Start of change#1>

10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements

10.1.19C.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement.

The accuracy requirements in Table 10.1.19C.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-5 [39] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.19.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	Valid information for the SAN serving the target cell has been provided.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19C.2.1-1.

Table 10.1.19C.2.1-1: CSI-RS based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
| [5] | [9.5] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -70 |
| [8.5] | [11.5] | -3 | NR_FDD_SAB_FR1_A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2A. |  |  |  |  |  |  |  |

10.1.19C.2.2	Relative Accuracy

The relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RS resources of the serving cell.

The accuracy requirements in Table 10.1.19C.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-5 [39] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.19.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	Valid information for the SAN serving the target cell has been provided.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19C.2.2-1.

Table 10.1.19C.2.2-1: CSI-RS based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
| [3] | [4] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2A. |  |  |  |  |  |  |  |

# <End of change#1>