3GPP TSG-RAN WG1 Meeting #115															R4-2414446

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.17.5

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized under a single thread [129] in RAN4#112. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Testability and interoperability issues for CSI compression and CSI prediction

### Sub-topic 4-1

Option 3 feasibility study steps

In the previous meeting it was agreed to perform a feasibility study for Option 3(fully specified decoder) based on “Option 1” as follows:

Option 1

companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

The next steps should be discussed and agreed

Issue 4-1: Option 3 next steps

- Proposals

  - Option 1:

- Agree on the simulation assumptions for dataset generation (completed)

- Agree on the AI-ML model assumptions for feasibility analysis (completed)

- Agree on the procedure for vendor-specific encoder training

- Agree on the framework/format for AI-ML model sharing among companies

- Agree on the framework for dataset sharing among companies including specific set of datasets to be used for encoder training

- Select one or more decoder models for further analysis

  - Companies bring simulation results for the pair of trained Encoder/Decoder models

    - Check and confirm simulation results alignment

    - If alignment is not achieved, agree on modified/additional simulation assumptions and/or model assumptions for the next round.

  - Select one or more decoder models for further analysis

  - Agreed models are shared with other companies based on the agreed framework/format

- Collect datasets for model training

- Check feasibility of Option 3 (i.e. to design/train UE-specific encoder based on the agreed common Decoder)

  - Agree on the criteria to confirm feasibility of Option 3

  - Companies bring simulation results for the pair of trained company-specific Encoder and common Decoder models

  - Check feasibility based on the agreed criteria

  - Option 2:

  - Option 3: others

- Recommended WF

  - Discuss the next steps, edit/amend as needed:

- Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Select one or more decoder for further analysis

  - selection criteria to be discussed

- Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

Note: some steps could be reused for Option 4, FFS on what can be reused

Discussion:

E///: ok in general, for step 1, there are some differences in the parameters for the simulations. we should expend on step whether we want to iterate on alignment or not

Nokia: for alignment we need to further refine some assumptions. for the reference encoder, it should be used to derive the test decoder. how do we do that? we would derive the reference later on?

Apple: agree with E///. we need to align the results. we should see sufficient gain over eType II codebook. without gain, encoder/decoder is too simplistic. we will not necessarily get the same feasibility result with a more complicated result.

Intel: we should continue with the alignment. there is a big difference even in eType II performance. logistics should not be discussed separately. we need to decide formats for data, etc. it might have impact on the technical discussions. we need to align the training procedure. we can make the training jointly, or we can make the training based on the decoder input.

Samsung: 2nd sub-bullet of 3. should be removed. if we have misalignment, it could come from different sources, even training data from the system sims could end up being different. how to solve the misalignment is also a part of the feasibility study

Oppo: for the alignment definition. there could be different definitions. we might want to try the minimum performance or try to align the output

Huawei: feasibility study is incomplete. testability is also important. we are trying to verify whether it is feasible to derive a model which has sufficient gain.

Qualcomm: all companies except 1 already showed gain, whether this is enough or not, should be left to a potential WI phase

Ericsson: gain discussion would be a WI. we should simplify this discussion, we will learn about how to go about alignment, sharing models, training encoder against a shared model, etc. if this is not feasible, it would definitely not work.

Apple: we are checking feasibility in this study. should be based on some reasonable encoder/decoder. just aligning based on a simple model might be useless. how to judge if performance is good or not. if the gain is not good enough, decoder is not suitable. if the SGCS gain is very small, the throughput gain will probably be very small. we are benchmarking AI/ML against eType II.

R&S: we are discussing about alignment. do we need to align? the complexity here is too high anyway.

Nokia: what kind of degradation we will get is the question of the study.

Intel: we should discuss the sharing framework logistics

NTU: we should remove step 1. we will have a selection anyway. we are investigating the alignment under the agreed decoder model. if we can align CNN, it does not imply we can align transformes. we do not need alignment now, we can go directly to step 2.

vivo: we would like to add a note that these steps can be used for Option 4.

Samsung: we should remove sub-bullet of Step 3

Apple: we should also add data for inference

Nokia: Step 2 is a precursor to step 3-4.

Apple: some of the steps we might have to iterate on

Intel : we do not have a feasibility criteria for Option 3. excluding decoder flexibility

### Sub-topic 4-2

Simulation results discussion

Several companies brought simulation results based on the agreed parameters in the email discussion and shown in R4-2411786.

Issue 4-2: Simulation results

- Proposals

Option 1: Check on the simulation results and on whether there is a need for more alignment

- Recommended WF

  - To be discussed whether alignment is good enough or not

  - Depending on need, discuss which parameters should be agreed on to improve alignment

Discussion:

E///: we would have to align if this were a real WI. is there something useful we could learn. it is possible to create models which are aligned? it would be useful to understand if possible to align. we still need a way to choose.

Apple: based on the SGCS alignment, we do not think we have enough alignment. our observation is that even baseline (eType II performance) is not aligned. we can discuss what else to align, for example the training data. assumptions on the system level we can align better. talking about the channels, how to average, SVD, eigen vector, etc. this is all implementation issue in RAN1. we can fix what we can for the assumptions.

Intel: it would be good to align on more parameters. this is based on system level simulators. for system level simulators there is a big spread. there will be mismatch. eType II performance has implementation specific aspects. there could be differences. I would suggest to align more on the model, for example how to do quantization, fixed or trainable.

Oppo: how to align the parameters? we cannot align simulation platforms. if we try to align many paraemters, we do nothing in the end.

Nokia: we would like to understand the gains. the main source of misalignment is the dataset in our opinion. only way to do fix is to use the dataset. if companies want to share, we could understand what to align. if companies can share the postprocessing of the channel information.

Proposed WF:

- perform a round of alignment until RAN4#112-Bis

- further discuss what parameters to better align

### Sub-topic 4-3

Criteria to select decoder for feasibility study

One of the steps of the Option 3 feasibility study will be to select a decoder for companies to train an encoder. Selection criteria should be discussed. This decoder and the criteria is only to be used for this feasibility study, the selection criteria will be rediscussed in a potential WI as needed.

Issue 4-3: Decoder selection criteria

- Proposals

  - Option 1: Best performance (SGCS) and/or mean performance of corresponding eType II CSI feedback

  - Option 2: select multiple and work in parallel

    - few decoders can be selected and a subset of companies check feasibility on one decoder

  - Option 3: random selection, as long as performance is similar it does not matter which decoder is selected

  - Option 4: decoder trained with the highest number of sample

  - Option 5: To use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies.

  - Option 6: other criteria

- Recommended WF

  - To be discussed

Discussion

Apple: we can skip this until we get to this step

### Sub-topic 4-4

Options for Option 4

Option 4 for CSI compression testing has not been discussed much. There are several sub-options about this could work. The list of options below also includes some proposals from previous meetings:

Issue 4-4: Options for Option 4

- Proposals

  - Option 1: Nokia(R4-241022)

    - Select a reference encoder/decoder pair to create a dataset for training Option 4 test decoders.

      - A reference encoder/decoder pair can be used as the source of the dataset {V, c} that specifies the feedback interface and ensures test decoder interoperability for Option 4.

      - Test repeatability in Option 4 can be addressed by developing a test decoder validation procedure including decoder performance specifications using the specified dataset

  - Option 2: NTU (R4-2412130)

    - Partial mapping alignment

  - Option 3: ZTE (R4-2412792)

    - To consider standardizing the reference model or reference encoder and latent space for option 4.

  - Option 4: Huawei (R4-2412768)

    - Option 4a: Model structure is not specified in RAN4. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder.

  - Option 5: Huawei (R4-2412768)

    - Option 4b: Model structure is specified in RAN4. Training dataset is not specified for verifying the encoder at DUT. The test decoder developed by TE vendor needs verification.

      - FFS: How to determine the test metric for test decoder developed by each TE vendor.

  - Option 6: Qualcomm (R4-2407334)

- Option 4a-1 standardized dataset

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

  - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4a-2 standardized aggregated dataset

  - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

  - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

  - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

  - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4b reference encoder/decoder pair

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

  - Step 2: RAN4 capture the encoder/decoder as a reference encoder/decoder pair and the encoder input data generation procedure in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

  - Option 7: Xiaomi (R4-2407847): Standardized data / dataset format + Dataset exchange between NW-side and UE-side

  - Option 8: vivo(R4-2408294)

  - Option 4a: Reference encoder + test decoder model structure, and channel generation method are in the spec;

  - Option 4b: Test decoder model structure + reference encoder model structure, and dataset (PMI and corresponding channel) are in the spec;

  - Option 9: Nokia (R4-2408659)

- Option 2 (Dataset based):

  - Option 2a: Freeze complete training data while leaving model architecture for implementation.

  - Option 2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

- Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

  - Option 10: Ericsson (R4-2408492)

    - the latent space needs to be standardized

    - standardize a reference encoder in order to capture the latent space

    - Procedure for identifying reference encoder can be based on the same methodology as selecting test decoder/reference encoder for option 3

  - Option 11: CATT (R4-2411343)

    - Scenarios/configurations (which affect the characteristics of training data and they will be specified in TCs)

    - Data format

    - Latent space needs to be standardized

    - Model backbone

  - Option 12: others

Option grouping based on commonality of proposals:

  - Option 1: dataset based

    - 1. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder

    - 2 : (Nokia)

      - Option-2a: Freeze complete training data while leaving model architecture for implementation.

      - Option-2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

    - 3 : (Huawei)Model structure is not specified in RAN4. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder.

    - 4: (Qualcomm)

      - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

      - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

      - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

    - 5: (NTU)

      - partial mapping of encoder input/decoder output and subset of latent space

    - 6: (Xiaomi): Standardized data / dataset format + Dataset exchange between NW-side and UE-side

    - 7: (CATT):

      - Scenarios/configurations (which affect the characteristics of training data and they will be specified in TCs)

      - Data format

      - Latent space needs to be standardized

      - Model backbone

  - Option 2: Reference encoder

    - 1: (Ericsson) standardize a reference encoder and a test data framework and test dataset/channel model:

    - 2: (Qualcomm)

      - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

      - Step 2: RAN4 capture the encoder as a reference encoder and the encoder input data generation procedure in the specification.

      - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

    - 3: (vivo) specify encoder

    - 4: (vivo) specify encoder backbone

    - 5: (ZTE): standardize the reference model or reference encoder and latent space for option 4.

  - Option 3: Model structure/architecture based:

    - 1: (Nokia) (Model architecture-based):

      - Option-1a: Freeze a complete model architecture while leaving training data for implementation.

      - Option-1b: Freeze a backbone of model architecture while leaving complete training data and model architectural details for implementation.

    - 2: (Huawei)Option 4b: Model structure is specified in RAN4. Training dataset is not specified for verifying the encoder at DUT. The test decoder developed by TE vendor needs verification.

      - FFS: How to determine the test metric for test decoder developed by each TE vendor.

    - 3: (vivo) specify model structure of decoder

  - Option 4: aggregated data set: (Qualcomm)(Apple)

    - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table) for the test decoder.

    - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

    - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

    - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

  - Option 5: Other – “hybrid”

    - 1: (Nokia)Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

  - Option 6: Others

- Recommended WF

  - To be discussed, options are not exclusive

Likely multiple options need to be chosen/combined RAN4 should agree on a minimum set such that companies can continue the study

Discussion:

Nokia: there are many options. question is where the dataset is coming from: can be done based on a reference encoder/decoder, this is somehow standardizing the encoder-decoder interface. then it should be possible to design more freely the encoder and the decoder

NTU: our proposal is without any dataset or reference encoder. this follows the concept of vector quantization. we can have all the quantization subset in a certain order and minimize the error. this is a new approach. any dataset based approach will lead to the same thing.

R&S: we would need some encoder to verify the decoder. we should wait until we finish Option 3. if we align by next meeting and figure out which parameters are important, it would be useful for this also.

Ericsson: we propose a reference encoder to represent the latent space. Option 1 and 2 is related. we can see how well we can align with a common data set or statistics. if we cannot align, Opiton 4 is surely unfeasible. if we can align on the latent space, Option 4 should be feasible.

Qualcomm: we do not support the 4(aggregated data set) option anymore even though we proposed in a previous meeting

Huawei: Option 1 is the same as Option3, we specify the data set generically. in RAN1, specifying the dataset for model training is a different test. we would prefer to specify a reference encoder.

Apple: we still need to answer whether we work in parallel on Options 3 and 4. we chose Option 3 in the previous meeting. some of the steps can be shared among Option 3 and 4. we can wait a bit until we better align step 1 in Option 3. the reason for our proposal on the aggregated dataset is also based on RAN1 study. best is to get a labelled data set from multiple UEs(basically aggregating), this would get the decoder to be versatile enough.

Intel: there are many similarities between Option 3 and 4. we are going to specify a decoder in Option 3. we might need to have a data set.  Option 4 is mostly about the dataset. for Option 4 we think that data set approach is reasonable. a reference encoder is not part of the SI conclusion. we do not see the need for making a decision right away.

Option 4 sub-options:

4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

Companies are invited to bring further analysis on how to perform a feasibility study for these options.

### Sub-topic 4-5

CSI prediction accuracy metric

Some companies presented proposals for the CSI prediction accuracy metric. KPIs for CSI were discussed before but prediction was not explicitly discussed so far

Issue 4-5: CSI prediction accuracy metric

- Proposals

  - Option 1: CSI feedback predicted and reported by the UE in legacy format (e.g., eType II) can be used by SS for PDSCH precoding, i.e., like in legacy requirements.

    - RAN4 to ensure that relative throughput based on AI/ML CSI feedback with the same overhead is at least not worse than in legacy tests with the same parameters

  - Option 2: use relative throughput with baseline throughput based on:

    - Option 1: Throughput achieved by following random PMI (with the same codebook used for the reported predicted PMI)

    - Option 2: Throughput achieved by following UE’s last reported PMI given by UE measurement

- Recommended WF

  - To be discussed, options are not exclusive

Discussion

Nokia: for Option1, the issue we need the UE to support Type II feedback. it could be a problem because this is not mandatory. Option 2 would work with any UE. we have a way to compare performance with Type II in Option 2? we should because we can define the requirement based on Type II.

E///: not clear we need to relate prediction performance with the one without prediction. we are aiming how to achieve minimum performance, not that AI is better. We can take Option 2 and then further discussion with sub-option to adopt.

Apple: there is not much difference between these options. we should see some gain over legacy reporting scheme. based on existing codebook, there would be a mismatch but prediction should make this difference small.

Samsung: there is similary between Option 2-2 and Opttion 1. Option 2-2 means that we need to have some throughput gain. there is a risk with Option 2-1, how we can identify the gain from AI is not clear.

QC: for prediction, RAN1 is comparing the results with eType II. some UEs might not support it. requirements can be defined with respect to random PMI. we can define the requirements such that UE would do better than eTYPE II even though UEs would not support it.

Apple: Option 2 is relative between what and what?

Huawei: we suggest to take Option 2-1 as basleine. we do not know how to get the ground truth for Option 2-2.

E///: we have to take one of the suboption together with option 2.

QC: what is 2-2 ? last reported PMI is based on what? legacy reported PMI(Rel-15)?

Samsung: it doesn’t really matter what is legacy, AI should be better than anything the UE can do before. it has to be relative to a supported codebook.

Apple: now Option 2 is confusing. we need to clarify what is the baseline, basically what it is relative to

oppo: we should test the PMI, how it is obtained should not matter. what matter is the improvement, not how it is obtained.

Nokia: option 2-1 is also confusing. with eType II it might not work, this is hard to randomize.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

### Sub-topic 4-6

Test decoder verification

Issue 4-6: Test decoder verification

- Proposals

  - Option 1: For test decoder’s verification fully specified reference encoder should be used.

    - For test decoder’s verification, the minimum performance target should be agreed and specified considering various results from different stakeholders

    - FFS how the actual validation procedure is performedReference encoder may be implemented by UE vendors, TE vendors or a third-party implementation of UE sided operations including reference encoder.

  - Option 2: Others

- Recommended WF

  - To be discussed, options are not exclusive

Discussion:

### Sub-topic 4-7

Generalization issues

Issue 4-7: Generalization Issues

- Proposals

  - Option 1: test generalization with different deployment parameters (antenna port numbers, antenna spacings, different carrier frequencies, UE speeds, etc)

  - Option 2: test generalization with various bandwidths and/or subband sizes, different CSI feedback payloads

  - Option 3: For the verification/testing of generalization/scalability related aspects in RAN4 for AI/ML enabled CSI feedback enhancement, RAN4 should define different scenarios based on parameters listed in the tables below:

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR | Good / Bad Radio conditions |
| Outdoor / Indoor | Position of the UE |
| LOS/NLOS |  |
| Propagation Model | UMa / Umi |

| Parameters | Description |
| --- | --- |
| Number of Antenna ports | (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) |
| Carrier Frequency | FDD, TDD at sub-band level |
| Bandwidth | E.g., 10MHz, 20MHz |

  - Option 4: use of non-static testing environment

  - Option 5: Others

- Recommended WF

  - To be discussed, options are not exclusive

Discussion:

## Testability and interoperability issues for beam management

### Sub-topic 2-6

Measurement error impact evaluation

Multiple companies brought up the need to evaluate the impact that the UE measurement error and the error in the training data have on inference accuracy

Issue 2-6:	Measurement error impact

- Proposals

  - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study

  - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now

  - Option 3: No need to evaluate the measurement error impact

- Recommended WF

  - To be discussed

Discussion:

Nokia: we have shown inference degradation because of this error, we would like to go with Option 1.

QC: this will influence different metrics in different ways. we haven’t yet downselected those metrics, it will be hard to evaluate. we would prefer to postpone the discussion

E///: measurement error is always there. we understand the effect, why shouldn’t we discuss and leave it until the metrics are decided.

Nokia: I agree that we are still discussion those metrics. we shoed impact on the L1 RSRP prediction accuracy. we could study this.

Samsung: Nokia should clarify about what this study should do. do we do system study without error and with some error modelled, for example, a gaussian distribution of the measurement error?

Nokia: we shown the impact of errors on inference prediction based on system sims. we have to evaluate the impact on the prediction accuracy, this will impact the requirements. option 1 is about studying the impact of the measurement error. we should come up with proposals on how to study and how the study will help/impact the definition of the requirements.

E///: we should have a common understand that measurement error will impact different metrics. we can define the exact simulation configurations and other paraemters later. measurement error will influence the performance .

CATT: RAN1 has done some study already, RAN4 work should be based on that.

Huawei: what will we do if it is found that the measurement error influences the prediction

Moderator: this should be taken into account when we will define the prediction requirements like accuracy

Apple: we need this study, this was not very well considered in the RAN1 study, was only added later. we need some model for the error. there is value to do this in RAN4.

vivo: simulation is necessary, we provide some results in this meeting. there is an impact. we will see what modelling we need to do.

Samsung: measurement error is not ok,

Intel: what will be the outcome, used for performance requirements definition? should we instead provide some info to RAN1?

Moderator: this would be used when we define requirements for inference.

Apple: we agree with the moderator. we do not need to define the model, we need to define the side conditions. we can use the current side conditions for L1-RSRP. based on that we can do real measurements and quantify the prediction performance. in case performance is poor, we might need to revisit the side condition. after some rounds of sim, we can converge to certain SNR level. there is another debate whether we should assume the same accuracy or something.

oppo: we are ok with the wording, add a not about not enhancing the requirements

E///: the measurement error in the training phase will also impact the prediction error.

R&S: the test equipment will also have some error so we might need to understand how those will impact the testing requirements

Agreement:

RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study.

impact on different prediction accuracy metrics should be taken into account

studies already performance by other groups(RAN1) could also be taken into account

Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

### Sub-topic 2-7

UE reporting for network side models

One company brought up a possible need for RAN4 to introduce different reporting schemes to help train the network side models.

Issue 2-7:	UE reporting for network side models

- Proposals

  - Option 1: RAN4 shall introduce the necessary core requirement on supporting data collection for NW-side AI/ML model inference/training (for BM-Case1 & 2), by considering:

    - Potential enhancement on L1 measurement/reporting for inference: e.g., beam reporting for more than 4 beam in L1 signaling, and overhead reduction;

    - Potential enhancement on MDT-based measurement/reporting for training.

  - Option 2: RAN4 cannot introduce new reporting schemes, proposal should be made in another WG

  - Option 3: others

- Recommended WF

  - To be discussed

Discussion:

Samsung: if RAN1 introduces something, we need to define requirement.

Agreement:

postpone discussion until other WGs define any reporting mechanisms

### Sub-topic 2-8

Beam prediction and TCI state related issues

Some companies brought up the issue that the UE will predict the gNB Tx beam but it might know the UE Rx beam. This could affect the known/unknown TCI state conditions

Issue 2-8:	Beam prediction and TCI state related issues

- Proposals

  - Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam

    - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms

  - Option 2: UE knows the Rx beam paired with the predicted Tx beam

    - FFS whether the TCI state associated with a predicted Tx beam is known or unknown

  - Option 3: UE knows the Rx beam paired with the predicted Tx beam

    - TCI state should be considered known , FFS whether side conditions for known TCI state should be modified (relaxed) for this case

  - Option 4: others

- Recommended WF

  - To be discussed

Discussion:

Docomo: beam prediction is based on RSRP, the UE Rx beam should be known, we support Option 2.

Samsung: this is related to what we discussed yesterday about the input to the UE. it highly depends on the neural network input. if it is only RSRP as input, it doesn’t seem that UE would know. Option 2 would make sense if UE knows the Rx beam.

QC: seems most companies support RSRP prediction as a metric, this implies that UE knows the Rx beam. we can go with Option 2 and discuss later how to handle the TCI state

Apple: why this matters? UE known or unknown? UE needs to predict the Tx beam, how to receive that it is UE’s choice. if we need additional CSI-RS, this is up to RAN1. it is possible that network configures CSI-RS to refine the beam. RAN1 should decide this

Nokia: in our opinion, known or unknown will have a direct impact on the switch delay. we agree with most companies that we should go with Option 2. we could have a direct impact as stated in Option 3 and should leave the door open for Option 3.

MTK: we support Option 1. as we all know, in Rel-18 SI, RAN1 studied beam pair prediction and Tx beam prediction. UE does not know the Rx beam.

Samung:we could have a sliding window, we could use that as an input for the neural network. in RAN1, the evaluated both. they don’t say anything about this Rx beam. Option 1 seems the safe option for us.

oppo: we hare similar view with MTK. only Tx beam is included in the RAN1 evaluation. we do the prediction to get set A. for different Rx beam, we could get different Rx beams.

QC: we have a different understanding of the RAN1 evaluation. It is assume that the UE knows the best Rx beam. the Rx beam should be known.

### Sub-topic 2-10

KPIs for beam prediction

Different metrics/KPIs have been discussed and were captured in the TR:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-	Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

KPIs and metrics were discussed in the previous meeting without any agreement, RAN4 should continue to discuss what metrics are more appropriate and how they impact requirement definition and testing

Issue 2-10: Metrics/KPIs for BM

Proposals

  - Option 1: Use Option 1

  - Option 2: Use Option 2

  - Option 3: Use Option 1,2,3 depending on use case

  - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed

  - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs.

    - consider defining multiple tests under different conditions but with the same outcome(same predicted beam)

  - Option 6: Other

- Recommended WF

  - To be discussed

Companies suggesting to use different metrics(new) should come up with a concrete proposal

Discussion:

Nokia: for Option 5, we are considering that there is a known ground truth for the evaluation. the ground truth is not available. one option is not to predict the prediction to the ground truth but to compare predictions between each other. if the best beam is predicted, it’s still the best beam even in another test. we can use both tests, ground truth and something like this with different predictions

Moderator: how do we know that the UE doesn’t always predict the wrong beam. some ground truth relationship is still there.

CATT: we should consider related to monitoring. in our view, we should not make any downselection at this stage. we should wait for more decisions on monitoring.

E///: we understand Nokia’s intention. the metrics can be discussed in performance, we can use different tests to evaluate different models.

### Sub-topic 2-9

UE side requirements

Beam prediction will have an impact on the UE requirements, potential impact is to be discussed.

Issue 2-9:	UE side beam prediction requirements impact

- Proposals

  - Option 1:

| UE- sided model | Data collection | - FFS new clause for data collection for UE-sided model training | FFS RAN4 impact from data collection for UE-sided model training (depending on RAN1/2 input). |
| --- | --- | --- | --- |
|  | Additional assistance information | - Added as new side condition: consistency/association shall be guaranteed for inference and perf. monitoring requirement testing | For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed as side condition. |
|  | Inference | - New clause for UE-side AI/ML model inference | RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results (2) the delay of reporting inference results (containing the inference latency). |
|  | Perf. monitoring | - FFS new clause is needed for Type 1 - Option 1 - New clause for Type 1 - Option 2 - No RAN4 impact expected for Type 2 | For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as: - Type 1, Option 1 (NW-side performance monitoring): The necessity of RAN4 requirement on data collection for monitoring is not significant, because it is similar to data collection for other purposes. - Type 1, Option 2 (UE-assisted performance monitoring): RAN4 requirement on data collection for monitoring can be specified to test the accuracy of performance metrics calculated by UE. - Type 2 (UE-side performance monitoring): No RAN4 requirement is needed because no UE feedback will be performed. |
|  | LCM | - New clause for RAN4 RRM requirement for “Network decision, network-initiated” LCM | For UE-sided model LCM, the necessity of RAN4 requirement: - “Network decision, network-initiated” LCM: RAN4 requirement is needed. - FFS the detailed RAN4 requirement (depending on RAN1/2 input). |

  - Option 2: Postpone the discussion until RAN1/2 agree on more details

  - Option 3: Others

- Recommended WF

  - To be discussed

Discussion:

## Testability and interoperability issues for positioning accuracy enhancement

### Sub-topic 3-1

Requirements for case 1

In the previous meeting it was discussed whether to introduce requirements for case 1 or not but no agreement was reached.

Issue 3-1: Requirements for case 1

- Proposals

  - Option 1: RAN4 should not define requirements for case 1 because positioning accuracy is not testable

  - Option 2: RAN4 to further study defining requirements for case 1, companies are invited to bring further analysis:

    - how to establish the ground truth ( what should the UE report)

    - how to define accuracy requirements

    - what are the inputs for the AI/ML model

    - what side conditions to consider

  - Option 3: RAN4 should study how to define relative requirements (e.g. how the reported position changes based on changing inputs)

  - Option 4: Others

- Recommended WF

To be discussed

If Option 2 is agreed, other aspects to be studied can be discussed.

Discussion:

CMCC: we are not ok with Option 1. it is to early. this case is representative for the UEs in the field. UE may not report the positioning, this is still under discussion. for monitoring, the UE could report the positioning to the LMF for monitoring. another concern is privacy, the report to the LMF is transparent to the gNB. accuracy can be defined based on prediction vs. ground truth.

E///: we still have concern about having such requirement, it is hard to agree to anything right now.

Docomo: from operator’s point of view, we agree with CMCC. practically, it will be difficult to define any requirement/test. the important issue will be to have an accurate training dataset.

Nokia: we are repeating the same discussion. it’s not the first time we test something like this, we have A-GNSS tests. we are proposing a full testing procedure.

### Sub-topic 3-2

Requirements for case 2a

Some companies are proposing to define requirements for case 2a

Issue 3-2: Requirements for case 2a

- Proposals

  - Option 1: RAN4 should not define any positioning accuracy requirements because positioning is LMF based

  - Option 2: RAN4 should continue to study how to define requirements on the positioning accuracy of the LMF

  - Option 3: others

- Recommended WF

  - Option 1

Discussion:

Agreement: Option 1

### Sub-topic 3-3

Requirements for reported metrics for case 2a

Even if requirements for the positioning accuracy computed by the LMF are not defined for case 2a, some companies are proposing to introduce requirements on the metrics reported by the UE as inputs to the LMF positioning model.

Issue 3-3: Reported metrics for case 2a

- Proposals

  - Option 1: RAN4 to define requirements for LOS/NLOS reporting (reported metric agreed by RAN1)

    - companies to bring analysis on how such requirements can be defined and tested

  - Option 2: RAN4 to continue studying how to define requirements for other reported metrics (RSTD, UE Rx-Tx, etc)

    - Companies to bring analysis on the difference with existing requirements, how to define new requirements and tests

  - Option 3: Defining requirements for reported metrics is not feasible and should not be further discussed

  - Option 4: Wait for other groups to agree on the reporting framework before continuing this discussion.

- Recommended WF

  - To be discussed, options are not exclusive

Discussion:

CATT: for existing measurement quantities, existing accuracy requirements are the baseline. currently there is no defined new measurement quantities.

E///: for 2a, RAN1 agreed that RSTD or Rx-Tx, maybe we can look into different requirements.

Nokia: about reusing the existing as a baseline, we should focus on new type of measurement. we would like to prioritize the new reports.

QC: we prefer reusing the existing measurement quantities as baseline. we are not sure about any new reported metrics.

oppo: feasibility of reusing existing requirements is not clear to us. AI/ML based methods improve the measurements a lot. this is because of non-linear processing. we should check for new requirements.

Huawei: we need some clarification, does RAN1 evaluate the performance gain of the model? they evaluated the positioning accuracy. if RAN1 doesn’t do this, how can we find the impact of report accuracy on the positioning? this doesn’t seem feasible.

CMCC: according to RAN1 agreement, RSTD, Rx-Tx and LOS/NLOS are report. for RSTD and Rx-Tx we can take legacy requirements as baseline. for LOS/NLOS indicator we should consider defining requirements.

vivo:we agree with CMCC to use legacy requirements as a starting point. for LOS/NLOS we have to study how to define requirements. we need to wait on other metrics.

E///: RAN1 has done this evaluation, check the TR. how the models impact the overall positioning.

Huawei: companies didn’t try to align the model at the LMF side, we should align the models.

Agreement:

- RAN4 will take existing requirements for existing metrics as baseline

  - FFS whether to revisit these requirements

- RAN4 will study how to define requirement for any new reported metrics

  - FFS how to define requirements for LOS/NLOS

### Sub-topic 3-4

Requirements for case 2b

Issue 3-4: Requirements for reported metrics

- Proposals

  - Option 1: RAN4 should not define any positioning accuracy requirements because positioning is LMF based

  - Option 2: RAN4 to further discuss how to define requirements for the LMF based positioning accuracy (LMF based positioning model)

  - Option 3: others

- Recommended WF

  - Option 1

Discussion:

### Sub-topic 3-5

Requirements for reported metrics for case 2b

Case 2b is based on UE reports of different metrics which are not computed based on AI/ML models. Existing reported metrics (e.g. RSTD) can also be reused

Issue 3-5: Reported metrics for case 2b

- Proposals

  - Option 1: RAN4 to discuss requirements for case 2b only if new reported metrics are introduced by other groups or if the measurement for the reported metric is changed/modified (e.g. for path based and sample based reporting).

    - FFS whether existing requirements for legacy reported metrics will not be revisited

  - Option 2: RAN4 should revisit the existing requirements for legacy reported metrics

    - Companies to bring analysis on what can be improved(what has changed) for existing requirements

  - Option 3: Others

- Recommended WF

  - Option 1

Discussion:

E///: for this RAN1 discussed whether to report path based and/or sample based. we should define  requirements when RAN1 makes an agreement.

QC: we should first wait to see what RAN1 agrees to. we shouldn’t rush into any agreements.

Agreement:

postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported.

### Sub-topic 3-6

Requirements for reported metrics for case 3a/3b

It was agreed not to define positioning accuracy requirements for case 3a/3b, however, some companies are proposing to define requirements for the metrics reported by gNBs which are inputs to the positioning models

Issue 3-5: Reported metrics for case 3a/3b

- Proposals

  - Option 1: RAN4 to further study how to define requirements for reported metrics for case 3a/3b

    - companies should bring concrete analysis on how the requirements could be defined and tested

  - Option 2: RAN4 should not define any requirements for gNB reported metrics

  - Option 3: Others

- Recommended WF

  - To be discussed

Discussion:

E///: Option 2. we never had such requirements on gNB

CMCC: Option 1. we have some requirements for Rx-Tx accuracy and SRS based positioning. those could be taken as baseline.

E////: we would like to highlight that this is a one sided model. we should not proceed with this. we should at least wait until there is a concrete agreement in RAN1.

Agreement:

postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

# References

[1] R4-2412831, “Topic summary for [112][129] NR_AIML_air”, Moderator(Qualcomm Incorporated), RAN4#112