# 3GPP TSG-RAN WG4 Meeting #113	R4-2419786

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:			7.8.5

Source:	Samsung

Title:	Way Forward for [113][322] NTN_testing_NGSO_channel_model

Document for:	Approval

Introduction

This t-doc provides WF for [113][323] NTN_testing_NGSO_channel_model.

WF

Sub-topic 1-1 TE emulated channel model for satellite mobility

Issue 1-1-1 Time varying Doppler shift and propagation delay derivation procedure

- WF

- For Keplerian definition motion base method, consider the following general procedure to generate the time varying Doppler shift and propagation delay. The detail formulars for each step can be considered as reference in Appendix 1.4.

    - Step 1: Set the initial ephemeris information parameters with format of position/velocity state vector in Earth-centered earth-fixed frame (ECEF) format according to the existing RAN5 test and initial UE location parameters with the following tables

      - Step 1.1: covert the initial ephemeris information in ECEF format in table 1 into the format of orbital parameters (Keplerian orbital elements) in ECI (Earth-centered inertial) format in table 2 for orbit generation based on two-body Keplerian model at epoch time T0

      - Step 1.2: covert the configured UE location in a format of latitude, longitude, and altitude in table 3 into the ECEF format in table 4 to align with the satellite position/velocity

      - FFS on the value in table 1, 3 for initial ephemeris information and UE location information

Table 1: Initial ephemeris information in format of position/velocity state vector in ECEF

| - PositionVelocity-r17 | - Unit | - Value | - Abbreviation/ Symbol | - Note |
| --- | --- | --- | --- | --- |
| - positionX-r17 | - meter | - TBD | - rx(0) | - 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3 |
| - positionY-r17 | - meter | - TBD | - ry(0) | - 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3 |
| - positionZ-r17 | - meter | - TBD | - rz(0) | - 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3 |
| - velocityVX-r17 | - meter/second | - TBD | - vx(0) | - 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06 |
| - velocityVY-r17 | - meter/second | - TBD | - vy(0) | - 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06 |
| - velocityVZ-r17 | - meter/second | - TBD | - vz(0) | - 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06 |
| - Note: The symbol represents the actual value derived from the value for the corresponding to the parameter |  |  |  |  |

Table 2: Initial ephemeris information in format of orbital parameters (Keplerian orbital elements) in ECI

| - Orbital-r17 | - Unit | - Value | - Abbreviation/ Symbol | - Note |
| --- | --- | --- | --- | --- |
| - semimajorAxis-r17 | - Meter | - TBD | - a | - 33 bits with 4.249*10-3 m resolution, i.e., actual value = 6500000+field value*4.249*10-3 |
| - eccentricity-r17 | - NA | - TBD | - e | - 20 bits with 1.431*10-8 resolution, i.e., actual value = field value*1.431*10-8 |
| - periapsis-r17 | - Radian | - TBD | - ω | - 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8 |
| - longitude-r17 | - Radian | - TBD | - Ω | - 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8 |
| - inclination-r17 | - Radian | - TBD | - i | - 27 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8 |
| - meanAnomaly-r17 | - Radian | - TBD | - M | - 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8 |

Table 3: Initial UE location parameters in a format of latitude, longitude, and altitude

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Table 4: UE location parameters in a format of position state vector in ECEF

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,x | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| UEt,y | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| UEt,z | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Table 5: Constant value

| Parameters | Abbreviation/ Symbol | Unit | Value |
| --- | --- | --- | --- |
| earth gravitational constant | μ | m3/s2 | 3986004.418e8 |
| velocity of light | c | m/s | 299792458 |
| Earth radius | aE | km | 6378.137 |
| Square of Earth eccentricity | eE2 | N/A | 6.6943799014 x 10-3 |
| Earth angular speed | ωE | rad/s | 7.2921151467 x 10-5 |

    - Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1

      - Step 2.1: Calculate the Mean Anomaly value

      - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation

      - Step 2.3: Derive the True Anomaly at time T1

      - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format with considering the earth rotation

      - Note: The detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation, for example

        - Numerical method

          - Newton-Raphson method

        - Number of iterations

          - Option 1: 4

        - Accuracy requirement E-E0>ϵ

          - Option 1: ϵ=10-8

          - Option 2: ϵ=10-6

      - Note: Satellite position/velocity estimation period, T1-T0, is less than or equal to the satellite ephemeris information update period in Issue 1-1-3-4

    - Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1.

      - Step 3.1: Obtain the satellite position/velocity at time T1

      - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1)

      - Note: The update granularity of propagation delay and Doppler shift should be [1] slot

      - Note: The details interpolation method can be up to implementation, with meets some accuracy requirement, for example

        - Numerical method

          - Option 1: cubic spline interpolation

          - Option 2: linear interpolation

          - Option 3: calculate propagation delay and Doppler shift at any time instant

          - Other options are not precluded

        - Accuracy requirement

          - Option 1:

            - Max|delay-error| ~ 3e-13 s

            - Max|delay-rate error| ~ 3.28e-12

          - Other options not precluded

    - Note: RAN4 will conduct the calibration work for each step in post-meeting. Companies are encouraged to provide the calibration and comparison results with GAMT before RAN4#114 meeting. Based on calibration and comparison results, further discuss how to capture the time varying Doppler shift and propagation delay derivation procedure and formulars into specification in next meeting.

- The proponents to use the equation of motion to derive the satellite position/velocity in Steps 1/2, as an alternative method as the Keplerian model, is encouraged to provide more detail procedure and the comparison results with the Keplerian model in the next meeting

Issue 1-1-2: NGSO NTN test environment

- WF

- RAN4 to consider the test environment for NGSO NTN UE testing

Issue 1-1-3-1: Assumption of Satellite altitude

- WF

- Considering satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modelling for satellite motion trajectory in this WI

- The satellite configuration to be used in each individual performance test shall follow pre-existing conditions for altitude, and as a baseline, no new test cases introduced in this WID to cover for different satellite altitudes

Issue 1-1-3-2: Assumption of UE motion

- WF

- It is common understanding that the earth rotation should be considered when deriving TE-emulated varying Doppler shift and propagation delay between the satellite and DUT(UE)

- Not to consider UE motion during RRM transmit timing test case based on pre-existing test case

Issue 1-1-3-3: Assumption of updated granularity for propagation delay and Doppler shift calculation

- WF

- Option 1 (WF in the last meeting)

    - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model per slot

    - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot

Issue 1-1-3-4: Assumption of updated granularity for Satellite ephemeris information

- WF

- Unless otherwise stated on existing test configuration, the TE is expected to refresh satellite ephemeris in SIB 19 at least once every 10.24s

    - For RF and demodulation requirement, use 10.24s to update the satellite ephemeris information

    - For RRM requirement, use 2.56s to update the satellite ephemeris information

Issue 1-1-3-5: Assumption of Multiple satellite scenario

- WF

  - Focus on TE emulated channel modelling for single satellite scenario, whether and how to extend multiple satellites scenario pending on RRM session

Issue 1-1-3-6: Assumption of Elevation angle range at time 0

- WF

  - Set more satellite position/velocity depending on test purpose and pre-existing test configuration

    - For demodulation test

      - Option 1: starting from [30°] for both NR and IoT NTN

      - Option 2:

        - starting from [ 30°] for NR NTN

        - starting from [10°] for IoT NTN

    - For RF test

      - starting from [10°] for both NR and IoT NTN with considering maximum positive/negative doppler

      - starting from [60°] for both NR and IoT NTN with considering the half maximum positive/negative doppler

    - For RRM test

      - Option 1: starting from [30°]

      - Option 2: starting from [90°]

Issue 1-1-3-7:  Initial ephemeris information at time 0

- WF

  - Option 1

    - Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

  - Option 2

    - GMAT

  - Other options are not precluded

Issue 1-1-3-8:  Time varying of doppler shift and propagation delay generation procedure

- WF

  - Set the configured UE location to Longitude: 121.56076999, Latitude: 25.08439333, and Altitude: 0.0 as same as TS 38.508-1 and TS36. 521-4.

Sub-topic 1-2 RAN4 requirements

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- WF

  - Option 1: Consider Keplerian model for UE assumption on the propagator model

  - Option 2:

    - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

  - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information

Issue 1-2-2 Impact on RAN4 demodulation requirements

- WF

  - Option 1:

    - RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update

    - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

Sub-topic 1-3 Inclusion in Specfication

Issue 1-3-1 Specification of Channel Model

- WF

  - RAN4 to incorporate the TE-emulated channel model with time varying Doppler shift and propagation delay into the annex of TS 38.101-5 and TS 36.102

  - FFS on how to capture into specification

- Reference

[1] R4-2416542, “Topic summary for [113][323] NTN_testing_NGSO_channel_model “, Moderator (Samsung)

- Appendix (for information)

  - SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 | -22027.8799 (km) |  |
| positionY-r17 | 27362209 | 35570.8717 (km) |  |
| positionZ-r17 | 4019393 | 5225.2109 (km) |  |
| velocityVX-r17 | -336 | -0.02016 (km/s) |  |
| velocityVY-r17 | -208 | -0.01248 (km/s) |  |
| velocityVZ-r17 | 1 | 0.00006 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.3182 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 | -3603.0046 (km) |  |
| positionY-r17 | 4509243 | 5862.0159 (km) |  |
| positionZ-r17 | 2455572 | 3192.2436 (km) |  |
| velocityVX-r17 | 20837 | 1.25022 (km/s) |  |
| velocityVY-r17 | -46176 | -2.77056 (km/s) |  |
| velocityVZ-r17 | 109528 | 6.57168 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 | -2859.0536 (km) |  |
| positionY-r17 | 3404229 | 4425.4977 (km) |  |
| positionZ-r17 | 3535794 | 4596.5322 (km) |  |
| velocityVX-r17 | 35394 | 2.12364 (km/s) |  |
| velocityVY-r17 | -74414 | -4.46484 (km/s) |  |
| velocityVZ-r17 | 94682 | 5.68092 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 | -3370.6699 (km) |  |
| positionY-r17 | 4245650 | 5519.345 (km) |  |
| positionZ-r17 | 2024520 | 2631.876 (km) |  |
| velocityVX-r17 | 19359 | 1.16154 (km/s) |  |
| velocityVY-r17 | -43278 | -2.59668 (km/s) |  |
| velocityVZ-r17 | 116553 | 6.99318 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

Detail formula for each step to generate the dopple shift and propagation delay

Input values: Parameters; Unit; Description. r0ECEF=r0,xECEFr0,yECEFr0,zECEF; km; Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF). v0ECEF=v0,xECEFv0,yECEFv0,zECEF; km/s; Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF). t; second; Time to derive the satellite position and velocity. Output values: Parameters; Unit; Description. rtECEF=rt,xECEFrt,yECEFrt,zECEF; km; Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF). vtECEF=vt,xECEFvt,yECEFvt,zECEF; km/s; Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF). Constant parameters: Parameters; Description; Values; Unit. μ; Gravitational parameter for Earth; 3.986004 x 105; km2/s2. ωE; Earth angular speed; 7.292115 x 10-5; rad/s. Step 1: Derive the orbital parameters (a, e, i, Ω, ω, M0) Step 1-0: Convert the initial position/velocity state vectors from ECEF format to ECI format r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF = v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF Step1-1 Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h) r=r0=r0,x2+r0,y2+r0,z2 v=v0=v0,x2+v0,y2+v0,z2 vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x h=h=hx2+hy2+hz2 Step 1-2 Inclination (INC, i) K=[001] i=cos-1h⋅Kh=cos-1hzh Note the range of INC is between 0 and π (radian). Step 1-3 Right Ascension of the Ascending Node (RAN, Ω) n=nxnynz=K×h=-hyhx0 n=n=nx2+ny2+nz2 Ω=cos-1nxn,ny≥02π-cos-1nxnny<0 Note the range of RAN is between 0 and 2π (radian). Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17. - Step 1-4 Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P) e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT e=e=ex2+ey2+ez2 a=h2μ(1-e2) Period of the satellite around earth, P (sec), is given by: P=2πa3μ Step 1-5 Argument of Periapsis (AP, ω) ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0 Note the range of AP is between 0 and 2π (radian). - Step 1-6 Mean Anomaly at time 0 (MA, M0) True Anomaly at time 0 (ν0): ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0 Eccentric Anomaly at time 0 (E0): E0=2tan-11-e1+etanv02 Mean Anomaly at time 0 (M0): M0=E0-esinE0 Note the range of M0 is between 0 and 2π (radian). Step 2: Determine UE position Input values: Parameters; Unit; Description. UElatitude; degree; Initial UE latitude. UElontitude; degree; Initial UE longitude. UEaltitude; km; Initial UE altitude. Output values: Parameters; Unit; Description. UEt,xUEt,yUEt,z; km; UE position state vector at time t in Earth-centered earth-fixed frame (ECEF). Constant parameters: Parameters; Description; Values; Unit. aE; Earth radius; 6378.137; km. eE2; Square of Earth eccentricity; 6.694385 x 10-3. ωE; Earth angular speed; 7.292115 x 10-5; rad/s. ϕ=π180UElatitude λ0=π180UElongitude N=aE1-eE2sin2ϕ UEt,x=N+UEaltitudecosϕcosλ0 UEt,y=N+UEaltitudecosϕsinλ0UEt,z=N1-eE2+UEaltitudesinϕ - Step 3 Determine the satellite position and velocity at time t (sec) - Step 3-1 Mean Anomaly at time t (Mt): Mt=M0+2πtP=M0+tμa3 Step 3-2 Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation Build the Kepler’s equation between Mt and Et: Mt=Et-esinEt - Step 3-3 Derive True Anomaly at time t (νt) νt=2tan-11+e1-etanEt2 Note the range of νt is between 0 and 2π (radian). - Step 3-4 Convert the orbital elements to the state vector rt,xwrt,ywrt,zw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0 vt,xwvt,ywvt,zw=μh-sinνt 1+cosνt0 Ωt=Ω+ωE⋅t rt,xrt,yrt,z=cos-ω-sin-ω0sin-ωcos-ω00011000cos-i-sin-i0sin(-i)cos(-i)cos-Ωt-sin-Ωt0sin-Ωtcos(-Ωt)0001rt,xwrt,ywrt,zw vt,xvt,yvt,z=cos-ω-sin-ω0sin-ωcos-ω00011000cos-i-sin-i0sin(-i)cos(-i)cos-Ωt-sin-Ωt0sin-Ωtcos(-Ωt)0001vt,xwvt,ywvt,zw - Step 4 Doppler shift and propagation delay Input values: Parameters; Unit; Description. UEt,xUEt,yUEt,z; km; UE position state vector at time t in Earth-centered earth-fixed frame (ECEF). rtECEF=rxtECEFrytECEFrztECEF; km; Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF). vt=vxtECEFvytECEFvztECEF; km/s; Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF). fc; Hz; Carrier frequency (e.g., 2.0 x 109 for band n256). Output values: Parameters; Unit; Description. Doppler; Hz; Doppler shift at time t. Delay; sec; Propagation delay at time t. Constant parameters: Parameters; Description; Example of values; Unit. c; Speed of light; 300000; km/s. ρt=ρt,xρt,yρt,z=rt,xECEF-UEt,xrt,yECEF-UEt,yrt,zECEF-UEt,z Range(t)=ρt=ρt,x2+ρt,y2+ρt,z2 RangeRate(t)=rtECEF⋅vtECEFRnage(t)=rt,xECEF⋅vt,xECEF+rt,yECEF⋅vt,yECEF+rt,zECEF⋅vt,zECEFRange(t) Delay(t)=Range(t)c Doppler(t)=-RangeRate(t)×fcc

Template of calibration for each step to generate the doppler shift and propagation delay

    - For example, the following is the template to verify the transformation of position/velocity state vector in ECEF format to orbit information in ECI format

Table 1: Initial ephemeris information in format of position/velocity state vector in ECEF

| Position / Velocity | ECEF Frame |
| --- | --- |
| X | -3315105.300000 [m], |
| Y | 5396426.100000 [m] |
| Z | 2941944.200000 [m] |
| VX | 1318.200000 [m/s] |
| VY | -2889.840000 [m/s] |
| VZ | 6846.540000 [m/s] |

Table 2: converted orbital parameters (Keplerian orbital elements) in ECI format according to Issue 1-1-1 Step 1.

| Keplerian orbital 6 elements | Company (Anritsu) | Company (Huawei) | Company (Ericsson) |
| --- | --- | --- | --- |
| A | 7.039754380856870E+3 [ km ] | 7.039754380856869e+06 [m] | 7.039754378890123e+06 [m] |
| e | 8.691235939278610E-3 | 8.691235939278730e-03 | 8.691235683711872e-03 |
| I | 8.415808332004220E+1 [deg.] | 1.468835646102458e+00 [rad] (8.415808332004224e+01 [deg]) | 8.415808338321723e+01 [deg] |
| Ω | 1.188388944155350E+2 [deg.] | 2.074129986980988e+00 [rad] (1.188388944155350e+02 [deg]) | 1.188388944452215e+02 [deg] |
| ω | 2.203829430398870E+0 [deg.] | 3.846407971280572e-02 [rad] (2.203829430398539e+00 [deg]) | 2.203828720716626e+00 [deg] |
| M (Mean Anomaly at time 0) | 2.246638007502270E+1 [deg.] | 3.921123033136071e-01 [rad] 2.246638007502330e+01 [deg] | 2.246638078165695e+01 [deg] |
| ωE (Earth angular speed) |  |  | 7.2921150e-5 [rad/s] |
| earth gravitational constant |  |  | 3.986004418e5 [km^3/s^2] |