3GPP TSG-RAN WG4 Meeting #113						R4-2419824

Orlando, US, November 18 – 22, 2024

Source:	Nokia

Title:	TP to TR 38.908 clause 6 – Conformance testing

Agenda item:	6.11.4

Document for:	Discussion

Introduction

This is text proposal to Technical Report TR 38.908 v0.1.0. with clause 6 for conformance testing and procedures.

TP to TR 38.908

<Start of TP>

# 6	Conformance testing requirement and procedures

## 6.1	General

The key requirements of ITU Resolution 220 [2] are:

- compliance with values of Expected EIRP (EEIRP) above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to ITU Resolution 220 [2].

The EEIRP calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

## 6.2	The calculation for EEIRP mask

    - 6.2.1. Angle definition

The angles in the spherical coordinate system used in this section is as shown in figure 6.2.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 6.2.1-1: Angle definition for EEIRP

    - 6.2.1 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θHL and θHH as shown in Figure 6.2.1-1.

Figure 6.2.1-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRP EIRPθ,φ is be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

    - 6.2.1.1 Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 5.1.1.1-1 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over two discrete elevation angles with equal number of points in azimuth.

Figure 6.2.1.1-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.2.1.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.2.1.1-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθHLi , θHHi=12π360°×sinθHHi-sinθHLiθLθH-180°π180°πEIRPθ,φ cosθ ∆φ ∆θ    ,                     (2)

By substituting the following:

dφ≈∆φ=2π360°M

dθ≈∆θ=(θHHi-θHLi)Nπ2N

the overall equation can be written as:

EIRPθHLi , θHHi=π(θHHi-θHLi)×π180°2MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (3)

where

θn	is the nth midpoint between the elevation intervals (midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°).

φm	is the mth midpoint between the azimuth intervals

nL	is the lowest elevation point within the θHLi , θHHi range

nH	is the highest elevation point within the θHLi , θHHi range

M	is the number of azimuth points within the (-180°, 180°) range

N	is the number of elevation points within the θHLi , θHHi range

Note: The equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

<End of TP>