# 3GPP TSG-RAN WG4 Meeting #113	R4-2420082

# Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.15.4

Source:	Moderator (Apple)

Title:	Ad-hoc minutes for NR_RRM_Ph5 WI

Document for:	Information

# Introduction

- Mainly discuss on

  - Topic #1 (BSF reduction): Issue 1-1-3, 1-1-4, 1-1-5, 1-3, 1-2-1, 1-1-2, 1-2-3

  - Topic #2 (CSSF reduction): Issue 2-5, 2-7, 2-1, 2-2, 2-4-1, 2-4-2, 2-4-3, 2-4-4, 2-3.

# Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor (7.15.2.1)

Issue 1-1-3: searcher number assumption for RRM measurement with two panels activated on single carrier

- Option 1 (CATT, NTT DCM, CTC, Apple, Samsung, ZTE, QC, vivo): For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

- Option 2 (LGE): For Multi-Rx based L3 measurement, 2 searcher is assumed for single carrier.

- Option 3 (HW, Ericsson): RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion.

- Recommended WF

[Moderator]: check if option 1 is agreeable.

Agreement:

Directly discuss the SSB processing time.

[LGE]: we can compromise to option 1.

[HW]: the intention is not to change the CSSF table but only for the processing time, we can accept that.

[vivo]: the searcher number not only related with the CSSF table, but also impact the other RRM requirement when extend to CA case.

[Nokia]: this searcher discussion will not help here, focus on the processing time. When we go to CA, we can discuss searcher number.

[Apple]:we need baseline assumption for UE processing capability when 2 panels are used on this carrier, this number of searcher is the reference we can use.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

- Option 1 (CATT, CTC, Apple, vivo, Nokia): if option 1 is adopted in issue 1-1-3, additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

  - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

  - where TSSB_processing = 2 ms

- Option 2 (CATT): For handover and RRC re-establishment/RRC Connection Release with Redirection, no additional processing delay is needed.

- Option 3 (NTT DCM): SSB processing delay/time for this BSF optimization should be same as the legacy requirements for all the cases because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB.

- Option 4 (OPPO): Proposal 3: For scenario 1/2/3/4/7, the additional SSB processing delay is M*TSSB_processing, and for scenario 5, the additional SSB post-processing delay is M* Tmargin, where TSSB_processing, and Tmargin can be up to 2ms.

- Option 5 (Ericsson): Extra processing time can be applied to handover interruption time, e.g. Tmargin, if necessary. No extra processing time for cell identification delay.

- Option 6 (QC):

Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue): - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + M * Tmargin ms - Where, - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios. - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

- Recommended WF

[Moderator]: FFS the following summarized options:

- For scenario 1/2/3/4

  - Option 1: additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

  - Option 2: the additional SSB processing delay is M*TSSB_processing, where TSSB_processing, can be up to 2ms.

  - Option 3: no need to add additional processing time in the delay requirement.

[QC]: the delay is already very long, why we still need this 2ms.

[Apple]: since the delay is that long, 2ms processing time shall be acceptable

[Nokia]:preference is option 3, 2ms shall be the thing we shall worry because the total delay is very long.

[Ericsson]: is that any essential reason to add 2ms processing time?

[Apple]: if the SMTC window is at the very end of the periodicity, 2ms processing time shall be used after the SMTC window.

[HW]: support option 1.

Agreement:

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

- For scenario 5

  - Option 1: to consider an additional processing delay (2ms) in the handover delay equation

  - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

  - Option 3: no need to add additional processing time in the delay requirement.

[QC]: if UE can have multiple simultaneous Rx beams, the processing time shall be scaled by parallel Rx beam number

[HW]: what’s the difference from scenario 1/2/3/4

[QC]: the identification time is already very long for scenario 1/2/3/4

[Apple]: we don’t need to scale the Rx beam, because we only support 2 simultaneous Rx beams.

- For scenario 7

  - Option 1: to consider an additional processing delay (2ms) after the last SMTC periodicity

  - Option 2: the additional SSB processing delay is M*TSSB_processing, where TSSB_processing, can be up to 2ms.

  - Option 3: SSB processing time does not need to be introduced to handle optimized Rx BSF in scenario 7

Agreement:

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

- Option 1 (CATT, NTT DCM, LGE, CMCC, CTC, Nokia, Samsung): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

- Option 2 (Ericsson):

  - L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX.

  - RAN4 to study whether DRX can be ignored when FBS is valid.

- Recommended WF

  - [Moderator]: Is option 1 agreeable?

Agreement:

L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

- Option 1(Ericsson, Samsung):

  - to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional.

- Recommended WF

  - [Moderator]: is option 1 agreeable.

Agreement:

- Discuss this capability detail by end of the core part

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- Option 1 (CATT, CMCC, CTC, Apple, Intel, vivo, Nokia, Ericsson, Samsung, ZTE, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability

  - Option 1a (CATT, CMCC, CTC, ZTE):

    - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

  - Option 1b (Intel):

    - Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

  - Option 1c (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4

  - Option 1d (Nokia):

    - At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported.

    - RAN4 to cover the impact of Rel-19 BSF reduction through an additional scaling factor across all supported requirements. This factor may depend on UE indicated N if multiple values for N are supported.

  - Option 1e (Samsung): To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

- Option 2 (Xiaomi):

  - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection.

- Option 3 (LGE, Intel, vivo): Introduce the UE capability to indicate the reduced M value (M=sample*BSF),

  - Option 3a (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value

  - Option 3b (Intel): Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

  - Option 3c (vivo): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Option 4 (vivo):

  - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Recommended WF

  - [Moderator]: Check if option 1/1a is agreeable

    - Option 1 (CATT, CMCC, CTC, Apple, Intel, vivo, Nokia, Ericsson, Samsung, ZTE, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability

      - Option 1a (CATT, CMCC, CTC, ZTE):

        - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Agreement:

For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability:

- FFS: details of the capability

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

- Option 1 (NTT DCM):

  - For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided.

  - For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed..

- Option 2 (LGE, OPPO, Ericsson, MTK, CTC):  UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

  - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Option 3 (HW, Samsung): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

- Option 4 (vivo): RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions.

- Option 5 (Ericsson): RAN4 to study the below FBS deactivation conditions:

  - After the UE have activated FBS:

    - The condition of activating FBS is invalid.

      - Define a minimal during time for FBS/non-FBS.

  - The condition of activating FBS still is valid but,

    - L3 measurement with FBS has been conducted for a time period with respect to a pre-defined measurement period or time.

    - Internal overheating is detected or other limitation from UE perspective. (similar as option 2)

  - Meet activation conditions but not activate FBS yet:

    - Internal overheating is detected or other limitation from UE perspective. (similar as option 2)

  - UE shall report NW of deactivating FBS once the deactivation condition is met.

- Option 6 (Samsung): Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

    - Dynamic indication; Semi-static configuration

- Option 7 (Samsung, Ericsson):

  - Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Option 8 (Samsung):

  - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- Option 9 (Nokia):

  - When L3 event-triggered reporting conditions are used as a trigger to start FBS (e.g. event A2), UE is allowed to return back to normal beam sweeping when event condition (e.g. A2) is no longer met, or when another event condition (e.g. A1) is met, depending on network configuration.

- Recommended WF

[Moderator]: discuss the following options(wording can be revised/polished) and companies can add more details if needed.

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

- Option 2 (LGE, OPPO, Ericsson, MTK, CTC):  UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

  - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

- Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

- Option 1 (CATT, Apple):

  - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

- Option 2 (Ericsson): Regarding the transition time,

  - For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met.

- FBS activation delay shall at the least includes below items.

  - When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command.

DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS

  - Where:

    - TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

    - TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS.

    - Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS.

  - For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed.

- Proposal (ZTE):

  - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options:

    - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens.

    - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement.

- Recommended WF

  - TBA

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions):

Agreement in last meeting: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

- Option 1(CATT, LGE, CMCC, Apple, HW, vivo, OPPO, ZTE, MTK, QC, Samsung): RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - Option 1a(CATT, OPPO, QC, Samsung): The existing relaxed measurement criteria including low mobility and not at cell edge can be reused to define trigger conditions of Multi-Rx operation for L3 measurement.

    - For a UE supporting Multi-Rx operation,

      - When both low mobility criterion and not at cell edge criterion are fulfilled, UE is required to perform the L3 measurement with legacy beam sweeping factor, i.e., 8. (CATT, QC)

      - When at least one of the criteria is not fulfilled, UE is required to perform the L3 measurement with Multi-Rx operation, i.e., enhanced beam sweeping factor.

  - Option 1b(CMCC, HW): When the measured RSRP is lower than a network configured threshold, FBS is activated. Otherwise, FBS is deactivated

    - where the serving Cell could be PCell or a SCell which is up to NW configuration (HW)

  - Option 1c(vivo): For scenario 1/2/3/4, RAN4 to consider the Qin+XdB as the criterion, where X is configure by RRC signalling, to evaluate the serving cell quality for activating/de-activating the L3 BSF.

  - Option 1d(ZTE): A relative RSRP/SINR threshold between the serving cell measurement and neighbour cell measurement is preferred as the condition to activated or deactivate the fast L3 measurement.

- Option 2(CATT, Apple, OPPO, MTK, QC, Samsung): RAN4 to introduce UE mobility status based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - Opponent to option 2 (CMCC, Intel): it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS

  - Option 2a(CATT, OPPO, QC, Samsung): The existing relaxed measurement criteria including low mobility and not at cell edge can be reused to define trigger conditions of Multi-Rx operation for L3 measurement.

    - For a UE supporting Multi-Rx operation,

      - When both low mobility criterion and not at cell edge criterion are fulfilled, UE is required to perform the L3 measurement with legacy beam sweeping factor, i.e., 8. (CATT, QC)

      - When at least one of the criteria is not fulfilled, UE is required to perform the L3 measurement with Multi-Rx operation, i.e., enhanced beam sweeping factor.

  - Option 2b(MTK): On UE mobility status, RAN4 to consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

  - Option 2c(QC): High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T

- Option 3(Nokia, Ericsson): Use L3 event-triggered reporting conditions for triggering FBS

  - Option 3a (Nokia):

    - Use at least event A2 to trigger FBS

    - RAN4 to discuss whether any additional signaling would be needed from network side to indicate that a L3 reporting event is expected to trigger FBS, and send an LS to RAN2 based on the conclusion.

  - Option 3b (Ericsson): UE shall report FBS validity to NW after activating FBS with respect to the NW RRC signaling.

- Option 4(Apple): RAN4 to introduce UE power consumption based triggering condition to activation/deactivate multi-Rx for L3 measurement

  - Opponent to option 4 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced

- Option 5(Apple): RAN4 to introduce UE QoS based triggering condition to activate/deactivate multi-Rx for L3 measurement

- Option 6(Nokia): Conditional handover configuration is used to trigger FBS.

  - Option 6a (Ericsson): For CHO, FBS is applied during Tmeasure after the condition for handover is met.

- Option 7(Xiaomi): Activating/Deactivating FBS for L3 measurements (intra/inter-frequency) can upon the conditions without additional NW signalling (e.g. DRX configuration).

- Option 8(Intel): Conditions for UE to apply multi-Rx reception and reduced beam sweeping factor include the below aspects:

  - Multi-Rx simultaneous reception active, following Rel-18 definitions as much as reasonable, including,

    - Group based beam reporting is configured for the UE not long ago prior to the expected L3 reporting,

  - Measurement configurations meet processing limitations, including,

    - Single carrier is configured,

    - Measurement periodicity is large enough compared to the number of SSB to be measured in a burst,

    - Existing side conditions for L3 measurements.

  - Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions.

- Option 9 (Ericsson): BFD and/or RLM status may trigger UE to enable FBS.

  - If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

  - If beam failure or radio link failure is detected, FBS may be applied.

- Option 10 (QC): Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

  - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

  - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

- Option 11(CATT): Send LS to RAN2 about the criteria/thresholds for triggering Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

- Recommended WF

[Moderator]: discuss the following options(wording can be revised/polished) and companies can add more details if needed.

- Option 1(CATT, LGE, CMCC, Apple, HW, vivo, OPPO, ZTE, MTK, QC, Samsung): RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - If option 1 is agreed, discuss whether option 1a is agreeable

Agreement in online session:

- RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

- Option 2(CATT, Apple, OPPO, MTK, QC, Samsung): RAN4 to introduce UE mobility status based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - Opponent to option 2 (CMCC, Intel): it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS

  - If option 2 is agreed, discuss whether option 2a is agreeable.

Agreement:

- Not support UE mobility status based triggering condition in Rel-19.

- Option 3(Nokia, Ericsson): Use L3 event-triggered reporting conditions for triggering FBS

  - Discuss whether option 3 is agreeable.

- Discuss Option 3b and option 10 to decide if the UE FBS status report to network is needed.

- Other options can be discussed online based on companies’ comments.

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

- Option 1 (CATT):

  - The reduced Rx BSF can be applied to the following formula according to the trigger conditions to be specified:

    - TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

    - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

    - Tsearch for FR1-FR2 HO and FR2-FR2 HO;

    - Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

    - Tidentify-NR for RRC connection release with redirection to NR.

  - The reduced Rx BSF can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

- Option 2 (NTT DCM): For each scenario, the highlighted factors should be replaced with capability value:

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

- Option 3(Nokia): To apply the scaling factor Nreduced to the fixed value of Tidentify_intra/inter_NR e.g., MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i).

- Recommended WF

  - [Moderator]: can use option 2 as starting point for discussion.

Issue 1-4-1: FFS: other requirement imapcts

- Proposal (Ericsson):

  - RAN4 to study the UE behaviour, provided the UE deactivates FBS and then the UE determines that the deactivation condition is invalid or the activation condition is met again. There are two exemplary behaviors as follow:

    - UE directly re-activates FBS.

    - UE keeps FBS deactivated and awaits until the new RRC signaling enabling the event-triggered activation procedure.

  - RAN4 to study whether the existing ReportInterval  shall be extended to contain a smaller value for the measurement with FBS enabled.

- Recommended WF

  - TBA

# Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.15.2.2)

Issue 2-5: feature capability of L3 measurement delay reduction by optimizing CSSF for tentative solution 1 and 2

[Moderator]: the 3 searcher related capability is discussed in the solution 3 of issue 2-4-3

- Option 1 (CATT): The feature capability depends on the solutions and can be discussed at the end of core part:

  - For solution 1 and solution 2, a new UE capability is needed to indicate the UE support of new solution.

  - For solution 3, the existing per-FR gap capability can be reused..

- Option 2 (Nokia, CTC, Apple, Ericsson, Samsung): RAN4 to introduce a new individual capability for CSSF reduction in R19.

  - Option 2a (CTC, Apple, HW, Ericsson, Samsung): delay the capability discussion to the end of the core part.

- Recommended WF

  - Moderator note: Check if following option is agreeable:

Agreement:

  - RAN4 to introduce new individual capability(s) for CSSF reduction in R19.

    - delay the capability detail discussion to the end of the core part.

Issue 2-7: Other WID scope discussion

- Proposal  1(CMCC, ZTE, CTC, Ericsson): after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

- Recommended WF

  - [Moderator note]:

    - Check if proposal 1 is agreeable?

Agreement:

- after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

The WID RP-242380 agreed in the RANP#105 meeting is:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

- Option 1 (CATT, Xiaomi, CMCC, Apple, ZTE): RAN4 to follow the revised WID (WID RP-242380) in RANP#105 meeting, and no need to have further discussion on this issue.

- Option 2 (Nokia):

  - A UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

  - If a UE is configured with CSSF optimization (CA or DC), the UE is not required to operate with the multi-Rx based L3 measurement.

  - The independency between CSSF optimization and multi-Rx L3 measurement shall be clear. This would allow the discussion on the CSSF optimization solutions to proceed without impact from the multi-Rx L3 measurement discussion.

- Option 3 (vivo):

  - RAN4 to develop CSSF optimization solutions without considering multi-Rx and further study if it is applicable to multi-Rx scenario after more progress are made.

- Option 4 (Ericsson): The solution of CSSF optimization is same to the below cases:

  - A UE not operating with multiple-Rx based simultaneous L3 measurement

  - A UE not capable of multiple-Rx based simultaneous L3 measurement

- Option 5 (Samsung):

  - Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. For feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist

  - Text revision Alt:

    - Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx simultaneous reception features/capabilities

    - If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

- Recommended WF

  - [Moderator]: to discuss if option 1 is agreeable, and also to discuss if any extra information from option 2/3/4/5 need to be added in this agreement.

Agreement:

- RAN4 to follow the revised WID (WID RP-242380) in RANP#105 meeting, and no need to have further discussion on this issue.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement in RAN#112 meeting: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

- Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority.

- Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

  - Aspect 1: SSB based Intra-frequency measurement without MG, including:

    - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

    - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

  - Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered.

- Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion.

- Recommended WF

  - [Moderator]: As a compromise, is option 3 agreeable?

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

- Option 1 (CATT, Nokia, CMCC, CTC, Apple, vivo, Ericsson, ZTE, MTK): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band

  - Option 1A (CATT, Apple, vivo, ZTE): Reuse the legacy principle for neighbour cell measurement (UE can measure one serving CC per band if multiple serving CCs are in the same band).

    - Measure PCC if PCC in band or

    - Measure PSCC if PSCC in band or

    - Measure SCC with neighbor cell MO or

    - UE implementation

  - Option 1B (Nokia, CMCC, CTC, Ericsson, ZTE): Based on network indication

    - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band.

    - Network indicates which of the MOs to be measured. The UE is required to measure the indicated CCs i.e. MO and network can trigger measurement reporting on any of the MOs as in legacy

    - In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured, but the rules shall be studied further. (Ericsson)

    - Further discussion is needed to justify the introduction of a new NW indication for Option 1B (Huawei)

- Recommended WF

  - Moderator option 2 for merging all options:

  - Option 2: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band

    - Network indicates which of the MOs to be measured. The UE is required to measure the indicated CC, i.e. MO, and network can trigger measurement reporting on any of the MOs as in legacy

    - In case of no NW indication, UE is allowed to determine the CC in the band to be measured,

      - Reuse the legacy principle for neighbour cell measurement

        - Measure PCC if PCC in band or

        - Measure PSCC if PSCC in band or

        - Measure SCC with neighbor cell MO or

        - UE implementation

Session chair: any companies have concern on the direction in option 2, while the details to be further discussed.

Intel, OPPO, HW, QC, Samsung have concern.

[Intel]: can this new indication be applied to neighbor cell measurement?

[Samsung]: if no indication, we are not sure if neighbor cell can be included here

[QC]: we have concern on the network indication. If this indication is needed, then R15 IE shall be set as “need R” rather than “need M”.

[Wayforwad]:

FFS the following solution:

  - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band or

      - Measure SCC with neighbor cell MO or

      - UE implementation

    - FFS whether indication is per-UE or per-band

      - Other options are not precluded.

  - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

- Proponent companies: MTK, HW, Apple, CMCC, CATT, vivo, ZTE, Nokia, Xiaomi, OPPO

- Opponent companies: Samsung, Ericsson

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

- Option 1 (CTC, Apple, HW, vivo, OPPO, ZTE, Ericsson): UE can change the searcher occupancy ratio of measurement among the measured CCs

  - Option 1A(CATT): UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g., NW can indicate the priority of the SCCs. If the priority is not provided, the measurement order is up to UE implementation which is same as the legacy.

  - Option 1B(Nokia, CATT): The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

  - Option 1C(CTC, Apple, HW, vivo, OPPO, ZTE, Ericsson): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

    - (Apple) The conditions can be not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

    - (vivo) Dynamic ratio changed as, e.g.,

      - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

      - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

      - FFS details and X value, e.g., 50.

      - FFS whether specific conditions for this ratio change are needed

    - (Ericsson, ZTE) NW indicates the ratio between PCC and SCC

      - When two searchers are applied, NW to indicate the candidate PCC/PSCC/SCC(s) or the frequency range of the PCC/PSCC/SCC(s) which applies a specified percentage X% of the searcher resource, the other SCC(s) together with PCC/PSCC share the other (1-X)% of the searcher resource. If no NW indication, UE follows the legacy searcher sharing rules. (Ericsson)

      - Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point. (Ericsson)

- Recommended WF

  - Moderator option 2 for merging all options based on option 1C, please CATT and Nokia check if it’s acceptable

  - Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

    - NW indicates the ratio between PCC, PSCC and SCC.

      - Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point.

    - Details is FFS

Wayforward:

FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1:

  - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement.

    - FFS whether network indication is needed

      - Details is FFS

    - FFS on whether and how PCC and PSCC resource sharing is supported

[QC, Nokia, MTK, CATT]: don’t want to touch PCC and PSCC.

[Ericsson]: if it’s based on network indication can we use PCC/PSCC resource sharing?

[QC]: no.

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

In WID: - FR2-1 SSB based L3 measurement delay reduction for connected mode - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 Agreement in last meeting: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

- Option 1(CATT, CMCC, Nokia, HW): consider to use 3 searchers for CSSF reduction.

  - Option 1A(CATT): For the solution based on 3 searchers assumption, define the applicable scenarios (e.g., all the measurements are outside gap and both FR1 and FR2 measurement objects are configured).

  - Option 1B(Nokia):

    - To define a new UE capability to indicate the support for 3 searchers.

    - For UE indicating the new UE capability, it shall apply the measurement requirement defined based on 3 searchers.

    - The 3 searcher solution is independent from per-FR gap or per-UE gap capability.

    - The solution based on 3 searchers should not relax the measurement requirements assuming 2 searchers.

    - RAN4 to focus the discussion on how the 3 searchers are assumed for the SSB-based measurement outside gaps.

  - Option 1C(Apple): for 3 searchers to extend to CSSF outside MG, RAN4 only considers UE is using 3 searchers for FR1 scenario only (FR1 NR CA or FR1 NR-DC).

    - If companies cannot achieve consensus, RAN4 work can stick to the baseline assumption in WID, i.e., 2 searchers.

  - Option 1D(HW):

    - To facilitate the discussion of this issue, we recommend utilising the Rel-15 version of the CSSF as a baseline for solution discussion. Once the solution is settled, we can then explore the incorporation of the features introduced in Rel-16, Rel-17 and Rel-18 into CSSF table.

    - The 3-searcher solution can be utilised under the following conditions

      - When all MOs need to be measured outside the gap.

      - Only for the FR1 +FR2 CA scenario.

      - Any other conditions can be FFS.

  - Option 1E (Ericsson): When 3 searchers is supported,

    - 1 searcher for PCC,

    - 1 searcher for other SCCs and frequency layers following legacy equalling sharing;

    - 1 additional searcher is used for the specific PCC/PSCC/SCCs based on NW indication.

    - NW indicates the cell list which applied the measurement by the additional searcher.

    - RAN4 to discuss the use limitation on this additional searcher.

  - Option 1F (QC): RAN4 introduces a new optional capability to allow UE to signal support of three searchers for CSSF enhancement.

    - Note: UE can indicate the support of different levels of three searchers in the new capability:

    - Indication 00: three searchers are not supported.

    - Indication 01: three searchers are supported, in total. However, at most two searchers are supported in FR2, i.e., UE supports combination of {one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

    - Indication 10: UE can support three searchers in total and in FR2, too. That means, UE supports the combination of {three FR2 searchers, one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

- Option 2 (Ericsson, Apple): RAN4 to check whether the 3rd searcher only can be used for CSSF optimization when no gap is configured.

- Option 3 (MTK, Apple, OPPO): RAN4 to prioritize optimizing CSSF outside gap based on 2 searchers as a baseline assumption, and whether to optimize CSSF based on 3 searchers can be postpended after concluding 2 searchers.

- Recommended WF

  - [Moderator]: Discuss Option 1/2/3. If option 1 is agreed, before the discussion of solution details, recommend to firstly discuss the scenario/conditions to support 3 searchers, i.e., option 1B/1C/1D.

Agreement:

RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is:

    - FFS: FR1 scenario (CA and NR-DC) only.

    - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

    - FFS: All scenarios.

Issue 2-4-4: Other tentative solutions

- Option 1 (Nokia): RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

- Option 2 (Nokia, OPPO): To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

- Recommended WF

  - [Moderator]: Recommend proponents of option 1/2 to ask for support from other companies, we may need to prioritize the solutions for future meetings. Solution 1/2/3 have most interests so far.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Option 1 (Nokia): To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

- Option 2 (Ericsson): We suggest covering SA, EN-DC and NE-DC in the scope without deprioritization any of them.

- Recommended WF

  - TBA