# 3GPP TSG-RAN WG4 Meeting #113	R4-2420098

# Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.22.3

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Introduction

Note: The detail open issue list is captured in the moderator summary R4-2418282.

# Topic #1: On-demand SSB(OD-SSB) requirements

### Sub-topic 1-1: General On-demand SSB(OD-SSB) requirements

Agreement:

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective.

  - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective.

- Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

Issue 1-1-2: L1 measurement before SCell activation

- Proposals

  - Option 1: L1 measurements are not supported for deactivated SCell.

  - Option 2: Wait RAN1’s further clarification on the applicable scenario.

### Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement

Agreement:

For the case of CSSF = 1:

- OD-SSB periodicity regardless of measCycleSCell and DRX cycle.

  - FFS whether a lower bound will be specified.

FFS for the other cases of CSSF.

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement

- Option 1: The same CSSF as legacy

- Option 2:

  - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements.

    - RRM requirements on PSCC and other SCCs are not applicable during the time-window.

  - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed.

Other option is not precluded.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined?

Agreement:

The time window is based on the number of OD-SSB samples and UE provided one valid report.

- The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy

- UE should have enough number of OD-SSB samples to achieve the measurement requirement.

- The number of OD-SSB is FFS.

- FFS whether UE measurement reporting is needed

Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2?

FFS.

### Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 1-1: The measurement periodicity is OD-SSB periodicity

  - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB

- Option 2: UE always follow legacy requirements based on measCycleSCell

- Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

### Sub-topic 1-4: OD-SSB based SCell activation

Issue 1-4-1: The known cell condition – measurement reporting

Agreement:

- In case 1, known/unknown condition should be defined based on only OD-SSB.

- In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB.

  - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue.

  - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB.

  - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity

Agreement:

- In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity.

  - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity.

- In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation.

  - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply.

- Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

### Sub-topic 1-5: Others

Issue 1-5-1: The type of OD-SSB based SCell activation

FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios.

- Option 1: Direct SCell activation at SCell addition

- Option 2: Multiple SCells activation

- Option 3: Deactivated PUCCH SCell activation

- Option 4: Rel-17 Fast SCell Activation

- Option 5: Rel-18 SCell Activation with the L3 reporting during activation