3GPP TSG-RAN WG4 Meeting #113	 														R4-2420101

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.24.3

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	Approval

# Topic #1: Inter-CU

#### Issue 1-1: R19 inter-CU subsequent LTM

<Agreement>

- The Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM.

- Note: this can be revisited if any critical issue identified.

# Topic #2: Event triggered L1 measurement reporting

#### Issue 2-1: RRM scope of Event triggered L1 measurement reporting

<Agreement>

- Confirm that CSI-RS based event triggered L1 reporting is included in RAN4 scope of this work item.

#### Issue 2-2: starting point in event triggered L1 report requirements

<Agreement in RAN4#112bis>

- Use the following as starting point. Wording can be discussed in CR.

  - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

<Recommended WF>

- Issue is closed.

#### Issue 2-3: ending point in event triggered L1 report requirements

<Agreement>

- Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface.

#### Issue 2-4: periodic report and single report

<Agreement>

- RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR.

#### Issue 2-5: Event triggered reporting delay for L1 reporting

<Agreement>

- Agree on the following high level proposal:

  - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells].

    - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0.

  - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

#### Issue 2-6: impact of time-to-trigger (TTT) on RRM

<Candidate options>

- Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi)

<Recommended WF>

- Continue discussion.

#### Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category

<Candidate options>

- Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO)

- Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson)

- Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

<Recommended WF>

- Continue discussion.

#### Issue 2-8: whether to define a separate capability for supported number of events triggered L1 measurement reporting

<Candidate options>

- Option 1: Yes (MTK, Samsung, CMCC)

- Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC)

  - Option 2a: RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support. (E///)

- Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

<Recommended WF>

- Continue discussion.

#### Issue 2-9: how to define requirements for reporting criteria per measurement category

<Candidate options>

- Proposal 1: For the requirements for reporting criteria per measurement category, whether ‘SS-RSRP and CSI-RSRP’ in current description include both L1-RSRP and L3 RSRP should be clarified. (CATT)

- Proposal 2: RAN4 may not need to additionally emphasize and add L1-RSRP separately for measurement category of intra-frequency and inter-frequency. (CATT)

- Proposal 3: (MTK)

  - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

    - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

    - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

- Proposal 4: Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM. (vivo)

- Proposal 5: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC)

- Proposal 6: RAN4 to agree that Ecat capability is not shared between  L3 events and LTM L1 events. (E///)

<Recommended WF>

- Continue discussion.

#### Issue 2-10: whether and how to coordinate with R19 MIMO

<Candidate options>

- Option 1: (MTK)

  - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

  - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

- Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple)

- Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia)

<Recommended WF>

- Continue discussion.

#### Issue 2-11: L1 filtering assumption in event evaluation

<Candidate options>

- Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///)

  - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

  - Otherwise, there is no impact on RAN4.

- Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW)

  - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC)

<Recommended WF>

- Continue discussion.

#### Issue 2-12: beam level or cell level measurement result for new LTM event evaluation

<Candidate options>

- Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for LTM event evaluation. (CATT, Xiaomi, Samsung, CMCC, ZTE, Apple, CTC, vivo)

- Option 2: RAN4 should not discuss whether the event-based L1 measurement report should be evaluated per beam or per cell. The requirement can be defined in a way that the evaluation is agnostic to the granularity. If needed, it can be properly incorporated into the definition of the event condition, which is the starting point of the event-based measurement report requirement. (QC)

<Recommended WF>

- This issue is covered by issue 2-5. Issue is closed.

#### Issue 2-13: accuracy in event triggered L1 report

<Agreement in RAN4#112bis>

- for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

- for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

<Recommended WF>

- No new agreement is needed. Issue is closed.

#### Issue 2-14: where to capture event triggered L1 report requirement

<Candidate options>

- Option 1: The event triggered reporting for LTM L1 measurement requirements can be defined in clause 9.14.3 and clause 9.15.3. (CATT, ZTE, HW, CTC, vivo)

- Option 1a: confirm that new requirements for SSB based event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. For CSI-RS based event triggered L1 measurement reporting, it will be captured in the new section for requirements of CSI-RS based L1 measurement on neighbor cell. (Apple)

<Agreement>

- The event triggered reporting for SSB based L1 measurement requirements can be defined in clause 9.14.3 and clause 9.15.3, respectively for intra-frequency and inter-frequency.

- FFS on event triggered reporting for CSI-RS based L1 measurement.

#### Issue 2-15: Impact of UL resource waiting time.

<Candidate options>

- Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi)

  - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore.

- Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo)

- Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///)

  - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel.

<Recommended WF>

- Continue discussion.

#### Issue 2-16: Impact on other LTM requirements

<Candidate solutions>

- Option 1: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: (Nokia, E///)

  - Known TCI state condition for early TCI state activation and cell switch

  - Applicability of cell switch delay requirements

- Option 2: RAN4 to postpone discussions on the options listed under ‘Issue 1-12: Impact on other LTM requirements’ in the WF (R4-2416862) approved at RAN4#112bis until the group achieves a certain level of detailed requirements for the other components. (QC)

<Agreement>

- Proponents of option 1 to bring detailed analysis on the potential impact.

#### Issue 2-17: Impact on fast ASN.1 decoding

<Candidate solutions>

- Option 1: Event-triggered L1 report or reporting condition becoming fulfilled triggers fast ASN.1 decoding. (Nokia)

- Option 2: postpone. (QC)

<Recommended WF>

- Continue discussion.

#### Issue 2-18: Impact on early TCI state activation

<Candidate solutions>

- Option 1: Event-triggered L1 report or reporting condition becoming fulfilled triggers DL synchronization for the candidate cell’s active TCI states, if not valid at the time of report triggering. (Nokia)

- Option 2: postpone. (QC)

<Recommended WF>

- Continue discussion.

#### Issue 2-19: Impact on L1 measurement

<Candidate solutions>

- Option 1: RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cells fulfilling the reporting condition. (Nokia)

- Option 2: postpone. (QC)

<Recommended WF>

- Continue discussion.

#### Issue 2-20: others

<Candidate solutions>

- Proposal 1: Discuss the number of supported events at the end of the WI. (Nokia)

- Proposal 2: RAN4 to define the event triggered measurement report for known cell scenario. (E///)

<Recommended WF>

- Continue discussion.

# Topic #3: CSI-RS based L1 measurement

#### Issue 3-1: Known cell definition in CSI-RS based L1 measurement

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- Agreement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - Further check whether to consider SSB-based L3 measurement only

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet.

  - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

    - Further discuss whether to use detectable or measurable.

    - Further discuss the side condition

The following is concluded in RAN4#112bis meeting.

Issue 3-1-1: Whether to consider SSB-based L3 measurement only?

< Agreement >

- Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition.

Issue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet

<Candidate solutions>

- Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo)

- Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson)

  - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi)

- Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE)

<Recommended WF>

- Continue discussion.

Issue 3-1-3: Whether to use detectable or measurable?

<Candidate solutions>

- Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT)

- Option 2: Use detectable. (Apple, ZTE)

- Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia)

<Recommended WF>

- Continue discussion. RAN4 can first try to align understanding on what “detectable” and “measurable” mean.

Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell

<Candidate solutions>

- Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK)

- Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung)

<Recommended WF>

- Continue discussion.

#### Issue 3-2: Measurement procedure

<Agreement>

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether sStep 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

#### Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

<Agreement>

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and

    - It is applied for SCS = 60KHz

  - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

#### Issue 3-4: Categorization of CSI-RS based L1 RSRP measurement

<Candidate solutions>

- Option 1: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (Apple, CATT, CT)

- Option 2: Prefer to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. We can compromise to call “CSI-RS based L1-RSRP measurement within active BWP” intra-frequency if it is the majority view. (MTK)

- Option 3: How to category CSI-RS based L1-RSRP measurement is pending on whether/how to define CSI-RS based intra-frequency L1 measurement. (CMCC)

  - 3a: If the definition of CSI-RS based intra-frequency L1 measurement is same as that for CSI-RS based intra-frequency L3 measurement, CSI-RS based L1-RSRP measurement are categorized into intra-frequency and inter-frequency.

  - 3b: Otherwise, CSI-RS based L1-RSRP measurement are categorized into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP can be considered.

<Recommended WF>

- Covered by issue 3-3. This issue is closed.

#### Issue 3-5: Measurement on CSI-RS with repetition ON

<Candidate solutions>

- Option 1: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1. (MTK)

- Option 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. (Huawei, CATT)

  - Option 2a: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1. (vivo)

<Recommended WF>

- Continue discussion.

#### Issue 3-6: Measurement on CSI-RS with repetition OFF

<Candidate solutions>

- Option 1: From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. (vivo)

  - Option 1a: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1. (MTK)

  - Option 1b: In FR2, RAN4 will define the requirements for the following case, (CATT)

    - CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

- Option 2: Define first requirements for the case where the repetition is ‘OFF’ (Nokia)

<Recommended WF>

- Continue discussion.

#### Issue 3-7: CSI-RS resource type

<Candidate solutions>

- Option 1: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells. (CATT)

- Option 2: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS. (Apple, vivo)

- Option 3: Start from periodic CSI RS measurements, and wait for wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support. (Nokia)

- Option 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent. (Samsung)

- Option 5: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS (ZTE)

<Recommended WF>

- Continue discussion.

#### Issue 3-8: supported measurement type

<Agreement>

- CSI-RS based L1 measurement without gap is supported.

- FFS on CSI-RS based L1 measurement with gap

#### Issue 3-9: Measurement gap related

<Candidate solutions>

- Option 1: In Rel-19 LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement. (CATT)

- Option 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer. (CATT)

- Option 3: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements. (CATT)

- Option 4: RAN4 to further discuss type of MG for the CSI-RS based L1-RSRP measurements. (Ericsson)

<Recommended WF>

- Continue discussion.

#### Issue 3-10: CSI-RS based L1 measurement w.r.t active BWP

<Candidate solutions>

- Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT)

- Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO)

- Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE)

- Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo)

- Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

  - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

  - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

  - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

<Recommended WF>

- Continue discussion.

#### Issue 3-11: Restriction on CSI-RS resource configuration

<Candidate solutions>

- Option 1: It is proposed to introduce a CSI-RS based L1 measurement timing configuration. (Apple)

- Option 2: RAN4 requirements apply only when all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type. (Huawei, Qualcomm)

- Option 3: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion. (Huawei)

- Option 4: The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type. (vivo)

<Recommended WF>

- Continue discussion.

#### Issue 3-12: Scenarios to be considered w.r.t. RTD

RAN4 concluded the following in RAN4#112bis meeting.

- RTD is the received time difference among CSI-RS and SSB from different cells.

  - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

  - Alt 2: only define requirements for RTD<CP.

    - NW shall only enable this when RTD<CP is guaranteed.

<Candidate solutions>

- Option 1: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case. (MTK)

- Option 2: Either Alt 1 or Alt 2 is ok. RTD reporting might be needed if option 2 is adopted. (Apple)

- Option 3: Support RTD>CP case for CSI-RS based L1-RSRP measurement. (CT)

  - Option 3a: define the requirements for RTD>CP. Need further clear definition of RTD.

- Option 4: Define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. (vivo)

- Option 5: RTD < CP, and RTD > CP (Ericsson, ZTE)

  - Option 5a: RTD < CP, and RTD > CP UE capability for supporting RTD>CP can be introduced. (CMCC, vivo)

- Option 6: As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability or not can be FFS. (Nokia)

- Option 7: RAN4 to limit the applicable condition for CSI-RS-based L1-RSRP measurement to scenarios where RTD is not larger than a certain value, e.g., CP length. The detailed side condition can be discussed together with the solution for direct/indirect RTD value reporting to NW in RAN4#114. (Qualcomm)

- Option 8: RAN4 to consider introducing a solution to address the ambiguity issue regarding RRM requirement applicability due to RTD being larger than CP. The detailed solution can be discussed in RAN4#114. (Qualcomm)

<Recommended WF>

- Continue discussion.

#### Issue 3-13: Definition of frequency layer

<Candidate solutions>

- Option 1: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain. (Ericsson)

- Option 2: Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity) (Nokia)

- Option 3: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”(MTK)

- Option 4: From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement. (vivo)

  - The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type.

  - No RRM requirements are defined, if CSI-RSs on the same frequency layer are configured with the different SCS or CP type.

  - Note: No restriction on the center frequency and/or BW of the CSI-RS.

<Recommended WF>

- Continue discussion.

#### Issue 3-14: Requirement applicability

<Candidate solutions>

- Option 1: CSI-RS based measurement requirements are applicable if the UE measured SSB based L3 measurements less than X seconds before the CSI-RS measurement. (Ericsson)

- Option 2: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion. (Huawei)

- Option 3:  The requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type. (Huawei)

- Option 4:  There are some implicit restrictions on CSI-RS resources configuration, e.g., the CSI-RS resources on time domain are supposed to be aligned with measurement gap. However this is network configuration, and there is no specification impact. (Huawei)

<Recommended WF>

- Continue discussion.

#### Issue 3-15: Whether to consider CSI-RS based L1-SINR measurement

<Candidate solutions>

- Option 1: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. (CATT)

- Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1. (Apple, Qualcomm, ZTE)

- Option 3: RAN4 focus on CSI-RS based L1-RSRP measurement. (Samsung)

<Recommended WF>

- Continue discussion.

#### Issue 3-16: CSI acquisition on candidate cell(s) before or during LTM cell switch

<Candidate solutions>

- Option 1: RAN4 to wait for further progress on the details of early CSI acquisition from other working groups. (Qualcomm, ZTE)

<Recommended WF>

- Postpone and wait for further progress from other WGs.

#### Issue 3-17: CSI-RS accuracy requirements

<Candidate solutions>

- Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (ZTE, Apple)

<Recommended WF>

- Continue discussion.

#### Issue 3-17: other

<Candidate solutions>

- Option 1: Although point A of the CSI-RS is per-resource configured, RAN4 not to consider CSI-RS based L1 measurement on any SCell (s) in any candidate CG in Rel-19.

<Recommended WF>

- Continue discussion.