3GPP TSG-RAN WG4 Meeting #113	 														R4-2420102

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.24.3

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

< Agreement > ad hoc

  - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells.

  - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

Issue 1-1-2: Measurement requirements on conditional LTM

< Way forward > FFS the following proposal:

- Support L3 measurement based conditional LTM for FR1 from RAN4 perspective.

- Note: QC need further check. Continue to discuss in the next meeting.

Issue 1-1-3: Whether cover both RACH-based and RACH-less conditional LTM

< Agreement >

  - Cover both RACH-based and RACH-less conditional intra-CU LTM.

Issue 1-1-4: The specification structure of the requirements for conditional LTM

< Agreement >

  - The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Early synchronization and TCI state activation

Issue 1-2-1: Early UL Synchronization

RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.

For UE based TA measurement:

< Way forward > FFS the following proposals:

  - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT)

  - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

For PDCCH ordered RACH:

< Agreement >

- PDCCH ordered RACH is supported for conditional intra-CU LTM from RAN4 perspective.

  - Note: If no new PDCCH ordered RACH procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

Issue 1-2-2: Early TCI state activation

RAN2#127bis - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

< Agreement >

- Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM from RAN4 perspective.

  - Note: If no new procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

Conditional LTM delay

Issue 1-3-1: General aspects on Conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM.

- Proposal 2: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for each component of C-LTM delay.

- Proposal 3: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.

- Proposal 4: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

Issue 1-3-2: The starting point of conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

- Proposal 2: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration.

- Proposal 3: Similar as event-triggered L1 reporting. The CondEvent is met over the air.

- Proposal 4: Wait for more RAN2 agreements.

Issue 1-3-3: The ending point of conditional LTM delay

< Agreement > ad hoc

  - The ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell.

Issue 1-3-4: The components of conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

    - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

- Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution.

- Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution.

- Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

    - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused.

- Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups.

- Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements.

- Proposal 7: Wait for RAN2 progress.

Issue 1-3-5: TRRC

< Way forward > FFS the following proposal:

- Proposal 1: TRRC is the RRC procedure delay.

Issue 1-3-6: TEvent_DU

< Way forward > FFS the following proposals:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Proposal 2: Wait for more RAN2 progress.

    - Proposal 2a: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

- Proposal 3: Wait for more RAN2 agreements.

Issue 1-3-7: Tmeasure

< Way forward > FFS the following proposals:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

    - Proposal 1b: If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

    - Proposal 1c: FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM.

- Proposal 2: Wait for more RAN2 progress.

Issue 1-3-8: TCLTM_execution / TLTM_execution

< Way forward > FFS the following proposal:

- Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

Issue 1-3-9: Tinterrupt

< Way forward > FFS the following proposals:

- Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- Proposal 2: It highly depends on further discussion on procedure design in RAN2 group.

- Proposal 3: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure.

- Proposal 4: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Issue 1-3-10: The components of Tinterrupt

< Way forward > FFS the following proposal:

  - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Subsequent Conditional LTM

Issue 1-4-1: General aspects on Subsequent Conditional LTM delay

RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

< Way forward > FFS the following proposals:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay.

    - Proposal 1b: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

- Proposal 2: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for subsequent conditional LTM.

Issue 1-4-2: The starting point of Subsequent conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

- Proposal 2: Wait for RAN2 further progress.

Issue 1-4-3: The ending point of subsequent conditional LTM delay

< Agreement > ad hoc

  - The ending point of subsequent conditional LTM delay is the time the UE transmits the first UL transmission on the target cell.

Issue 1-4-4: The components of subsequent conditional LTM delay

< Way forward > FFS the following proposal:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.