3GPP TSG-RAN WG4 Meeting #113	R4-2420136

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_pos_enh2-Core) Draft CR on RRM core requirements for PRS BW aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In last meeting RAN4 endorsed a draft big CR R4-2416884 for RRM core part for Rel-18 positioning. The following issues are identified: - It was agreed to replace Teffect as Tmargin when defining PRS measureement period with BW aggregation. However when Tmagin=0, the total period for measuring aggregated PRS resource, e.g.Taggregated_RSTD, will be wrong. - For the interruption requirements at SRS BW aggregation, the victim cell should be indicated by posSRS-BWA-AffectedBandList-r18 regardless of per-FR MG capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To correct the above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements with Rel-18 PRS BW aggregation will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.10, 9.9.4.9, 8.2.2.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

#### 9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation

When UE physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform RSTD measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in Clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TRSTD_aggregate, Total defined as:

TRSTD_aggregate, Total = Tnon_aggregate_RSTD + Taggregate_RSTD + Tmargin,

where,

Tnon_aggregate_RSTD is the total measurement period for RSTD measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate_RSTD is based on clause 9.9.2.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replaced by L-1*T'marginT'marginTmargin.

Tnon_aggregate_RSTD is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

If UE is configured to perform positioning measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation, Tmargin = T’margin is delay margin to account for delay between RSTD measurement performed by UE on PFLs that contain PRS resources for aggregation and RSTD measurement performed by UE on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

T'margin= max(Teffect,i), when both aggregated PFLs and non-aggregated PFLs are configured for positioning measurement.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

Taggregate_RSTD, total measurement period for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RSTD=i=1GTRSTD_aggregate,i +(G-1)*T'marginTmarginmaxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for RSTD measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRSTD_aggregate,i is the measurement period for PRS RSTD measurement in the effective PFL i as specified below:

TRSTD_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective PFL i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective PFL i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG_agg,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

Otherwise,

kmultiTEG_agg,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEGagg,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective PFL i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective PFL, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective PFL i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective PFL i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of PRS RSTD measurement samples.

Tlast_agg,i is the measurement duration for the last PRS RSTD sample in the effective PFL i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the PRS RSTD measurement in the effective PFL i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PFL i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective PFL i. If more than one PRS periodicities are configured in effective PFL i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective PFL is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i}is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth  in TS 37.355 [34].

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2  specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the aggregated RSTD measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.2.10.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.2.10.

If during the measurement period of one or more effective PFLs, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD_aggregate,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for aggregated PRS resources, if the PRS resources to be aggregated are across two sampling duration of Nagg within duration Lavailable_PRS_agg,i.

The measurement requirements do not apply, if time span of the instance of the PRS resources to be aggregated (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability Nagg.

The requirements in clause 9.9.2.10 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while aggregated RSTD measurements are being performed, then the UE shall continue and complete the on-going aggregated RSTD measurements. The aggregated RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23A.2.

------------ END OF CHANGE 4--------------

------------ START OF CHANGE 5--------------

#### 9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation

When UE physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform UE Rx-Tx time difference measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in Clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TUE_RxTx_aggregated, Total defined as:

TUE_RxTx_aggregated, Total = Tnon-aggregate_RxTx + Taggregated_RxTx + Tmargin,

where,

Tnon_aggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate is based on clause 9.9.4.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replaced by L-1*TmarginT'marginT'margin.

Tnon_aggregate_RxTx is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

Taggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26].

If UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation, Tmargin = T'margin is delay margin to account for delay between Rx-Tx measurement performed by UE on PFLs that contain PRS resources for aggregation and Rx-Tx measurement performed by UE on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

T'margin= max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

Taggregate_RxTx, total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RxTx=i=1GTRxTx_aggregate,i +(G-1)*T'marginT'marginmaxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for UE Rx-Tx time difference measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRxTx_aggregate,i is the measurement period for UE Rx-Tx time difference measurement in the effective positioning frequency layer i as specified below:

TRxTx_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam_agg,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

Otherwise,

kmultiTEG,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective positioning frequency layer i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective positioning frequency layer i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective positioning frequency layer i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of UE Rx-Tx time difference measurement samples.

Tlast_agg,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the effective positioning frequency layer i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the UE Rx-Tx time difference measurement in the effective positioning frequency layer i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PRS frequency layer i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective positioning frequency layer i. If more than one PRS periodicities are configured in effective positioning frequency layer i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective positioning frequency layer is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i}is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN  in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth in TS 37.355 [34].

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequested.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the aggregated UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with UE RX-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.4.9.

When PRS-RSRPP is also reported by UE together with UE Rx-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.4.9.

If during the measurement period of one or more effective positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TUE_RxTx_aggregated,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

The requirements in clause 9.9.4.9 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete. If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25A.2.

------------ END OF CHANGE 5--------------

------------ START OF CHANGE 6--------------

##### 8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning

The requirements in this clause are applicable to SRS transmission for positioning with bandwidth aggregation, when SRS resource configured in a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured in an UL active BWP of a CC with PUSCH/PUCCH.

SRS resource sets across two or three CCs are aggregated for positioning when UE is configured with linkage information SRS-PosResourceSetLinkedForAggBWList [34], provided that:

-	the aggregated SRS transmission satisfy all the conditions defined in Clause 6.2.1.4.2 in TS 38.214 [26];

-	the aggregated SRS transmission and the guard period before and after the SRS transmission are not colliding with any other UL transmission or/and DL reception with higher priority in victim cells defined in TS 38.214 [26];

-	the aggregated SRS transmission plus guard period before and after does not collide with any NR L3 or L1 measurements on victim cells.

For interruption caused by SRS transmission for positioning with bandwidth aggregation, the victim cell is identified based on the entry number of the band indicated by UE in posSRS-BWA-AffectedBandList-r18 regardless of per-FR MG capability.

Note 1: UE may indicate no other bands in the band combination are affected by the SRS carrier switching, in which case, only the band with the aggregated SRS transmissions is affected.

Note 2: If the capability is not indicated, interruption is allowed on all serving cells.

For each SRS transmission, interruption is allowed on any active serving cell on the band as indicated in posSRS-BWA-AffectedBandList-r18, during the guard periods before and after aggregated SRS transmission and the aggregated SRS transmission duration with up to X OFDM symbols, where X equals to (Y+Z+1). Interruption length of one extra OFDM symbol is added to cover the impact of RTD.

The supported guard period is reported by UE in guardPeriod in NR-UL-SRS-Capability [34] and the total duration Y of guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols in Table 8.2.2.2.21-1.

Table 8.2.2.2.21-1: Total duration Y of guard periods before and after aggregated SRS transmission (OFDM symbols)

|  | Guard period (s) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 0 | 30 | 100 | 140 | 200 |
| 15 | 0 | 1 | 3 | 4 | 6 |
| 30 |  | 2 | 6 | 8 | 12 |
| 60 |  | 4 | 12 | 16 | 23 |
| 120 |  | 7 | 23 | 32 | 45 |

The duration of aggregated SRS transmission is given by nrofSymbols in corresponding SRS-Resource configurations [34]. This duration in number of nrofSymbols OFDM symbols is converted to Z OFDM symbols in victim cells according to the SCS configurations among the aggressor cell and victim cell(s). Z is specified in Table 8.2.2.2.21-2, Table 8.2.2.2.21-3 and Table 8.2.2.2.21-4 for nrofSymbols equal to {1, 2, 4} respectively with different SCS configurations.

Table 8.2.2.2.21-2: Duration Z of aggregated SRS transmission with nrofSymbols = 1 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 |
| 30 | 2 | 1 | 1 | 1 | 1 | 1 |
| 60 | 4 | 2 | 1 | 1 | 1 | 1 |
| 120 | 8 | 4 | 2 | 1 | 1 | 1 |

Table 8.2.2.2.21-3: Duration Z of aggregated SRS transmission with nrofSymbols = 2 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 2 | 1 | 1 | 1 | 1 | 1 |
| 30 | 4 | 2 | 1 | 1 | 1 | 1 |
| 60 | 8 | 4 | 2 | 1 | 1 | 1 |
| 120 | 16 | 8 | 4 | 2 | 1 | 1 |

Table 8.2.2.2.21-4: Duration Z of aggregated SRS transmission with nrofSymbols = 4 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 4 | 2 | 1 | 1 | 1 | 1 |
| 30 | 8 | 4 | 2 | 1 | 1 | 1 |
| 60 | 16 | 8 | 4 | 2 | 1 | 1 |
| 120 | 32 | 16 | 8 | 4 | 1 | 1 |

------------ END OF CHANGE 6--------------