3GPP TSG-RAN4 Meeting #113	R4-2420187

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5127 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IE for UE capability for PC6 UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SCell activation delay, correct the UE capability measEnhCAInterFreqFR2-r18 for PC6 UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In SCell activation delay, correct the UE capability measEnhCAInterFreqFR2-r18 for PC6 UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	If UE supports shortMeasInterval-r18, then

-	TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise.

-	Otherwise

-	TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise.

otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	If UE supports shortMeasInterval-r18, then

-	6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	Otherwise

-	6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled,

-	The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

-	The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell., and

-	The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Tactivation_time is

-	Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB

-	Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for SCell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB

-	Tfirst_TRS + 2*TTRS +5 ms, when the EPRE difference is larger than 12 dB but smaller than or equal to [30] dB

For a UE supporting scellWithoutSSB-InterBandCA-r18, when UE receive SCell activation command for more than one SSB-less SCell, the SCell activation delay for each of the to-be-activated SCell is the same as the single SCell activation delay, when

-	SSB-less SCells being activated are on different bands, or

-	all SCells being activated are SSB-less SCells, the SCells are contiguous on the same band, and all to-be-activated SCells have the same QCL-typeC QCL source cell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if highSpeedMeasFlagFR2-r17 is configured for the FR2 power class 6 UE supporting scellWithoutSSB and measEnhCAInterFreqFR2-r18, Tactivation_time is 3 ms , provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), or

-	6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, or

-	3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

where,

TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

Trs, enhanced is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFirstSSB_MAX, enhanced: For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

X1 is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

TL1-RSRP, enhanced_measure is

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

TFirstTRS: is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

TTRS is the periodicity of periodic CSI-RS burst for SCell activation.

TFirstATRS: is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

TATRS is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

Tgap is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Change 1>