3GPP TSG-RAN WG4 Meeting # 113	R4-2420240

# Orlando, USA, 18th November - 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to performance parts for CPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some values in DL RSCPD/DL RSCP accuracy requirements are TBD and they should be defined. - Margin values (Y) for CPP accuracy requirements need to be defined. - Clarify the applicable conditions for CPP accuracy requirements, e.g., comb size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define values for the TBDs in DL RSCPD/DL RSCP accuracy requirements. - Define margin values (Y) for CPP accuracy requirements. - Clarify that the CPP accuracy requirements are not applicable for PRS resources with comb size 12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for DL RSCPD/DL RSCP are not fully completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.43.2, 10.1.44.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2417693. R4-2420147. |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.43.2 		Measurement Accuracy Requirements

The accuracy requirements for DL RSCPD measurement are based on single measurement sample in single PFL and shall be within ±(X+Y) degree, provided that the following conditions are met:

- Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled for FR1.

- Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled for FR2.

- Conditions for DL RSCPD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- The measurements to derive DL RSCPD are performed on PRS resources  within same set of symbols.

- DL RSCPD measurements are based on PRS resources with comb size other than 12.

The requirements in this clause are derived based on AWGN channel and on two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz).

X is defined in Table 10.1.43.2-1 for AWGN channel and Table 10.1.43.2-2 for two-tap channel for FR1, and is derived assuming no carrier frequency offset at TRP.

X is defined in Table 10.1.43.2-3 for AWGN channel and Table 10.1.43.2-4 for two-tap channel for FR2, and is derived assuming no carrier frequency offset at TRP.

Y is 14 degrees for FR1 and 28 degrees for FR2.

Table 10.1.43.2-1: DL RSCPD absolute accuracy in FR1 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -6dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 16 | 3 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 11 | 5 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 8 | 4 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 16 | 3 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 11 | 4 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 17 | 3 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 8 | 4 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |  |

Table 10.1.43.2-2: DL RSCPD absolute accuracy in FR1 for two-tap channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 7 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 7 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 6 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 7 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 4 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.43.2-3: DL RSCPD absolute accuracy in FR2 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -6dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 17 | 3 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 9 | 5 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 7 | 3 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 17 | 3 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 10 | 6 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 6 | 3 |  | ≥ 128 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.43.2-4: DL RSCPD absolute accuracy in FR2 for two-tap channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  | Minimum Io | Maximum Io |
| degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 5 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 5 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 4 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 5 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 7 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 5 |  | ≥ 128 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

10.1.44.2 		Measurement Accuracy Requirements

The relative accuracy of DL RSCP measurement in this clause is defined as accuracy of the difference between two DL RSCP measurements, each based on single measurement sample in single PFL.

The requirements in this clause are derived based on AWGN channel and based on two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz).

The DL RSCP relative measurement accuracy requirements in this clause shall not apply, if:

- NTA_offset defined in Table 7.1.2-2 changes during the DL RSCP with UE Rx-Tx measurement period, or

- if the uplink transmission timing changes during the DL RSCP with UE Rx-Tx measurement period due to the network-configured Timing Advance.

The DL RSCP relative measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the DL RSCP with UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2 then:

-	DL RSCP and UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission.

When a serving cell change occurs during the DL RSCP with UE Rx-Tx measurement period, UE Rx-Tx measurement accuracy requirements and DL RSCP measurement requirements do not apply.

The relative DL RSCP accuracy requirements defined in clause 10.1.44 are valid under the following conditions:

- Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled for FR1.

- Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled for FR2.

- Conditions for DL RSCP measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- DL RSCP measurements to derive the relative accuracy are performed on PRS resources within the same set of symbols.

- DL RSCP measurements are based on PRS resources with comb size other than 12.

The accuracy requirement for relative DL RSCP shall be within ±(X+Y) degree.

The values of X for accuracy requirements for relative DL RSCP measurement in FR1 are defined in Table 10.1.44.2-1 for AWGN channel and in Table 10.1.44.2-2 for two-tap channel, and is derived assuming no carrier frequency offset at TRP.

The values of X for accuracy requirements for relative DL RSCP measurement in FR2 are defined in Table 10.1.44.2-3 for AWGN channel and in Table 10.1.44.2-4 for two-tap channel, and is derived assuming no carrier frequency offset at TRP.

Y is 14 degrees for FR1 and 28 degrees for FR2.

Table 10.1.44.2-1: DL RSCP relative accuracy in FR1 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 14 | 4 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 9 | 4 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 | 3 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 14 | 5 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 9 | 3 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 14 | 4 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 10 | 4 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |  |

Table 10.1.44.2-2: DL RSCP relative accuracy in FR1 for two-tap channel

| Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 9 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 7 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 5 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 9 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.44.2-3: DL RSCP relative accuracy in FR2 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 14 | 4 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 8 | 4 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 6 | 3 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 16 | 5 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 9 | 4 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 7 | 3 |  | ≥ 128 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.44.2-4: DL RSCP relative accuracy in FR2 for two-tap channel.

| Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS∗LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  | Minimum Io | Maximum Io |
| degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 8 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 5 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 3 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 9 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 6 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 5 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |

<End of Change 2>