3GPP TSG-RAN WG4 Meeting#113                              R4-2420330

Orlando, USA, 18th ‒ 22nd November, 2024

Agenda item:			7.2.6

Source:	ZTE Corporation

Title:	 WF on 15GHz BS parameters

Document for:	Approval

- Agreement for RAN4#113

Issue 4-1-2: Typical channel bandwidth

Agreement

- Indicate 200MHz as typical bandwidth to ITU-R for their evaluation in the LS

  - Assume the SEM of FR1 intra-band CA of 200MHz aggregated bandwidth applicable for 200MHz bandwidth for 15GHz range.

    - 3GPP RAN4 will further discuss and specify the SEM requirements in this frequency range with understanding that the requirement may be relaxed than FR1 intra-band CA with 200MHz aggregated bandwidth.

    - Note: capture this bullet for assumption in the TR

  - 3GPP may specify the 100MHz, 200MHz as well as other channel bandwidths for the requirements in this frequency range.

    - Note: take 200Mhz as typical value and refer to TR for additional values of bandwidths in LS.

Issue 4-2-6: BS ACLR

Agreement:

- BS ACLR 30dB

Issue 4-2-3: BS Emissions mask

Agreement

- Do not include category B OBUE in reply

  - Mention in TR that regional regulation corresponding to category B is not yet decided

Issue 4-2-7: Spurious emission

Agreement

Reply that SM.329 applies

Issue 4-2-5: Sensivitity

Previous agreement:

For LS:  “To be specified”

For TR: Define based on NF and antenna gain

Agreement : define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz;

EISREFSENS=PkT+10∗log10BW+NF+IM+SNR−G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

Issue 4-2-8: Blocking response

Agreement

- ΔfOBUE and ΔfOOB 100MHz

- Define requirement as follows, where interferer is defined in radiated way

  - FFS how to define the requirements for 1-H

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength for OTA requirements (voltage/meter) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW |
| 12750 to FUL,low – 100 | EISREFSENS + 6 dB | 0.1 | CW |
| FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 | CW |

Issue 4-2-9: BS ACS

BS ACS 30dB

Issue 4-2-X: In-band blocking

Agreement

For LS: Mention “To be specified"
For TR: FFS for next meeting

Agreement on BS antenna parameters:

Table 4: Beamforming antenna characteristics for IMT in 14800 to 15350 MHz

|  |  | Macro suburban | Macro urban | Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  |  | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 5 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 90º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 16x24 | 16x24 | 16x24 | 4x4 |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.7a | Number of element rows in sub-array | 4 | 4 | 4 | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | 0.7 of wavelength for V | 0.7 of wavelength for V | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | 3 | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 17.2 | 17.2 | 7.2 | 8.0 |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | ±90 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 0-180 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | 6 | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 84.3 | 84.3 | 74.3 | 40.0 |

- Agreement for RAN4#112bis

Issue 3-2: Scope of scenarios

Agreement:

- Limit simulations to urban macro

Issue 3-9: ISD for simulation

Agreement:

- 450m

Agreement:

Issue 4-17: BS antenna parameters

For Urban Macro:

Agreement:

- Make assumption that reply can say that array size is  [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible.

- Double check the coverage and [3072] before making final decision on reply

  - If coverage issues is found for [3072], minimum array size [4096] to achieve coverage could be discussed

For Dense Urban:

Agreement:

- Dense urban antenna parameters Same as urban macro, mechanical tilt can differ in reply

For Indoor scenario:

Agreement:

- For reply LS : Antenna configuration without sub-array assumption: 4x4

- Simulation: Not prioritized

For output power for different BS class:

Agreement:

- For wide area BS, 46dBm for TRP for dual polarization;

- For dense urban BS, [36dBm]for TRP for dual polarization;

- For Indoor:  23dBm per polarization

Issue 4-10: Emissions mask

Agreement:

For the reply,  refer to the following table as basis

- [category B] and category A

- Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -13 dBm | 1MHz |

- [Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -15 dBm | 1MHz |

]

Issue 4-14: Spurious emission

Agreement:

For the reply,  refer to the following table as basis

- [category B] and category A

- Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

- Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

Issue 4-12: Sensivitity

Agreement:

For LS:  “To be specified”

For TR: Define based on NF and antenna gain

Issue 4-8: BS output power

Agreement:

- Do not mention average output power

- Open issues

Issu 4-13: ACLR

Potential agreement:

Postpone to next meeting according to the simulation results.

Issue 4-16: ACS

agreement:

Postpone to next meeting according to the simulation results.

Issue 4-15: Blocking response

Potential Agreement:

  - Further evaluate the power level for in-band requirement based on the simulation

  - Further evaluate the f_OOBB requirement based on the agreed IBB and OOBB power level;

- Agreement for previous RAN4 meeting

This sub-topic considers BS parameters

Issue 4-9: Power dynamic range

Previous agreement: 0dB

Issue 4-11: Noise figure

Previous agreement:

- 8 dB for Wide-Area BS

- 13 dB for Medium Range BS

- 16 dB for Local Area BS