3GPP TSG-RAN5 Meeting #105	R5-246205

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0490 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to applicability for some LTM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updated the applicabilities for LTM test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table 4.0-1 ,Table 4.0-4 and Table 4.2-3 for LTM test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.508-2 CR 0712 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4.0-1: Applicability of conformance test cases conditions

| C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A |  |
| --- | --- |
| <Unchanged contents are omitted> |  |
| C351 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 AND A.4.3.6-1/4 THEN R ELSE N/A |
| Cxx1 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/178 AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxx2 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxx3 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/175 AND NOT (A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxx4 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxx5 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/178 AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxx6 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxx7 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/175 AND NOT (A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxx8 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/181 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxx9 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxa IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6/97 THEN R ELSE N/A |
| Cxxb IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx5 THEN R ELSE N/A |
| Cxxc IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/178 AND A.4.3.2-1/xx1 AND A.4.3.1-1/xx6 THEN R ELSE N/A |
| Cxxd IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.2-1/xx1 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxe IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/175 AND NOT (A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/xx1 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxf IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.2-1/xx1 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxg IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/178 AND A.4.3.6-1/yy3 AND A.4.3.6-1/97 THEN R ELSE N/A |
| Cxxh IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/yy3 AND A.4.3.6-1/97 THEN R ELSE N/A |
| Cxxi IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/175 AND NOT (A.4.3.2-1/178 OR A.4.3.2-1/179) A.4.3.6-1/yy3 AND A.4.3.6-1/97 THEN R ELSE N/A |
| Cxxj IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) AND A.4.3.6-1/yy3 AND A.4.3.6-1/97 THEN R ELSE N/A |
| Cxxk IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/178 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxl IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx1 AND A.4.3.2-1/179 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxm IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/xx1 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx6 THEN R ELSE N/A |
| Cxxk1 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/xx1 AND A.4.3.2-1/178 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxxl1 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/xx1 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxxm1 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/xx1 AND A.4.3.6-1/yy5 THEN R ELSE N/A |
| Cxxn IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/178 AND A.4.3.2-1/183 AND A.4.3.2-1/184 AND A.4.3.2-1/185 AND A.4.3.2-1/186 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx4 THEN R ELSE N/A |
| Cxxo IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/184 AND A.4.3.2-1/185 AND A.4.3.2-1/186 AND A.4.3.6-1/yy5 AND A.4.3.2-1/xx4 THEN R ELSE N/A |
| NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25). NOTE 4: Cxxxw applicability is defined for mpr Power Boost related test (A.4.3.2-1/38). |  |

<End of modified section 1>

<Start of modified section 2>

Table 4.0-4: Subtest Selection Criteria for performance and RRM test cases

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| F001 | IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A | UEs supporting EN-DC FR1 AND CSI-RS based PRACH |
| F002 | IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting EN-DC FR1 and long DRX cycle |
| F003 | IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A | UEs supporting EN-DC FR2 and CSI-RS based PRACH |
| F004 | IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting EN-DC FR2 and long DRX cycle |
| F005 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |
| F006 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1 and long DRX cycle |
| F007 | IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A | UEs supporting 5GS NR SA FR2 and CSI-RS based PRACH |
| F008 | IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting 5GS NR SA FR2 and long DRX cycle |
| F009 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 AND A.4.3.12-1/2 AND A.4.3.1-7a/4 THEN R ELSE N/A | 1Rx RedCap UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |
| F010 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 AND A.4.3.12-1/2 AND A.4.3.1-7a/1 THEN R ELSE N/A | 2Rx RedCap UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |
| F011 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 AND A.4.3.12-1/2 AND A.4.3.1-7a/4 THEN R ELSE N/A | 1Rx RedCap UEs supporting 5GS NR SA FR1 and long DRX cycle |
| F012 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 AND A.4.3.12-1/2 AND A.4.3.1-7a/1 THEN R ELSE N/A | 2Rx RedCap UEs supporting 5GS NR SA FR1 and long DRX cycle |
| F013 | IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.12-1/2 AND 4.3.2-1/9 THEN R ELSE N/A | RedCap UEs supporting 5GS NR SA FR2 and CSI-RS based PRACH |
| F014 | IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/2 AND A.4.3.6-1/92 THEN R ELSE N/A | UEs supporting EN-DC FR2 and per-FR gap and Gap Pattern 13 |
| F015 | IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 AND A.4.3.6-1/2 AND A.4.3.6-1/92 THEN R ELSE N/A | UEs supporting EN-DC FR2, per-FR gap and long DRX cycle and Gap Pattern 13 |
| F016 | IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.11-1/7 AND A.4.3.6-1/2 AND A.4.3.6-1/21 THEN R ELSE N/A | UEs supporting EN-DC FR1, inter-frequency measurement enhancements in HST, per-FR gap and Gap Pattern 4 |
| F017 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.11-1/7 AND A.4.3.6-1/2 AND A.4.3.6-1/21 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, inter-freq measurement enhancements in HST, per-FR gap and Gap Pattern 4 |
| F018 | IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.3.6-1/2 AND A.4.3.6-1/21 THEN R ELSE N/A | UEs supporting E-UTRA, NR FR1 measurement, per-FR gap and Gap Pattern 4 |
| F019 | IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.3.5-1/1 AND A.4.3.6-1/2 AND A.4.3.6-1/21 THEN R ELSE N/A | UEs supporting E-UTRA, NR FR1 measurement, long DRX cycle, per-FR gap and Gap Pattern 4 |
| F020 | IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-1/1 OR A.4.1-1/2 OR A.4.3.12-1/5) AND A.4.1-3/1 AND A.4.3.12-1/2 AND Table A.4.3.1-7a/1 AND A.4.3.6-1/2 AND A.4.3.6-1/11 THEN R ELSE N/A | 2Rx RedCap UEs supporting 5GS NR SA FR1, E-UTRA, per-FR gap and Gap Pattern 4 |
| F021 | IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-1/1 OR A.4.1-1/2 OR A.4.3.12-1/5) AND A.4.1-3/1 AND A.4.3.12-1/2 AND Table A.4.3.1-7a/1 AND A.4.3.6-1/2 AND A.4.3.6-1/11 AND A.4.3.5-1/1 THEN R ELSE N/A | 2Rx RedCap UEs supporting 5GS NR SA FR1, E-UTRA, long DRX cycle, per-FR gap and Gap Pattern 4 |
| F022 | IF A.4.3.2-1/20 THEN R ELSE N/A | UE supporting alternative additional DMRS position for co-existence with LTE CRS |
| F023 | IF A.4.3.2-1/2 THEN R ELSE N/A | UE supporting 256QAM for PDSCH for FR1 |
| Fxx1 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx6 AND A.4.3.2-1/xxd THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, simultaneous intra-frequency L1-RSRP measurements for more than one cell when the max RTD among the cells is larger than CP length |
| Fxx2 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/xx5 AND A.4.3.2-1/xxd THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, simultaneous inter-frequency L1-RSRP measurements without measurement gaps for more than one cell when the max RTD among the cells is larger than CP length |

<End of modified section 2>

<Start of modified section 3>

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release |  | Applicability |  |  |  | Additional Information |  | Branch |  | Subtest Selection Criteria |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |  |
|  | << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.4 | LTM PDCCH-order Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.4.1 | PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP |  | Rel-18 |  | Cxxk |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), interruptions caused by PDCCH order |  | For Test 1 |  | 2Rx 4Rx |  |  |  |
|  |  |  |  |  | Cxxl |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement, reporting based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |  | For Test 2 |  | 2Rx 4Rx |  |  |  |
|  |  |  |  |  | Cxxm |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order |  | For Test 3 |  | 2Rx 4Rx |  |  |  |
| 6.3.2.4.2 | PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM |  | Rel-18 |  | Cxxn |  | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |  | For Test 1 |  | 2Rx 4Rx |  |  |  |
|  |  |  |  |  | Cxxo |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |  | For Test 2 |  | 2Rx 4Rx |  |  |  |
| 6.3.2.4.3 | PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP |  | Rel-18 |  | Cxxk1 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, interruptions caused by PDCCH order |  | For Test 1 |  | 2Rx 4Rx |  |  |  |
|  |  |  |  |  | Cxxl1 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, RACH-less LTM with dynamic grant and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |  | For Test 2 |  | 2Rx 4Rx |  |  |  |
|  |  |  |  |  | Cxxm1 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, interruptions caused by PDCCH order |  | For Test 3 |  | 2Rx 4Rx |  |  |  |
|  | << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4 | LTM PCell Switch |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 | Rel-18 |  | Cxxc |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | For Test 1A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxd |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), and do not support MAC-CE activated joint LTM TCI states |  | For Test 1B |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxe |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), and do not support MAC-CE activated joint or seperate LTM TCI states. |  | For Test 2A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxf |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with seperate DL/UL LTM TCI-state indication for LTM procedure, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) ,do not support MAC-CE activated joint or seperate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |  | For Test 2B |  | 2Rx 4Rx |  |  |  |
| 6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 | Rel-18 |  | Cxxg |  | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH without NR-DC configuration, MAC-CE activated joint LTM TCI states and inter-frequency L1-RSRP measurement and reporting with measurement gaps based on SSB(s) of candidate cell(s) |  | For Test 1A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxh |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, inter-frequency L1-RSRP measurement and reporting based with measurement gap on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint LTM TCI states |  | For Test 1B |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxi |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, inter-frequency L1-RSRP measurement and reporting with measurement gap based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint or seperate LTM TCI states. |  | For Test 2A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxxj |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with seperate DL/UL LTM TCI-state indication for LTM procedure, inter-frequency L1-RSRP measurement and reporting with measurement gap based on SSB(s) of candidate cell(s) ,do not support MAC-CE activated joint or seperate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |  | For Test 2B |  | 2Rx 4Rx |  |  |  |
| 6.3.4.3 |  | RACH-less Intra-frequency PCell switch from FR1 to FR1 |  |  | Rel-18 |  |  | Cxx1 |  |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, TA indication in cell switch command and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  | For Test 1A |  |  | 2Rx 4Rx |  |  |  |  |
|  |  |  |  | Cxx2 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, RACH-less LTM with dynamic grant, TA indication in cell switch command, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint LTM TCI states |  | For Test 1B |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxx3 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, RACH-less LTM with dynamic grant, TA indication in cell switch command, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint or seperate LTM TCI states. |  | For Test 2A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxx4 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with seperate DL/UL LTM TCI-state indication for LTM procedure, RACH-less LTM with dynamic grant, TA indication in cell switch command, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) ,do not support MAC-CE activated joint or seperate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |  | For Test 2B |  | 2Rx 4Rx |  |  |  |
| 6.3.4.4 | RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement | Rel-18 |  | Cxx5 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant and TA indication in cell switch command |  | For Test 1A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxx6 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated seperate LTM TCI states, RACH-less LTM with dynamic grant, TA indication in cell switch command and do not support MAC-CE activated joint LTM TCI states |  | For Test 1B |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxx7 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, RACH-less LTM with dynamic grant, TA indication in cell switch command and do not support MAC-CE activated joint or seperate LTM TCI states. |  | For Test 2A |  | 2Rx 4Rx |  |  |  |
|  |  |  |  | Cxx8 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, unified TCI with seperate DL/UL LTM TCI-state indication for LTM procedure, RACH-less LTM with dynamic grant, TA indication in cell switch command, do not support MAC-CE activated joint or seperate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |  | For Test 2B |  | 2Rx 4Rx |  |  |  |
| 6.4 | Timing |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |  |  |  |  |  |  |  |
|  | << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.20.1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Rel-17 |  | C212 |  | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with PRS |  |  |  | 2Rx 4Rx 8Rx |  |  |  |
| 6.6.21 | UE Rx-Tx time difference measurement for propagation delay compensation with TRS |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.21.1 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Rel-17 |  | C213 |  | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with TRS |  |  |  | 2Rx 4Rx 8Rx |  |  |  |
| 6.6.26 | LTM Intra-frequency L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.26.1 | Intra-frequency SSB based L1-RSRP measurement | Rel-18 |  | Cxx9 |  | UEs supporting 5GS NR SA FR1, intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx |  | Subtest 2: Fxx1 |  |
| 6.6.27 | LTM Inter-frequency L1-RSRP measurement with measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.27.1 | Inter-frequency SSB based L1-RSRP measurement with measurement gap | Rel-18 |  | Cxxa |  | UEs supporting 5GS NR SA FR1, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx |  |  |  |
| 6.6.28 | LTM Inter-frequency L1-RSRP measurement without measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.28.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap | Rel-18 |  | Cxxb |  | UEs supporting 5GS NR SA FR1, SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps for LTM |  |  |  | 2Rx 4Rx |  | Subtest 2: Fxx2 |  |
| 6.7 | Measurement performance requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |
|  | << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.17 | LTM L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.17.1.1 |  | Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 |  |  | Rel-18 |  |  | Cxxa |  |  | UEs supporting 5GS NR SA FR1, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  |  |  | 2Rx 4Rx |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. NOTE 3: Test cases in TS 38.533 [5] clause 6 only apply to FR1 non-RedCap UEs. For FR1 RedCap UEs, Test cases in TS 38.533 [5] clause 16 apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 3>