3GPP TSG-RAN5 Meeting #105	R5-246366

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0714 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As there are some new UE capability parameters (e.g., SON parameters) defined for RedCap in TS 38.306 and TS 38.331, these parameters needs to be added into the PICS table for test case applicablity update and UE capability check TC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Four new PICS were added for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of relevant PICS for test case applicablity update and UE capability check TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.12	RedCap Capabilities

According to TS 38.306 [17] clause 4.2.21:

CA, MR-DC, DAPS, CPAC and IAB (i.e the RedCap IE is not expected to act as IAB mode) related UE features and corresponding capabilities are not supported by the RedCap UEs.

-	PICS associated to the following features are as below:

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

-	UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branches or more than 1 UL MIMO layers are not supported by RedCap UE.

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported.

-	For FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported.

Table A.4.3.12-1: RedCap UE Capabilities

| Item | UE Capabilities | Ref. | Release | Mnemonic |  |  | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOf16DRB_RedCap_r17 |  |  | No |  |  |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOfRedCap_r17 |  |  | No |  |  | This PICS shall always be true for RedCap UE. |  |
| 3 | Void |  |  |  |  |  |  |  |  |  |  |
| 4 | Void |  |  |  |  |  |  |  |  |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_r17 |  |  | No |  |  | FDD FR1 only |  |
| 6 | Support of relaxed RRM measurements in RRC_CONNECTED for RedCap UE. | 38.306 4.2.21.5 | Rel-17 | pc_rrm_RelaxationRRC_ConnectedRedCap_r17 |  |  | No |  |  |  |  |
| 7 | Support of initiating UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. | TS 38.331 5.7.4.2 | Rel-17 | pc_UAI_rrm_RelaxationRRC_ConnectedRedCap |  |  | No |  |  | The UE will initiate UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. It is only applicable for RedCap UE. |  |
| 8 | Support of Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_Relaxed_Measurement_RedCap_r17 |  |  | No |  |  | It is optional for RedCap UE to support Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE |  |
| 9 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for overlap MFBI band undertest for RedCap UE | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_MFBI |  |  | No |  |  | FDD FR1 only It is only applicable when pc_halfDuplexFDD_TypeA_RedCap_r17=True |  |
| xx1 | Support of using (e)RedCap-specific initial DL BWP associated with NCD-SSB for SDT | 38.306 4.2.21.2 | Rel-17 | pc_ncd_SSB_ForRedCapInitialBWP_SDT_r17 | No |  |  |  |  |  |
| xx2 | Support of RRC-configured DL BWP without CD-SSB or NCD-SSB | 38.306 4.2.21.6.1 | Rel-17 | pc_bwp_WithoutCD_SSB_OrNCD_SSB_RedCap_r17 | No |  |  |  |  |  |
| xx3 | Support of storage of connection establishment failure or connection resume failure information and the reporting in UEInformationResponse message | 38.306 4.2.21.7 | Rel-17 | pc_cef_ReportRedCap_r17 | No |  |  |  |  |  |
| xx4 | Support of the storage of radio link failure information or handover failure information and the reporting in UEInformationResponse message | 38.306 4.2.21.7 | Rel-17 | pc_rlf_ReportRedCap_r17 | No |  |  |  |  |  |

[End of change]