3GPP TSG-RAN5 Meeting #105	R5-246420

# Orlando, United States, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0123 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.579-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At implementation of R5-220488 by mistake Figure 5.1.1-1 has been replaced by Figure 5.2.1-1. - Group "JSON_AliasDefinitions" has been removed from TTCN. - For MCData IP connectivity an IP address is needed for the application hosted at the emulated client B. - In Foreword clause the present specification is wrongly referred to as “part 1”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Figure 5.1.1-1 corrected. - "JSON_AliasDefinitions" removed from Table 8.1-1. - PIXIT px_MCData_IPConnectivityEndpointAddress_B added to Table 9.2.1-1. - “part 1” has been corrected into “part 5”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent or confusing test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 8.1, 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR .... |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR .... |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR .... |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

The present document is part 5 of a multi-part conformance test specification for Mission Critical services over LTE consisting of:

3GPP TS 36.579-1 [2]: "Mission Critical (MC) services over LTE; Part 1: Common test environment"

3GPP TS 36.579-2 [3]: "Mission Critical (MC) services over LTE; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification"

3GPP TS 36.579-3 [4]: "Mission Critical (MC) services over LTE; Part 3: Mission Critical Push To Talk (MCPTT) Server Application conformance specification"

3GPP TS 36.579-4 [5]: "Mission Critical (MC) services over LTE; Part 4: Test Applicability and Implementation Conformance Statement (ICS) proforma specification"

3GPP TS 36.579-5: "Mission Critical (MC) services over LTE; Part 5: Abstract test suite (ATS)" (the present specification)

3GPP TS 36.579-6 [59]: "Mission Critical (MC) services over LTE; Part 6: Mission Critical Video (MCVideo) User Equipment (UE) Protocol conformance specification"

3GPP TS 36.579-7 [60]: "Mission Critical (MC) services over LTE; Part 7: Mission Critical Data (MCData) User Equipment (UE) Protocol conformance specification"

<End of modification>

<Start of modification>

### 5.1.1	MCX Client on-network test model

The MCX Client on-network test model is depicted in figure 5.1.1-1. The test model consists of an IMS component and an HTTP component, on top of the multi-testers test model (E-UTRA) specified in TS 34.229-3 [28]. These parallel test components (PTCs) handle the IMS and HTTP signalling asynchronously.

The IMS PTC controls the IPCanEmu and the IP PTC. IPCanEmu is responsible for handling the E-UTRA cell(s) configuration in the SS as well as the E-UTRA/EPC level signalling and related procedures. The IPCanEmu is based on the TTCN implementation used for E-UTRA/EPC conformance testing according to TS 36.523-3 [27]. The IP PTC controls the IP related configurations as described in TS 36.523-3[27]. In addition there is an SRTP port at the MCX IMS PTC mapped to the system interface to configure SRTP/SRTCP for media streams and media control messages (see clause 7.1.1.3). The media control messages are exchanged between TTCN and the system simulator via the IP PTC's IP_SOCK port (see clause 7.1.1.4 and clause 7.1.1.5) whereas the media stream messages are handled by the SS (see clause 7.1.1.5).

Figure 5.1.1-1: MCX Client on-network test model with TTCN based E-UTRA/EPC implementation

<End of modification>

<Start of modification>

## 8.1	Codec requirements

In addition to the requirements specified in TS 36.523-3 [27] clause 8.1 and TS 34.229-3 [28] clause 7 the codec requirements of table 8.1-1 shall be applied.

Table 8.1-1: Codec requirements

| Type definitions | Codec requirements | Encoding rule in TTCN-3 |
| --- | --- | --- |
| MIKEY_TypeDefs | RFC 3830 [64] clause 6 with extensions in RFC 6509 [65] and RFC 6043 [66] | "MIKEY-Codec" |
| MIKEY_MCX_Extension_TypeDefs | TS 33.180 [43] Annex E.6 | "Key Parameter Payload Types" |
| MCDataMessage_TypeDefs | TS 24.282 [57] clause 15 | "MCData Messages" |
| MCPTT_MediaControl_TypeDefs |  |  |
| FloorControl_Group | TS 24.380 [10] clause 8.2 | "FloorCtrlCodec" |
| PreestablishedSessionControl_Group | TS 24.380 [10] clause 8.3 | "PreestablishedSessionCtrlCodec" |
| MCVideo_TransmissionControl_TypeDefs | TS 24.581 [56] clause 9 | "TransmissionCtrlCodec" |
| JSON | ES 201 873-11 [70] annex B, B.2 | "JSON" |

<End of modification>

<Start of modification>

### 9.2.1	MCX Client PIXIT

Table 9.2.1-1: MCX Client Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| Client relevant IXIT |  |  |  |  |
| px_MCX_Client_B_ID | charstring | "urn:uuid:cdcd34fb-c2f1-4488-a03a-6b5a5d360c47" |  | MCX client ID for a remote user: As defined in TS 24.379 [9] clause 4.10 it is a UUID URN according to RFC 4122 [62], i.e. has the format "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" |
| Users relevant IXIT |  |  |  |  |
| px_MCPTT_ID_User_A | charstring | "mcptt-user-A-id@ mcptt-op.gov" | NOTE 1 | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_ID_User_A | charstring |  | NOTE 1 | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_ID_User_A | charstring |  | NOTE 1 | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_A_Alias | charstring | "mcptt-user-A-alias" |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_User_A_Alias | charstring | "mcvideo-user-A-alias" |  | Alphanumeric alias of MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_User_A_Alias | charstring | "mcdata-user-A-alias" |  | Alphanumeric alias of MCData user. Ref. TS 24.483 [13]. |
| px_MCPTT_ID_FA_A | charstring | "mcptt-FA-A-id@ mcptt-op.gov" |  | The value is a "uri" attribute. TS 24.483 [13] clause 5.2.48W6 |
| px_MCData_ID_FA_A | charstring | "mcdata-FA-A-id@ mcptt-op.gov" |  | The value is a "uri" attribute. TS 24.483 [13] clause 10.2.97B3 |
| px_MCX_User_A_ParticipantType | charstring | "first responder" |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCX_DomainName_Organization_A | charstring | "mcptt-op.gov" |  | Domain name of the organization the user belongs to. |
| px_MCX_User_A_username | charstring | "MCPTT#U01" |  | UE's User username used for user authentication |
| px_MCX_User_A_password | charstring | "psw@MCPTT&7" |  | UE's User password used for user authentication |
| px_MCX_TokenRsaPrivateKey | octetstring |  |  | Private key being used to create signature for ID and Access Token |
| px_MCX_TrK | bitstring |  |  | Transport key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_TrK_ID | B32_Type |  |  | Transport key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_InK | bitstring |  |  | Integrity protection key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_InK_ID | B32_Type |  |  | Integrity protection key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCPTT_ID_User_B | charstring | "mcptt-user-B-id@ mcptt-op.gov" |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_ID_User_B | charstring | "mcvideo-user-B-id@ mcptt-op.gov" |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_ID_User_B | charstring | "mcdata-user-B-id@ mcptt-op.gov" |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |
| px_MCX_User_B_ParticipantType | charstring | "first responder" |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |
| px_MCPTT_ID_User_C | charstring | "mcptt-user-C-id@ mcptt-op.gov" |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_ID_User_C | charstring | "mcvideo-user-C-id@ mcptt-op.gov" |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_ID_User_C | charstring | "mcdata-user-C-id@ mcptt-op.gov" |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |
| px_MCX_User_C_ParticipantType | charstring | "first responder" |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |
| px_MCPTT_ID_User_D | charstring | "mcptt-user-D-id@ mcptt-op.gov" |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_ID_User_D | charstring | "mcvideo-user-D-id@ mcptt-op.gov" |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_ID_User_D | charstring | "mcdata-user-D-id@ mcptt-op.gov" |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |
| px_MCX_SIP_PublicUserId_A_1 | charstring |  |  | (First) public user id of the SIP subscriber (user A) for MCX; the parameter shall be a SIP URI |
| px_MCX_SIP_PrivateUserId_A | charstring |  |  | Private user id of the SIP subscriber (user A) for MCX |
| px_MCX_SIP_PrivateUserId_B | charstring |  |  | Private user id of the SIP subscriber (user B) for MCX |
| px_MCX_SIP_HomeDomain_A | charstring |  |  | Home domain name of the SIP subscriber (user A) for MCX |
| px_MCX_SIP_PublicUserId_B | charstring |  |  | Public user id of user B |
| px_MCX_SIP_PublicUserId_C | charstring |  |  | Public user id of user C |
| px_MCX_SIP_RegistrationWithTemporaryIdentities | boolean |  |  | If true the UE shall derive the public user id, private user id and home domain name from the IMSI for SIP registration |
| px_MCData_IPConnectivityEndpointAddress_B | charstring |  |  | IP address of the IP connectivity endpoint at the emulated client B: either an IPv4 or an IPv6 address |
| Groups relevant IXIT |  |  |  |  |
| px_MCPTT_Group_A_ID | charstring | "mcptt-group-A@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCVideo_Group_A_ID | charstring | "mcvideo-group-A@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCData_Group_A_ID | charstring | "mcdata-group-A@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_A_Name | charstring | "mcptt-group-A-name" |  | A human readable Group name for the group |
| px_MCVideo_Group_A_Name | charstring | "mcvideo-group-A-name" |  | A human readable Group name for the group |
| px_MCData_Group_A_Name | charstring | "mcdata-group-A-name" |  | A human readable Group name for the group |
| px_MCX_Group_A_Owner_Organization | charstring | "mcx-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_A_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| px_MCPTT_Group_B_ID | charstring | "mcptt-group-B@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCVideo_Group_B_ID | charstring | "mcvideo-group-B@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCData_Group_B_ID | charstring | "mcdata-group-B@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_T_ID | charstring | "mcptt-group-T@mcptt-op.gov" |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCVideo_Group_T_ID | charstring | "mcvideo-group-T@mcptt-op.gov" |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCData_Group_T_ID | charstring | "mcdata-group-T@mcptt-op.gov" |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| Miscellaneous IXIT |  |  |  |  |
| px_MCX_APN | charstring | "mcx-apn" |  | A single APN which the UE shall use to access each and all MCX relevant services. The APN is provided in the initial UE configuration as specified in TS 36.579-1 [2] Table 5.5.8.1-1. |
| px_MCX_InitialRegistration_TypeOfPDN1 | MCX_Registration_PDN_Type | mcx | ims, internet, mcx | First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’; ‘none’ is not applicable as first PDN) |
| px_MCX_InitialRegistration_TypeOfPDN2 | MCX_Registration_PDN_Type | none | ims, internet, mcx, none | Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration |
| px_MCX_InitialRegistration_TypeOfPDN3 | MCX_Registration_PDN_Type | none | ims, internet, mcx, none | Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration |
| px_MCX_CoordinateLatitude_Client_B | float | none |  | Simulated latitude value sent to the UE to inform about Client_B location |
| px_MCX_CoordinateLongitude_Client_B | float | none |  | Simulated longitude value sent to the UE to inform about Client_B location |
| px_MCX_CiphAlgo_Def | CiphAlgo | des_ede3_cbc | des_ede3_cbc, aes_cbc or nociph | Ciphering Algorithm for IPsec. NOTE: Unless specified otherwise in the test prose „nociph“ shall not be used for verification. |
| px_MCX_IPSecAlgorithm | IntAlgo | hmac_sha_1_96 | hmac_md5_96, hmac_sha_1_96 | Integrity Algorithm for IPsec |
| NOTE 1: According to TS 23.280 [54] clause 8.1.2 a MC service ID shall be a URI; nevertheless in context of this specification only URIs consisting of userinfo, host and path elements (see RFC 3986 [53]) are supported. Editor's note: A similar note may need to be added to other PIXITs too |  |  |  |  |

<End of modification>