3GPP TSG-RAN5 Meeting #105	R5-246581

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4716 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of single DCI test case 7.1.1.2.8 - FDRA - DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The definition and configuration for reference cell is not clear in NOTE1, it need core spec support to help reader to understand it. - Note2 is not clear, and RRCReconfiguration is not correct, it should be updated to RRCReconfigurationComplete - Loop mode should be set to on but return no data in uplink like 7.1.1.2.1/7.1.1.2.2.. to avoid side effects if UE deliver the MAC PDU received from SS to high layer. - physicalCellGroupConfig defined in the specific message content is not aligned with 38.331. physicalCellGroupConfig should be defined outside of spCellConfig - When SCell is configured to be scheduled by DCI 0-3 or DCI 1-3 in PCell, the SCell can only be scheduled by PCell, it cannot scheduled by the PDCCH on SCell, so the PDCCH on SCell can be deleted. [38.300 10.11] Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions: - When a serving cell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on a cell set, the PUSCH/PDSCH on serving cells in the cell set is always scheduled by a PDCCH on the serving cell; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add 38.213 10.1 into the Conformance requirements to explain the the two different configuration for reference cell. - Correct the Note1 and Note2 to clarify the reason for using DCI 00 to schedule RRCReconfigurationComplete. - Loop mode set to on but return no data in uplink. - Correct physicalCellGroupConfig. - Set PDCCH-Config to not present in the SCell in Table 7.1.1.2.8.1.3.3-12. - Correct typo and editorial update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | configuration is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.2.8	Multi-cell PDSCH scheduling with a single DCI / FDRA field based

7.1.1.2.8.1	Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Intra-band Contiguous CA

7.1.1.2.8.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state, SCell is configured, DCI format 1-3 is configured and ScheduledCellComboListDCI-1-3 is not configured }

ensure that {

when { UE receives DCI format 1-3 on scheduling Cell, and FDRA field of DCI format 1-3 indicated that PCell is scheduled Cell }

then { UE sends a HARQ feedback on the HARQ process }

}

(2)

with { UE in RRC_CONNECTED state, SCell is configured, DCI format 1-3 is configured and ScheduledCellComboListDCI-1-3 is not configured }

ensure that {

when { UE receives DCI format 1-3 on scheduling Cell, and FDRA field of DCI format 1-3 indicated that SCell is scheduled Cell }

then { UE sends a HARQ feedback on the HARQ process }

}

(3)

with { UE in  RRC_CONNECTED state, SCell is configured, DCI format 1-3 is configured and ScheduledCellComboListDCI-1-3 is not configured }

ensure that {

when { UE receives DCI format 1-3 on scheduling Cell, and FDRA field of DCI format 1-3 indicated that PCell and SCell are scheduled Cell }

then { UE sends HARQ-ACK information including two HARQ feedback and the timing of PUCCH carrying HARQ-ACK information is determined by the last PDSCH }

}

7.1.1.2.8.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.212 clause 7.3.1.0, 7.3.1.2.4 and T 38.213 clause 9.2.3 and 10.1.. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

…

Step 4B:

…

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

[TS 38.212, clause 7.3.1.2.4]

DCI format 1_3 is used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell.

The following information is transmitted by means of the DCI format 1_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Scheduled cell set indicator -  log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter MC-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter MC-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is not configured;

…

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3,  nBWP,RRCmax is the maximum number of DL BWPs configured by higher layers, excluding the initial DL bandwidth part, across all the cells configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured DL BWPs larger than 1, including the initial DL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

If scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with more than one entry, NcellDL is the number of scheduled cells indicated by Scheduled cells indicator field; if scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with only one entry, NcellDL is the number of cells configured by higher layer parameter scheduledCellComboListDCI-1-3; otherwise, NcellDL is the number of cells in the scheduled cell set configured by higher layer parameter scheduledCellListDCI-1-3. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K2NRBG, K2+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K2NRBG, K2+1/2, NRBG+1 bits if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', where NRBG, K2=NRBDL, BWP+NDL, BWPstartmodK2/K2, NRBDL, BWP is the size of the active DL bandwidth part, NDL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K2 is given by higher layer parameter resourceAllocationType1GranularityDCI-1-3. If the higher layer parameter resourceAllocationType1GranularityDCI-1-3 is not configured, K2 is equal to 1.

-	If resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K2NRBG, K2+1/2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

If the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

-	Time domain resource assignment - log2(ITDRA)  bits, where ITDRA is the number of entries in the higher layer parameter tdra-FieldIndexListDCI-1-3. This field is used to indicate an entry in the higher layer parameter tdra-FieldIndexListDCI-1-3 according to Table 7.3.1.2.4-2. Each entry in the higher layer parameter tdra-FieldIndexListDCI-1-3 contains the ‘Time domain resource assignment’ index for each BWP of each cell in the scheduled cell set, where the ‘Time domain resource assignment’ indexes for all the cells are placed according to an ascending order of a serving cell index, and the 'Time domain resource assignment' indexes for all the BWPs of a cell are placed according to an ascending order of the higher layer parameter BWP-Id.

-	VRB-to-PRB mapping - 0 or 1 bit

-	0 bit if the higher layer parameter vrb-ToPRB-Interleaver is not configured for any cell configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set;

…

-	PRB bundling size indicator - 0 or 1 bit

-	0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling' for any cell configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set;

…

-	Rate matching indicator - log2(IRM) bits, where IRM is the number of entries in the higher layer parameter rateMatchListDCI-1-3. This field is used to indicate an entry in the higher layer parameter rateMatchListDCI-1-3 according to Table 7.3.1.2.4-3. Each entry in the higher layer parameter rateMatchListDCI-1-3 contains the ‘Rate matching indicator’ index for each cell configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP in the scheduled cell set, where the ‘Rate matching indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘Rate matching indicator’ index is defined by the following:

-	0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger - log2(ICSIRS) bits, where ICSIRS is the number of entries in the higher layer parameter zp-CSI-RSListDCI-1-3. This field is used to indicate an entry in the higher layer parameter zp-CSI-RSListDCI-1-3 according to Table 7.3.1.2.4-4. Each entry in the higher layer parameter zp-CSI-RSListDCI-1-3 contains the ‘ZP CSI-RS trigger’ index for each cell configured with aperiodicZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP in the scheduled cell set, where the ‘ZP CSI-RS trigger’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘ZP CSI-RS trigger’ index is defined by the following:

-	0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured by higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModList.

For transport block 1:

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the modulation and coding scheme for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the new data indicator for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the redundancy version for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

For transport block 2:

…

-	HARQ process number - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the HARQ process number for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the HARQ process number for the cell with the smallest serving cell index. Each block is 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-3 configured for the cell corresponding to the block.

-	Downlink assignment index - number of bits as defined in the following

-	4 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the Downlink assignment index in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dL-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDynDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the two cells are the same.

-	One-shot HARQ-ACK request - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 or pdsch-HARQ-ACK-enhType3DCI-1-3 is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 is not configured;

…

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-3 is configured.

-	0 bit otherwise.

-	Antenna ports - number of bits determined by the following:

…

-	If antennaPortsDCI-1-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number  NcellDL

Each block corresponds to the Antenna ports information for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the Antenna ports information for the cell with the smallest serving cell index. Each block is defined below.

MAr above for the case of antennaPortsDCI-1-3= type1A or each block above for the case of antennaPortsDCI-1-3= type2 is defined by the following:

-	4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

-	Transmission configuration indication - number of bits determined by the following:

-	0 bit if higher layer parameter tci-PresentInDCI is not enabled;

…

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_3,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	SRS request - log2(ISRS) bits, where ISRS is the number of entries in the higher layer parameter srs-RequestListDCI-1-3. This field is used to indicate an entry in the higher layer parameter srs-RequestListDCI-1-3 according to Table 7.3.1.2.4-6. Each entry in the higher layer parameter srs-RequestListDCI-1-3 contains the ‘SRS request’ index for each cell in the scheduled cell set, where the ‘SRS request’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS request’ index is defined by the following:

-	2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - log2(Ioffset) bits, whereIoffset is the number of entries in the higher layer parameter srs-OffsetListDCI-1-3. This field is used to indicate an entry in the higher layer parameter srs-OffsetListDCI-1-3 according to Table 7.3.1.2.4-7. Each entry in the higher layer parameter srs-OffsetListDCI-1-3 contains the ‘SRS offset indicator’ index for each cell in the scheduled cell set, where the ‘SRS offset indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS offset indicator’ index is defined by the following:

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	DMRS sequence initialization - 1 bit. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field independently.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-3 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0DCI-1-3 is not configured;

…

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyDCI-1-3 or dormancyGroupWithinActiveTime is not configured;

…

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	0 bit if higher layer parameter pdcchMonAdaptDCI-1-3 is not enabled;

…

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDynDCI-1-3 is configured.

-	0 bit otherwise.

[TS 38.213, clause 9.2.3]

If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

[TS 38.213, clause 10.1]

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. If the UE monitors PDCCH candidates for detection of one or both of DCI format 0_3 and DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3, respectively, is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

7.1.1.2.8.1.3	Test description

7.1.1.2.8.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink,  System information combination NR-4 and in addition NR Cell 3 is configured as NR Active Scell.

7.1.1.2.8.1.3.2	Test procedure sequence

Table 7.1.1.2.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure SCell (NR Cell 3) and DCI format 1-3 on NR SpCell (NR Cell 1). (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell (NR Cell 1). | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits an Activation MAC control element to activate SCell on NR SpCell (NR Cell 1). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell (NR Cell 1) in slot N. | <-- | DCI format 1_3(UE C-RNTI, Frequency domain resource assignment indicated that only SpCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU on NR SpCell (NR Cell 1) in slot N. | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit HARQ ACK on NR SpCell (NR Cell 1) in slot N+K1? | --> | HARQ ACK | 1 | P |
| 8 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell (NR Cell 1) in slot N. | <-- | DCI format 1_3(UE C-RNTI, Frequency domain resource assignment indicated that only SCell is scheduled) | - | - |
| 9 | The SS transmits a MAC PDU on NR SCell (NR Cell 3) in slot N. | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit HARQ ACK on NR SpCell (NR Cell 1) in slot N+K1? | --> | HARQ ACK | 2 | P |
| 11 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell (NR Cell 1) in slot N. | <-- | DCI format 1_3(UE C-RNTI, Frequency domain resource assignment indicated that both SpCell and SCell are scheduled) | - | - |
| 12 | The SS transmits a MAC PDU on NR SpCell (NR Cell 1) and a MAC PDU on NR SCell (NR Cell 3) in slot N. | <-- | MAC PDU MAC PDU | - | - |
| 13 | Check: Does the UE transmit two HARQ ACK on NR SpCell (NR Cell 1) in slot N+K1? | --> | HARQ ACK, HARQ ACK | 3 | P |
| - | EXCEPTION: Steps 14a1 to 14a4 are executed if pc_dl_SchedulingOffset_PDSCH_TypeA | - | - | - | - |
| 14a1 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell (NR Cell 1) in slot N. | <-- | DCI format 1_3(UE C-RNTI, Frequency domain resource assignment indicated that both SpCell and SCell are scheduled, Time domain resource assignment indicated that K0 between PDCCH in step14a1 and PDSCH in step14a3 is not zero) | - | - |
| 14a2 | The SS transmits a MAC PDU on NR SpCell (NR Cell 1) in slot N. | <-- | MAC PDU | - | - |
| 14a3 | The SS transmits a MAC PDU on NR SCell (NR Cell 3) in slot X. | <-- | MAC PDU | - | - |
| 14a4 | Check: Does the UE transmit two HARQ ACK on NR SpCell (NR Cell 1) in the slot X+K1? | --> | HARQ ACK, HARQ ACK | 3 | P |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is also NR SpCell (NR Cell 1). Therefore, the single DCI (DCI format 1_3) is counted on the NR Cell 1. Note 2: The NR RRCReconfigurationComplete message is scheduled by DCI format 0_0 to avoid DCI size misalignment for DCI format 0_1 (according to TS 38.212 [14] clause 7.3.1.0 step 4C) between UE and SS during the reconfiguration. |  |  |  |  |  |

7.1.1.2.8.1.3.3	Specific message contents

Table 7.1.1.2.8.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.2.8.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-2: CellGroupConfig (Table 7.1.1.2.8.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.2.8.1.3.3-10 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.2.8.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.2.8.1.3.3-5 |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.2.8.1.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-3: ServingCellConfig (Table 7.1.1.2.8.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition DCI 1_3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.2.8.1.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.2.8.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-5: ServingCellConfigCommon (Table 7.1.1.2.8.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 7.1.1.2.8.1.3.3-6 |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-6: DownlinkConfigCommon (Table 7.1.1.2.8.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-52 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.2.8.1.3.3-7 |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-7: BWP-DownlinkCommon (Table 7.1.1.2.8.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon | Table 7.1.1.2.8.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-8: PDSCH-ConfigCommon (Table 7.1.1.2.8.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-101 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList | Table 7.1.1.2.8.1.3.3-9 |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-9: PDSCH-TimeDomainResourceAllocationList (Table 7.1.1.2.8.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 10 |  | SCS 30 |
|  | 5 |  | SCS 15 OR SCS 120 |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-10: PhysicalCellGroupConfig (Table 7.1.1.2.8.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  |  |
|  | semiStatic |  | pc_harqFeedbackType2_DCI_1_3=False AND pc_harqFeedbackType1_DCI_1_3=True |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-11: ServingCellConfig (Table 7.1.1.2.8.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.2.8.1.3.3-12 |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-12: BWP-DownlinkDedicated (Table 7.1.1.2.8.1.3.3-11)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.8.1.3.3-13: Physical layer parameters for DCI format 1_3 (steps 5, 8, 11 and 14a1, Table 7.1.1.2.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 5 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=2 | - | Step 8 |
|  | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=2 | - | Step 11,14a1 |
| Time domain resource assignment | The first entry of tdra-FieldIndexListDCI-1-3 | “00” | Step 5,8,11 |
|  | The second entry of tdra-FieldIndexListDCI-1-3 | “01” | Step 14a1 |

7.1.1.2.8.2	Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Inter-band CA

The scope and description of the present TC is the same as test case 7.1.1.2.8.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

7.1.1.2.8.3	Multi-cell PDSCH scheduling with a single DCI / FDRA field based / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.2.8.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

<End of modified section 1>