3GPP TSG-RAN5 Meeting #105	R5-246590

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

Source:	Huawei, HiSilicon, MCC TF160

Title:	Discussion to add new MBS test case to cover multiple beams scenario

Agenda item:	6.4.7

Document for:	Endorsement

Discussion

Observation 1: The field mtch-SSB-MappingWindowList-r17 and mtch-SSB-MappingWindowIndex-r18 are not tested. These field are only used for multi-SSB scenario, i.e Different SSB use different PDCCH monitoring occasion for receiving MBS broadcast.

- Normally SIG test case only use one SSB. If we try to cover this scenario, at least two SSB is needed to be configured in test case. Several SIG test cases TC 7.1.1.1.4, 8.2.3.9.1, 8.2.3.10.1, 8.1.3.1.13 already includes multi-beams, so two SSBs configuration will not impact test model in TTCN.

- Under multi-SSB scenario, mtch-SSB-MappingWindowIndex-r18 is mandatory present, so it is worth to cover it in the test case.

[38.331 6.3.6]

MBS-SessionInfoMulticast-r18 ::=   SEQUENCE {

mbs-SessionId-r18                  TMGI-r17,

g-RNTI-r18                         RNTI-Value                                   OPTIONAL, -- Need R

mrb-ListMulticast-r18              MRB-ListMulticast-r18                        OPTIONAL, -- Need R

mtch-SchedulingInfo-r18            DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r18             BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r18              PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r18    MTCH-SSB-MappingWindowIndex-r17              OPTIONAL, -- Cond MTCH-Mapping

thresholdIndex-r18                 INTEGER (0..maxNrofThresholdMBS-1-r18)       OPTIONAL, -- Need R

pdcp-SyncIndicator-r18             ENUMERATED {true}                            OPTIONAL, -- Cond RRCRelease

stopMonitoringRNTI-r18             ENUMERATED {true}                            OPTIONAL, -- Cond G-RNTI

...

}

MTCH-SSB-MappingWindowList-r17 ::= SEQUENCE (SIZE (1..maxNrofMTCH-SSB-MappingWindow-r17)) OF MTCH-SSB-MappingWindowCycleOffset-r17

MTCH-SSB-MappingWindowCycleOffset-r17 ::= CHOICE {

ms10                                      INTEGER(0..9),

ms20                                      INTEGER(0..19),

ms32                                      INTEGER(0..31),

ms64                                      INTEGER(0..63),

ms128                                     INTEGER(0..127),

ms256                                     INTEGER(0..255)

}

MBS-SessionInfoListMulticast field descriptions

mtch-SSB-MappingWindowIndex Indicates the index of MTCH-SSB-MappingWindowCycleOffset configuration entry in MTCH-SSB-MappingWindowList. The value 0 corresponds to the first entry in MTCH-SSB-MappingWindowList, the value 1 corresponds to the second entry in MTCH-SSB-MappingWindowList and so on. This field is set to the same value for all MBS sessions mapped to the same G-RNTI.

MTCH-SSB-MappingWindowList field descriptions

MTCH-SSB-MappingWindowCycleOffset Indicates the cycle and offset for MTCH PDCCH ocassions to SSB mapping. Values in unit of ms. ms10 corresponds to cycle of 10 ms with corresponding offset between 0 and 9 ms, value ms20 corresponds to cycle of 20 ms with corresponding offset between 0 and 19 ms, and so on. The mapping window starts at a subframe in a SFN where [(SFN number × 10) + subframe number] modulo (cycle) = offset. PDCCH monitoring occasions for MTCH in a mapping window which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered starting from 1 in the maping window. The [x×N+K]th PDCCH monitoring occasion for MTCH in this mapping window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH to SSB mapping transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes.

| Conditional Presence | Explanation |
| --- | --- |
| MTCH-Mapping | The field is mandatory present if the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 is more than 1, and searchSpaceMulticastMTCH is not set to zero (including the case where searchSpaceMulticastMTCH is absent and searchSpaceMulticastMCCH is not set to zero). Otherwise, it is absent, Need R. |

Proposal

Based on the above discussion, the followings are proposed:

Proposal 1: Add a new MBS test case to check that UE could receive the MBS broadcast under multiple beams scenario. Two SSBs(SSB#1,SSB#0) are configured in ssb-PositionsInBurst in SIB1, and then mtch-SSB-MappingWindowList-r17/ mtch-SSB-MappingWindowIndex-r17 are configured to indicate UE which PO is for SSB#1, which PO is for SSB#0. [1-2]

Reference

[1] R5-246594 Addition of MBS broadcast test case 14.1.1.5-Multiple beams, Huawei, HiSilicon, MCC TF160, RAN5#105

[2] R5-246595 Add test applicability for MBS broadcast test case 14.1.1.5, Huawei, HiSilicon, MCC TF160, RAN5#105