3GPP TSG-RAN5 Meeting #105	R5-246612

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5369 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ACB per PLMN test case 8.1.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As outlined in discussion paper R5-242949, it is necessary to introduce test cases for the ac-BarringPerPLMN-List parameters to meet Japanese requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.2.7a – " RRC Connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed / AC-Barring per PLMN" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not guaranteed up on reception of IEs related to Access Control per PLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.7a (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1473 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.2.7a	RRC Connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed / AC-Barring per PLMN

8.1.2.7a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having an Access Class with a value in the range 11..15 and having received a SystemInformationBlockType2 message including ac-BarringInfo-r12 indicating 0% access probability for AC 0..9 for both MO data and MO signalling, access restriction for AC 10 and no restriction for AC 11..15 for both MO data and MO signalling }

ensure that {
  when { UE has user data pending }

then { UE transmits an RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having an Access Class with a value in the range 11..15 }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 message including ac-BarringInfo-r12 indicating 0% access probability for AC 0..9 for both MO data and MO signalling, access restriction for AC 10 and no restriction for AC 11..15 for both MO data and MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.7a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

...

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

...

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

...

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

...

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

- initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

8.1.2.7a.3	Test description

8.1.2.7a.3.1		Pre-test conditions

System Simulator:

-	Cell 1 and 11, as specified in TS 36.508[18] clause 4.4.1.2 are configured according to Table 4.4.2-2 in [18] except for multiple broadcasted PLMN identities as shown in Table 8.1.2.7a.3.1-1: PLMN identifiers.

Table 8.1.2.7a.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1,11 | PLMN1 | 001 | 01 |
|  | PLMN2 | 001 | 02 |

UE:

-	The UE is equipped with a USIM containing default values (as per TS 36.508[18]) except for those listed in Table 8.1.2.7a.3.1-2.

Table 8.1.2.7a.3.1-2: USIM configuration

| USIM field | Value |
| --- | --- |
| EFFPLMN | PLMN2 |
| EFACC | File size: 2 Bytes Default values (BIN): Byte 1: 000100** Byte 2: ******** |
| Note: The test house may set any single bit shown by "*" to "1". This determines the access control class of the USIM |  |

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508[18].

8.1.2.7a.3.2	Test procedure sequence

Table 8.1.2.7a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions at the beginning of each iteration of the test body, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.2.7a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy SrxlevCell 1 > SrxlevCell 11. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Off | Off | The power level values are set to “cell off”. |

Table 8.1.2.7a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 11 level according to the row "T0" in table 8.1.2.7a.3.2-1. | - | - | - | - |
| 2 | The UE is switched on and brought to state Loop Back Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18]. | - | - | - | - |
| 3 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 4 | Wait for 1 s after the IP packet has been transmitted in step 4. (Note 1) | - | - | - | - |
| 5 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 7 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 8 | The UE transmits an RRCConnectionSetupComplete message on Cell 1. This message includes a SERVICE REQUEST message. | --> | RRCConnectionSetupComplete | - | - |
| 9 | The SS transmits a SecurityModeCommand message on Cell 1. | <-- | SecurityModeCommand | - | - |
| 10 | The UE transmits a SecurityModeComplete message on Cell 1. | --> | SecurityModeComplete | - | - |
| 11 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: Steps 12 and 13 can occur in any order. | - | - | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | The UE loop backs the IP packet received in step 4 on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 14 | The SS sends OPEN TEST LOOP message to open the UE test loop. | - | DLInformationTransfer | - | - |
| 15 | The UE responds with OPEN TEST LOOP COMPLETE message. | - | ULInformationTransfer | - | - |
| 16 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 17 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 18 | The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.7a.3.2-1. | - | - | - | - |
| 19 | Check: Does the test result of generic test procedure in TS 36.508[18] subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 11, and that the UE has performed TAU procedure with establishment cause ‘high priority access’? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |
| Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 4 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 6. |  |  |  |  |  |

8.1.2.7a.3.3	Specific message contents

Table 8.1.2.7a.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 11 (preamble and all steps, Table 8.1.2.7a.3.2-2)

| Derivation path: 36.508[18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { |  |  |  |
| plmn-Identity[1] | 00101 | PLMN1 |  |
| plmn-Identity[2] | 00102 | PLMN2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.7a.3.3-2: SystemInformationBlockType2 for Cell 1 and Cell 11 (preamble and all steps, Table 8.1.2.7a.3.2-2)

| Derivation Path: 36.508[18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE { |  |  |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | TRUE |  |  |
| ac-BarringForMO-Signalling-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.7a.3.3-3: CLOSE UE TEST LOOP (preamble, Table 8.1.2.7a.3.2-2)

| Derivation Path: 36.508[18], Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1010'B | 10 seconds |  |

Table 8.1.2.7a.3.3-4: RRCConnectionRequest (step 6 and step 19, Table 8.1.2.7a.3.2-2)

| Derivation Path: 36.508[18], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.7a.3.3-5: RRCConnectionSetupComplete (step 8, Table 8.1.2.7a.3.2-2)

| Derivation Path: 36.508[18], Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>