3GPP TSG-RAN5 Meeting #105	R5-246616

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4730 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a new R18 MUSIM test case 8.1.1.4.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new NR MUSIM test case 8.1.1.4.10 for early indication feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 8.1.1.4.10 or NR MUSIM devices is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.10(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR .38.523-2.. CR .0530.. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.1.4.10	RRC resume / MUSIM / early indication

8.1.1.4.10.1	Test Purpose (TP)

(1)

with { UE received SIB1 containing musim-CapRestrictionAllowed and UE is in RRC_CONNECTED state in network A and received RRCRelease message containing suspendConfig IE from network B and enters into RRC_INACTIVE state in network B }

ensure that {

when { UE capability is restricted for MUSIM operation and it is going on RRC Resume procedure on network B }

then { UE includes the musim-CapRestrictionInd in the RRCResumeComplete message }

}

8.1.1.4.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.3.13.4, and TS 38.300, clause 20.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.13.4]

The UE shall:

…

- set the content of RRCResumeComplete message as follows:

…

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCResumeComplete message upon determining it has temporary capability restriction;

[TS 38.300, clause 20.4]

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Network A is NR and Network B can either be E-UTRA or NR. The MUSIM device may request a temporary capability restriction only after the Network signals via RRC that this is allowed.

When configured to do so, a MUSIM device can indicate one or more of the following temporary capability restriction or removal of restriction to the Network A:

-	A MUSIM device can explicitly request SCell(s) or SCG to be released;

-	A MUSIM device can indicate its preference on temporary maximum MIMO layers and/or supported channel bandwidth for specific serving cells for both UL/DL;

-	A MUSIM device can indicate its preference on the temporary maximum number of CCs per UL/DL;

-	A MUSIM device can indicate its preference on the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. For affected band(s) and band combination(s), this preference can include temporary maximum MIMO layers and/or supported channel bandwidth for both UL/DL;

-	A MUSIM device can indicate the measurement gap requirement changes.

When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete/RRCReestablishmentComplete message while the MUSIM device is already in RRC_CONNECTED state in Network B.

When a MUSIM device is in RRC_CONNECTED state in both Network A and Network B, if Network B is NR, it is up to UE implementation to select which network to request temporary UE capability restriction; if Network B is E-UTRA, the request for temporary UE capability restriction can only be performed on Network A.

8.1.1.4.10.3	Test description

8.1.1.4.10.3.1	Pre-test conditions

System Simulator:

-   NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [4] and belong to the different frequency.

-  NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-  NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell ''.

UE:

- 	The UE is a Rel-18 MUSIM UE;

-    The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-    The UE is in 5GS state 3N-A on NR Cell 1 and in 5GS state 2N-A on NR Cell 12 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Table 4.4A.2-2.

-    The SIB1 of NR Cell 12 and NR Cell 1 are defined in TS 38.508-1 [4], clause 4.6.1-28 with condition MUSIM_r18 for all steps.

8.1.1.4.10.3.2	Test procedure sequence

Table 8.1.1.4.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NR Cell 12 unless explicitly stated otherwise. | - | - | - | - |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 3 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 4 | Check: Does UE transmit an RRCResumeComplete message include musim-CapRestrictionInd which set to true? | --> | NR RRC: RRCResumeComplete | 1 | P |
| NOTE: When UE supports Rel-18 MUSIM feature and is in RRC_CONNECTED mode on NR Cell 1,when UE needs data transmission on NR Cell 12, it determines that it has temporary capability restriction. |  |  |  |  |  |

8.1.1.4.10.3.3	Specific message contents

Table 8.1.1.4.10.3.3-1: Paging (step 1, Table 8.1.1.4.10.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.1.4.10.3.3-2: RRCResumeComplete (step 4, 8.1.1.4.10.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message |  | Not checked |  |  |
| selectedPLMN-Identity |  | Not checked |  |  |
| uplinkTxDirectCurrentList |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | Not present |  |  |
| measResultIdleEUTRA-r16 |  | MeasResultIdleEUTRA-r16 |  |  |
| measResultIdleNR-r16 |  | MeasResultIdleNR-r16 |  |  |
| scg-Response-r16 |  | Not present |  |  |
| ue-MeasurementsAvailable-r16 |  | UE-MeasurementsAvailable-r16 |  |  |
| mobilityHistoryAvail-r16 |  | Not present |  |  |
| mobilityState-r16 |  | Not present |  |  |
| needForGapsInfoNR-r16 |  | NeedForGapsInfoNR-r16 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| uplinkTxDirectCurrentTwoCarrierList-r16 |  | UplinkTxDirectCurrentTwoCarrierList-r16 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| needForGapNCSG-InfoNR-r17 |  | NeedForGapNCSG-InfoNR-r17 |  |  |
| needForGapNCSG-InfoEUTRA-r17 |  | NeedForGapNCSG-InfoEUTRA-r17 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| uplinkTxDirectCurrentMoreCarrierList-r17 |  | UplinkTxDirectCurrentMoreCarrierList-r17 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| needForInterruptionInfoNR-r18 |  | NeedForInterruptionInfoNR-r18 |  |  |
| musim-CapRestrictionInd-r18 |  | true |  |  |
| flightPathInfoAvailable-r18 |  | Not present |  |  |
| measConfigReportAppLayerAvailable-r18 |  | Not present |  |  |
| measResultReselectionNR-r18 |  | MeasResultIdleNR-r16 |  |  |
| reselectionMeasAvailable-r18 |  | Not present |  |  |
| nonCriticalExtension |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |