3GPP TSG-RAN5 Meeting #105	R5-246647

# Orlando, United States, 18th - 22nd November 2024

Agenda Item:	6.6.9.9

Source:	MCC TF160

Title:	Addition of Host header field to HTTP requests in 36.579-1 clause 5.5.4

Document for:	Endorsement

# 1	Introduction and discussion

According to RFC 2616 clause 14.23 "A client MUST include a Host header field in all HTTP/1.1 request messages". Nevertheless, so far HTTP requests sent by the client are checked to contain a Host header field only in cases where the MCX core specs explicitly specify a particular URI to be used in the Host header field (e.g. in case of MCData file distribution using HTTP). On the other hand, especially when the Request URI is an "abs_path" rather than an "absoluteURI" in terms of RFC 2616 clause 5.1.2 an HTTP proxy may need the information of the Host header field as it is not contained in the Request URI.

# 2	Proposal

Host header fields shall be added to the HTTP requests specified in clause 5.5.4 of 36.579-1:

Table 5.5.4.2-1: HTTP GET

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "GET" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCX_IdMS_auth_UriPath | points to the Authorisation endpoint of the IdM Server | TS 33.180 [94] | AUTH |
|  | px_MCX_InitialConfigServer_UriPath | points to initial UE Configuration document | TS 24.484 [14] | UEINITIALCONFIG |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID1 & "/users/" & XUI & ue-config-docname | points to UE Configuration document (NOTE 1a, 2, 3, 5) | TS 24.484 [14] | UECONFIG |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID2 & "/users/" & XUID & ""/" & user-profile-docname | points to UE User Profile document (NOTE 1b, 2, 4) | TS 24.484 [14] | UEUSERPROF |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID3 & "/global/service-config.xml" | points to UE Service Configuration document (NOTE 1c, 2) | TS 24.484 [14] | UESERVCONFIG |
|  | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & group-id | points to group configuration document (NOTE 6) | TS 24.481 [11] | GROUPCONFIG |
|  | URI as contained in the payload of the FD SIGNALLING PAYLOAD from the SS indicating the file upload |  |  | FD_HTTP |
| query | As described in Table 5.5.4.10.1-1 |  | TS 33.180 [94] | AUTH |
| HTTP-Version | "HTTP/1.1" |  |  |  |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  |  | RFC 2617 [72] | UECONFIG UEUSERPROF UESERVCONFIG GROUPCONFIG FD_HTTP MSG_STORE |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Authorization | not present |  |  |  |
| Host |  |  | RFC 2616 [26] | AUTH |
| host | px_MCX_IdMS_auth_IPAddress |  |  |  |
| port | px_MCX_IdMS_auth_Port if present |  |  |  |
| Host |  |  | RFC 2616 [26] | UEINITIALCONFIG |
| host | px_MCX_InitialConfigServer_IPAddress |  |  |  |
| port | px_MCX_InitialConfigServer_Port if present |  |  |  |
| Host |  |  | RFC 2616 [26] | UECONFIG UEUSERPROF UESERVCONFIG |
| host | xcap." & tsc_MCX_CMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  | RFC 2616 [26] | GROUPCONFIG |
| host | xcap." & tsc_MCX_GMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  | RFC 2616 [26] | FD_HTTP |
| host | tsc_MCData_MSF_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  |  | MSG_STORE |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.1.1 |  |
| port | not present |  |  |  |
| Content-Type | if present |  |  | AUTH |
| media-type | "application/x-www-form-urlencoded" |  |  |  |
| Content-Type | Not present |  |  |  |
| Message-body | Not present |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcptt.ue-config" for Condition MCPTT AUID1 = "org.3gpp.mcvideo.ue-config" for Condition MCVIDEO AUID1 = "org.3gpp.mcdata.ue-config" for Condition MCDATA NOTE 1b: AUID2 = "org.3gpp.mcptt.user-profile" for Condition MCPTT AUID2 = "org.3gpp.mcvideo.user-profile" for Condition MCVIDEO AUID2 = "org.3gpp.mcdata.user-profile" for Condition MCDATA NOTE 1c: AUID3 = "org.3gpp.mcptt.service-config" for Condition MCPTT AUID3 = "org.3gpp.mcvideo.service-config" for Condition MCVIDEO AUID3 = "org.3gpp.mcdata.service-config" for Condition MCDATA NOTE 2: XUID = "sip:" & px_MCPTT_ID_User_A for Condition MCPTT XUID = "sip:" & px_MCVideo_ID_User_A for Condition MCVIDEO XUID = "sip:" & px_MCData_ID_User_A for Condition MCDATA NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: user-profile-docname = "mcptt-user-profile-" & profile-index & ".xml" for Condition MCPTT user-profile-docname = "mcvideo-user-profile-" & profile-index & ".xml" for Condition MCVIDEO user-profile-docname = "mcdata-user-profile-" & profile-index & ".xml" for Condition MCDATA with profile-index being the same as in the <user-profile-index> attribute of the corresponding document NOTE 5: ue-config-docname = "mcptt-ue-configuration.xml" for Condition MCPTT ue-config-docname = "mcvideo-ue-configuration.xml" for Condition MCVIDEO ue-config-docname = "mcdata-ue-configuration.xml" for Condition MCDATA NOTE 6: group-id = px_MCPTT_Group_A_ID for Condition MCPTT group-id = px_MCVideo_Group_A_ID for Condition MCVIDEO group-id = px_MCData_Group_A_ID for Condition MCDATA |  |  |  |  |

Table 5.5.4.3-1: HTTP POST

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method | "POST" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCX_IdMS_auth_UriPath | points to the Authorisation endpoint of the IdM Server | TS 33.180 [94] | AUTH |
|  | tsc_MCX_IdMS_userauth_UriPath | points to the endpoint verifying the user authentication; same URI as provided to the UE in the action attribute of the HTML login form | TS 33.180 [94] HTML 4.01 Specification [105] | USERAUTH |
|  | tsc_MCX_IdMS_token_UriPath | points to the Token endpoint of the IdM Server | TS 33.180 [94] | TOKEN |
|  | UriScheme & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath | "KMS Initialize" request according to TS 33.180 [94] D.2.3 (NOTE 2) | TS 33.180 [94] | KMSINIT |
|  | UriScheme & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath | "KMS KeyProvision" request according to TS 33.180 [94] D.2.4 (NOTE 2) | TS 33.180 [94] | KMSKEY |
|  | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & temporary-group-id | Points to the temporary group configuration document to be created (NOTE 1) | TS 24.481[11] clause 6.3.14.2 | TEMPGROUP |
|  | tsc_MCData_MSF_URI | The absolute URI identifying the resource on a media storage function | TS 24.282 [87], clause 10.2.2.1 | FD_HTTP |
| HTTP-Version | "HTTP/1.1" |  |  |  |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  |  | RFC 2617 [72] | KMSINIT, KMSKEY, TEMPGROUP, FD_HTTP, MSG_STORE |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Host |  |  | RFC 2616 [26] | AUTH USERAUTH |
| host | px_MCX_IdMS_auth_IPAddress |  |  |  |
| port | px_MCX_IdMS_auth_Port if present |  |  |  |
| Host |  |  | RFC 2616 [26] | TOKEN |
| host | px_MCX_IdMS_token_IPAddress |  |  |  |
| port | px_MCX_IdMS_token_Port if present |  |  |  |
| Host |  |  | RFC 2616 [26] | KMSINIT KMSKEY |
| host | tsc_MCX_KMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  | RFC 2616 [26] | TEMPGROUP |
| host | xcap." & tsc_MCX_GMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  |  | FD_HTTP |
| host | tsc_MCData_MSF_Hostname | hostname identifying the media storage function | TS 24.282 [87], clause 10.2.2.1 |  |
| port | not present |  |  |  |
| Host |  |  |  | MSG_STORE |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.1.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  | AUTH, USERAUTH, TOKEN |
| media-type | "application/x-www-form-urlencoded" |  |  |  |
| Content-Type |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| media-type | "application/xml" |  | RFC 7303 [112] |  |
| Content-Type |  |  |  | TEMPGROUP |
| media-type | "application/vnd.3gpp.GMOP+xml" |  |  |  |
| Content-Type |  |  |  | FD_HTTP |
| media-type | "multipart/mixed" |  | TS 24.282 [87], clause 10.2.2.1 |  |
| Message-body |  |  |  | AUTH |
| Authentication Request | As described in Table 5.5.4.10.1-1 |  |  |  |
| Message-body |  |  | HTML 4.01 Specification [105] | USERAUTH |
| user | px_MCX_User_A_username |  |  |  |
| password | px_MCX_User_A_password |  |  |  |
| Message-body |  |  |  | TOKEN |
| Token request | As described in Table 5.5.4.10.3-1 |  |  |  |
| Message-body |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| Signed KMS Request | As described in Table 5.5.4.10.9-1 |  |  |  |
| Message-body |  |  |  | TEMPGROUP |
| Temporary Group Creation Document" | As described in Table 5.5.7.4-2 |  |  |  |
| Message-body |  |  |  | FD_HTTP |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| MIME-part-body | MCData-Info described in Table 5.5.3.2.1-3 |  |  |  |
| MIME body part |  | File content | TS 24.282 [87] clause 10.2.2.1 |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/octet-stream" |  |  |  |
| MIME-part-body | binary data representing the file |  |  |  |
| NOTE 1: temporary-group-id = px_MCPTT_Group_T_ID for Condition MCPTT temporary-group-id = px_MCVideo_Group_T_ID for Condition MCVIDEO temporary-group-id = px_MCData_Group_T_ID for Condition MCDATA NOTE 2: UriScheme is either "http" or "https" |  |  |  |  |

Table 5.5.4.4-1: HTTP PUT

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "PUT" |  |  |  |
| Request-URI | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & document name (NOTE 1) | XCAP URI in users tree where the XUI is set to a group creation XUI configuration parameter | TS 24.481 [11] clause 6.3.2.2.1 | GROUPCREATE |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  | TS 24.482 [12] A.2.3: Expected by the server to validate and identify the client | RFC 2617 [72] |  |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Host |  |  | RFC 2616 [26] | GROUPCREATE |
| host | xcap." & tsc_MCX_GMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  |  | MSG_STORE |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.1.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  | GROUPCREATE |
| media-type | application/vnd.oma.poc.groups+xml |  |  |  |
| Message-body |  |  |  | GROUPCREATE |
| Group Creation Document | As described in Table 5.5.7.4-1 |  |  |  |
| NOTE 1: document name is the name of the group document contained in the message body |  |  |  |  |

Table 5.5.4.5-1: HTTP DELETE

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "DELETE" |  |  |  |
| Request-URI | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & temporary-group-id | Points to the group configuration document (NOTE 1) | TS 24.481 [11] | TEMPGROUP |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  | TS 24.482 [12] A.2.3: Expected by the server to validate and identify the client | RFC 2617 [72] |  |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Host |  |  | RFC 2616 [26] | TEMPGROUP |
| host | xcap." & tsc_MCX_GMS_Hostname |  |  |  |
| port | not present |  |  |  |
| Host |  |  |  | MSG_STORE |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.1.1 |  |
| port | not present |  |  |  |
| NOTE 1: temporary-group-id = px_MCPTT_Group_T_ID for Condition MCPTT temporary-group-id = px_MCVideo_Group_T_ID for Condition MCVIDEO temporary-group-id = px_MCData_Group_T_ID for Condition MCDATA |  |  |  |  |