3GPP TSG-RAN5 Meeting #105	R5-246758

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3372 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to further coverage enhancement SIG test cases 7.1.1.1.X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update Further Cov_Enh SIG test cases 7.1.1.1.20 / 7.1.1.1.21 / 7.1.1.1.22 based on comments from MCC TF160: - Editorial issues in test purposes; - Normative reference needs to align with latest version of 38.321; - Messages in test procedure of TC 7.1.1.1.21 needs to be updated; - Message contents needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.20, 7.1.1.1.21, 7.1.1.1.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.1.1.20	Random access procedure / Msg1 repetition / Contention-based

7.1.1.1.20.1	Test Purpose (TP)

(1)

with { UE is in RRC_idle state and is configured with RACH resources for both legacy RACH and Msg1 repetition }

ensure that {

when { PL-RS RSRP results is higher than the threshold(s) for Msg1 repetition }

then { UE performs legacy RACH procedure }

}

(2)

with { UE is in RRC_idle state and is configured with RACH resources for both legacy RACH and Msg1 repetition and UE is performing legacy RACH }

ensure that {

when { UE is performing legacy RACH procedure and PREAMBLE_TRANSMISSION_COUNTER exceeds preambleTransMax }

then { UE does not perform Msg1 repetition }

}

(3)

with { UE is in RRC_idle state and is configured with RACH resources for both legacy RACH and Msg1 repetition }

ensure that {

when { PL-RS RSRP results is lower than the threshold for Msg1 repetition }

then { UE performs Msg1 repetition }

}

(4)

with { UE is in RRC_idle state and is configured with RACH resources for different Msg1 repetition numbers and is performing Msg1 repetition }

ensure that {

when { PREAMBLE_TRANSMISSION_COUNTER exceeds preambleTransMax-Msg1-Repetition or 2*preambleTransMax-Msg1-Repetition }

then { UE fallbacks to next higher PRACH repetition number }

}

(5)

with { UE is in RRC_idle state and is configured with RACH resources for Msg1 repetition and is performing Msg1 repetition }

ensure that {

when { UE receives a RAR scrambled by RA-RNTI based on the RO used by the last preamble of Msg1 repetition }

then { UE transmits Msg3 }

}

7.1.1.1.20.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.213 clause 8.1, 8.2, TS 38.321 clause 5.1.1b, 5.1.2, 5.1.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.213 clause 8.1]

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

…

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Within a time period, for set(s) of Npreamblerep valid PRACH occasions for a PRACH transmission with Npreamblerep preamble repetitions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any, is determined according to an ordering of valid PRACH occasions

-	first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	second, in increasing order of time resource indexes for time multiplexed PRACH occasions

where, for each frequency resource index for frequency multiplexed PRACH occasions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any,

-	is after msg1-RepetitionTimeOffsetROGroup consecutive valid PRACH occasions in time from the first valid PRACH occasion of the previous set, where each PRACH occasion is associated with same SS/PBCH block index(es) and each SS/PBCH block index is associated with same preambles, if msg1-RepetitionTimeOffsetROGroup is provided

-	is after the PRACH occasions for the previous set, if msg1-RepetitionTimeOffsetROGroup is not provided

…

[TS 38.213 clause 8.2]

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

…

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

…

1>	else if contention free Random Access Resources have not been provided for this Random Access procedure and the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources with msg1-Repetitions set to true and set(s) of Random Access resources without msg1-Repetitions set to true:

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 8 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 8.

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 4 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 4.

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 2 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 2.

2>	else if the RSRP of the downlink pathloss reference is not less than any configured rsrp-ThresholdMsg1-RepetitionNumX:

3>	assume Msg1 repetition is not applicable for the current Random Access procedure.

[TS 38.321, clause 5.1.3]

…

1>	else if an SSB is selected above:

2>	if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure:

3>	determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB).

…

[TS 38.321, clause 5.1.2]

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

[TS 38.321, clause 5.1.4]

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

…

2>	if the Random Access procedure is not completed:

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, ssb-SharedRO-MaskIndex and numberOfRA-PreamblesGroupA parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

…

7.1.1.1.20.3	Test description

7.1.1.1.20.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

7.1.1.1.20.3.2	Test procedure sequence

Tables 7.1.1.1.20.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T0" is applied for Preamble. Configurations marked "T1" are applied at the points indicated in the Main behaviour description in Table 7.1.1.1.20.3.2-3.

Table 7.1.1.1.20.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -93 | The power level is such that RSRP of PL-RS < rsrp-ThresholdMsg1-RepetitionNum2 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -77 | The power level is such that RSRP of PL-RS > rsrp-ThresholdMsg1-RepetitionNum8 |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |

Table 7.1.1.1.20.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -99 | The power level is such that RSRP of PL-RS < rsrp-ThresholdMsg1-RepetitionNum2 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -71 | The power level is such that RSRP of PL-RS > rsrp-ThresholdMsg1-RepetitionNum8 |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |

Table 7.1.1.1.20.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched UE identity. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit PRACH preambles with preamble index k on 2 consecutive ROs starting from the RO #n in radio frame #m, where - 8 ≤ k < 16 for FR1, 4 ≤ k < 8 for FR2; - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2. | --> | PRACH preamble | 3 | P |
| 3 | The SS ignores UE’s RACH attempts to let PREAMBLE_TRANSMISSION_COUNTER > preambleTransMax-Msg1-Repetition-r18 | - | - | - | - |
| 4 | Check: Does the UE transmit a PRACH preamble with preamble index k on 4 consecutive ROs starting from the RO #n in radio frame #m, where - 16 ≤ k < 24 for FR1, 8 ≤ k < 12 for FR2; - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2. | --> | PRACH preamble | 4 | P |
| 5 | The SS ignores UE’s RACH attempts to let PREAMBLE_TRANSMISSION_COUNTER > 2*preambleTransMax-Msg1-Repetition-r18 | - | - | - | - |
| 6 | Check: Does the UE transmit a PRACH preamble with preamble index k on 8 consecutive ROs starting from the RO #n in radio frame #m, where - 24 ≤ k < 32 for FR1, 12 ≤ k < 16 for FR2; - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2. | --> | PRACH preamble | 4 | P |
| 7 | The SS transmits Random Access Response with RAPID corresponding to preamble index k and RA-RNTI associated with the RO #n+7 in radio frame #m, where k,n and m are same as step 6 | <-- | Random Access Response | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message? | --> | MAC PDU (RRCSetupRequest) | 5 | P |
| 9-14 | Steps 3 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 15 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 16 | The SS changes NR Cell 1 power level according to the row "T1" in Table 7.1.1.1.20.3.2-1/2 | - | - | - | - |
| 17 | Wait 10s. | - | - | - | - |
| 18 | The SS transmits a Paging message including a matched UE identity. | <-- | NR RRC: Paging | - | - |
| 19 | Check: Does the UE transmit PRACH preambles with preamble index k without any repetition? where - 0 ≤ k < 8 for FR1, 0 ≤ k < 4 for FR2; | --> | PRACH preamble | 1 | P |
| 20 | The SS ignores UE’s RACH attempts to let PREAMBLE_TRANSMISSION_COUNTER > preambleTransMax | - | - | - | - |
| 21 | Check: Does the UE transmit PRACH preambles with any repetition? | --> | PRACH preamble | 2 | F |
| 22 | Power off the UE. | - | - | - | - |

7.1.1.1.20.3.3	Specific message contents

Table 7.1.1.1.20.3.3-1: SIB1 (Preamble, Cell 1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |  |
| initialUplinkBWP |  | BWP-UplinkCommon | Table 7.1.1.1.20.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.20.3.3-2: BWP-UplinkCommon (Table 7.1.1.1.20.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.1.20.3.3-3 |  |
| } |  |  |  |
| additionalRACH-perPCI-ToAddModList-r18 | Not present |  |  |
| additionalRACH-perPCI-ToReleaseList-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum2-r18 | 69 | Actual threshold = 69 -156 = -87dBm |  |
| rsrp-ThresholdMsg1-RepetitionNum4-r18 | 71 | Actual threshold = 71 -156 = -85dBm |  |
| rsrp-ThresholdMsg1-RepetitionNum8-r18 | 73 | Actual threshold = 73 -156 = -83dBm |  |
| preambleTransMax-Msg1-Repetition-r18 | n4 |  |  |
| } |  |  |  |

Table 7.1.1.1.20.3.3-3: RACH-ConfigCommon (Table 7.1.1.1.20.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreambles-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 3 entries |  |  |
| FeatureCombinationPreambles-r17[1] | FeatureCombinationPreambles specified in TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP | entry 1 |  |
| FeatureCombinationPreambles-r17[2] | FeatureCombinationPreambles-rep4 | entry 2 Table 7.1.1.1.20.3.3-4 |  |
| FeatureCombinationPreambles-r17[3] | FeatureCombinationPreambles-rep8 | entry 3 Table 7.1.1.1.20.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.20.3.3-4: FeatureCombinationPreambles-rep4 (Table 7.1.1.1.20.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| startPreambleForThisPartition-r17 | 16 | PRACH preamble indice 16 - 23 belong to this group | FR1 |
|  | 8 | PRACH preamble indice 8 - 11 belong to this group | FR2 |
| msg1-RepetitionNum-r18 | n4 | For PRACH repetition number 4 |  |
| } |  |  |  |

Table 7.1.1.1.20.3.3-5: FeatureCombinationPreambles-rep8 (Table 7.1.1.1.20.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| startPreambleForThisPartition-r17 | 24 | PRACH preamble indice 24 – 31 belong to this group | FR1 |
|  | 12 | PRACH preamble indice 12 - 15 belong to this group | FR2 |
| msg1-RepetitionNum-r18 | n8 | For PRACH repetition number 8 |  |
| } |  |  |  |

7.1.1.1.21	Random access procedure / Msg1 repetition / Msg1-based SI request

7.1.1.1.21.1	Test Purpose (TP)

(1)

with { UE is in RRC_idle state and is configured with RACH resources for both legacy SI request and SI request with Msg1 repetition }

ensure that {

when { PL-RS RSRP results is higher than the threshold for Msg1 repetition }

then { UE performs legacy SI request procedure }

}

(2)

with { UE is in RRC_idle state and is configured with RACH resources for both legacy SI request and SI request with Msg1 repetition }

ensure that {

when { PL-RS RSRP results is lower than the threshold for Msg1 repetition }

then { UE performs SI request with Msg1 repetition of the highest applicable repetition number }

}

7.1.1.1.21.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.213 clause 8.1, 8.2, TS 38.321 clause 5.1.1b, 5.1.2, 5.1.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.213 clause 8.1]

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

…

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Within a time period, for set(s) of Npreamblerep valid PRACH occasions for a PRACH transmission with Npreamblerep preamble repetitions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any, is determined according to an ordering of valid PRACH occasions

-	first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	second, in increasing order of time resource indexes for time multiplexed PRACH occasions

where, for each frequency resource index for frequency multiplexed PRACH occasions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any,

-	is after msg1-RepetitionTimeOffsetROGroup consecutive valid PRACH occasions in time from the first valid PRACH occasion of the previous set, where each PRACH occasion is associated with same SS/PBCH block index(es) and each SS/PBCH block index is associated with same preambles, if msg1-RepetitionTimeOffsetROGroup is provided

-	is after the PRACH occasions for the previous set, if msg1-RepetitionTimeOffsetROGroup is not provided

…

[TS 38.213 clause 8.2]

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

…

[TS 38.321, clause 5.1.1b]

1>	else if Random Access resources for SI request have been provided for this Random Access procedure:

2>	if Random Access Resources associated with Msg1 repetition for SI request and Msg1 repetition number have been provided for this Random Access procedure:

…

3>	else:

4>	select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

[TS 38.321, clause 5.1.1e]

The MAC entity shall:

1>	if si-RequestResourcesRepetitionNum8 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8:

2>	criteria to apply Msg1 repetition for SI request is considered met and Msg1 repetition number applicable is 8.

1>	else if si-RequestResourcesRepetitionNum4 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4:

2>	criteria to apply Msg1 repetition for SI request is considered met and Msg1 repetition number applicable is 4.

1>	else si-RequestResourcesRepetitionNum2 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2:

2>	criteria to apply Msg1 repetition for SI request is considered met and Msg1 repetition number applicable is 2.

[TS 38.321, clause 5.1.3]

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

7.1.1.1.21.3	Test description

7.1.1.1.21.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-3 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR Cell 1 and NR Cell 11.

UE:

-	None

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

7.1.1.1.21.3.2	Test procedure sequence

Table 7.1.1.1.21.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T0" is applied for Preamble. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 7.1.1.1.21.3.2-3.

Table 7.1.1.1.21.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -90 | Off | The power level is such that SrxlevNR Cell1 > 0 |
|  | Qrxlevmin | dBm | -106 | - |  |
|  | Qoffset | dB | - | 12 NOTE 2 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -90 | -84 | The power level values are assigned to satisfy RNR Cell 1 > RNR Cell 11 |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Qoffset | dB | - | 12 NOTE 2 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -90 | -84 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 11 |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Qoffset | dB | - | - |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -90 | -84 | The power level values are assigned to satisfy RNR Cell 1 > RNR Cell 11 |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Qoffset | dB | -12 NOTE 3 | 12 NOTE 2 |  |
| NOTE 1: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. NOTE 2: q-OffsetCell for Cell 11 is configured in SIB3 of Cell 1. NOTE 3: q-OffsetCell for Cell 1 is configured in SIB3 of Cell 11. |  |  |  |  |  |

Table 7.1.1.1.21.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -92 | Off | The power level is such that SrxlevNR Cell1 > 0 |
|  | Qrxlevmin | dBm | 2* ROUND((-110+Delta(NRfs))/2) | - |  |
|  | Qoffset | dB | - | 20 NOTE 2 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -92 | -82 | The power level values are assigned to satisfy RNR Cell 1 > RNR Cell 11 |
|  | Qrxlevmin | dBm | 2* ROUND((-110+Delta(NRfs))/2) | 2* ROUND((-110+Delta(NRfs))/2) |  |
|  | Qoffset | dB | - | 20 NOTE 2 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -92 | -82 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 11 |
|  | Qrxlevmin | dBm | 2* ROUND((-110+Delta(NRfs))/2) | 2* ROUND((-110+Delta(NRfs))/2) |  |
|  | Qoffset | dB | - | - |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -92 | -82 | The power level values are assigned to satisfy RNR Cell 1 > RNR Cell 11 |
|  | Qrxlevmin | dBm | 2* ROUND((-110+Delta(NRfs))/2) | 2* ROUND((-110+Delta(NRfs))/2) |  |
|  | Qoffset | dB | -20 NOTE 3 | 20 NOTE 2 |  |
| NOTE 1: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.2. NOTE 2: q-OffsetCell for Cell 11 is configured in SIB3 of Cell 1. NOTE 3: q-OffsetCell for Cell 1 is configured in SIB3 of Cell 11. |  |  |  |  |  |

Table 7.1.1.1.21.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes SS/PBCH EPRE level of NR Cell 11 according to the row "T1" in Table 7.1.1.1.21.3.2-1/2. | - | - | - | - |
| 2 | Wait 60s to ensure UE detects NR Cell 11. | - | - | - | - |
| 3 | SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1 of Short Message field is set to 1 to indicate the SI modification. | <-- | Short Message | - | - |
| 4 | The valueTag for SIB3 in the SIB1 message is increased and si-BroadcastStatus for SIB3 is set to ‘notBroadcasted’ and SS stops broadcasting SIB3. | - | - | - | - |
| 5 | Check: Does the UE transmit a preamble on PRACH using the preamble indicated by ra-PreambleStartIndex-r18 defined in SI-RequestConfigRepetition in SIB1 on 2 consecutive ROs starting from the RO #n in radio frame #m? Where - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2 | --> | PRACH Preamble | 2 | P |
| 6 | The SS transmits a RAR message addressed to UE RA-RNTI including a MAC subPDU with a matching RAPID only. (Note 1) | <-- | Random Access Response | - | - |
| 7 | The SS changes SS/PBCH EPRE level of NR Cell 1 and NR Cell 11, as well as the parameter ‘Qoffset’ in SIB3 of NR Cell 1 and NR Cell 11 according to the row "T2" in Table 7.1.1.1.21.3.2-1/2 and starts broadcasting SIB3. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 11 belonging to a new TA? | - | - | 2 | P |
| 9 | SS transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1 of Short Message field is set to 1 to indicate the SI modification. | <-- | Short Message | - | - |
| 10 | The valueTag for SIB3 in the SIB1 message is increased and si-BroadcastStatus for SIB3 is set to ‘notBroadcasted’ and SS stops broadcasting SIB3. | - | - | - | - |
| 11 | Check: Does the UE transmit a preamble on PRACH using the preamble indicated by ra-PreambleStartIndex defined in SI-RequestConfig in SIB1? | --> | PRACH Preamble | 1 | P |
| 12 | The SS transmits a RAR message addressed to UE RA-RNTI including a MAC subPDU with a matching RAPID only. (Note 1) | <-- | Random Access Response | - | - |
| 13 | The SS changes SS/PBCH EPRE level of NR Cell 1 and NR Cell 11, as well as the parameter ‘Qoffset’ in SIB3 of NR Cell 1 and NR Cell 11 according to the row "T3" in Table 7.1.1.1.21.3.2-1/2 and starts broadcasting SIB3. | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 1 belonging to a new TA? | - | - | 1 | P |
| Note 1: The UE will indicate the reception of an acknowledgement for SI request to upper layers after UE receives the RAR message including a MAC subPDU with a matching RAPID only, according to TS 38.321 [18] clause 5.1.4. |  |  |  |  |  |

7.1.1.1.21.3.3	Specific message contents

Table 7.1.1.1.21.3.3-1: SIB1 (Preamble, test procedure, step 4 and 10, Cell 1 and Cell 11)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 with condition SI_Msg1Repetition |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| si-SchedulingInfo |  | SI-SchedulingInfo | Table 7.1.1.1.21.3.3-2 |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| totalNumberOfRA-Preambles |  | 56 | Last 8 preamble indice are left for SI request and other purpose. |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { |  | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] |  | FeatureCombinationPreambles specified in TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| additionalRACH-perPCI-ToAddModList-r18 |  | Not present |  |  |
| additionalRACH-perPCI-ToReleaseList-r18 |  | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum2-r18 |  | 69 | Actual threshold = 69 - 156 = -87dBm |  |
| rsrp-ThresholdMsg1-RepetitionNum4-r18 |  | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum8-r18 |  | Not present |  |  |
| preambleTransMax-Msg1-Repetition-r18 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | SIB1-v1610-IEs |  |
| nonCriticalExtension SEQUENCE { |  |  | SIB1-v1630-IEs |  |
| nonCriticalExtension SEQUENCE { |  |  | SIB1-v1700-IEs |  |
| nonCriticalExtension SEQUENCE { |  |  | SIB1-v1740-IEs |  |
| nonCriticalExtension SEQUENCE { |  |  | SIB1-v1800-IEs |  |
| si-SchedulingInfo-v1800 |  | SI-SchedulingInfo-v1800 specified in TS 38.508-1 [4] Table 4.6.3-173AB |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.21.3.3-2: SI-SchedulingInfo (Table 7.1.1.1.21.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { | 2 entries |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus | broadcasting |  |  |
| si-Periodicity | rf32 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType2 |  |  |
| valueTag | 0 |  |  |
| areaScope | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SchedulingInfo[2] SEQUENCE { |  | entry 2 |  |
| si-BroadcastStatus | broadcasting |  | Preamble |
|  | notBroadcasting |  | Step 4, Step 10 |
| si-Periodicity | rf64 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType3 |  |  |
| valueTag | 0 |  | Preamble |
|  | 1 |  | Step 4 |
|  | 2 |  | Step 10 |
| areaScope | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| si-RequestConfig | SI-RequestConfig specified in 38.508-1 [4] Table 4.6.3-172A |  |  |
| } |  |  |  |

Table 7.1.1.1.21.3.3-3: Void

Table 7.1.1.1.21.3.3-4: SIB3 (Cell 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo { | 1 entry |  |  |
| IntraFreqNeighCellInfo[1] SEQUENCE { |  | entry 1 For NR Cell 11 |  |
| physCellId | 11 | PCI of NR Cell 11 specified in 38.508-1 [4] Table 4.4.2-2 |  |
| q-OffsetCell | dB12 |  | FR1 AND preamble |
|  | dB20 |  | FR2 AND preamble |
|  | dB0 |  | Step 7 |
| q-RxLevMinOffsetCell | Not present |  |  |
| q-RxLevMinOffsetCellSUL | Not present |  |  |
| q-QualMinOffsetCell | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.21.3.3-5: SIB3 (Cell 11)

| Derivation Path: TS 38.508-1 [4], Table 6.3.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo { | 1 entry |  |  |
| IntraFreqNeighCellInfo[1] SEQUENCE { |  | entry 1 For NR Cell 1 |  |
| physCellId | 0 | PCI of NR Cell 1 specified in 38.508-1 [4] Table 4.4.2-2 |  |
| q-OffsetCell | dB-12 |  | FR1 AND step 13 |
|  | dB-20 |  | FR2 AND step 13 |
|  | dB0 |  | Preamble |
| q-RxLevMinOffsetCell | Not present |  |  |
| q-RxLevMinOffsetCellSUL | Not present |  |  |
| q-QualMinOffsetCell | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.1.22	Random access procedure / Msg1 repetition / CE-only BWP

7.1.1.1.22.1	Test Purpose (TP)

(1)

with { UE is in RRC_connected state and is configured with only RACH resources for Msg1 repetition in active BWP }

ensure that {

when { UE has data available for transmission but does not receive any UL grant }

then { UE initiates a Random Access procedure with Msg1 repetition of configured repetition number }

}

7.1.1.1.22.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.213 clause 8.1, 8.2, TS 38.321 clause 5.1.1b, 5.1.2, 5.1.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.213 clause 8.1]

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

…

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Within a time period, for set(s) of Npreamblerep valid PRACH occasions for a PRACH transmission with Npreamblerep preamble repetitions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any, is determined according to an ordering of valid PRACH occasions

-	first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	second, in increasing order of time resource indexes for time multiplexed PRACH occasions

where, for each frequency resource index for frequency multiplexed PRACH occasions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any,

-	is after msg1-RepetitionTimeOffsetROGroup consecutive valid PRACH occasions in time from the first valid PRACH occasion of the previous set, where each PRACH occasion is associated with same SS/PBCH block index(es) and each SS/PBCH block index is associated with same preambles, if msg1-RepetitionTimeOffsetROGroup is provided

-	is after the PRACH occasions for the previous set, if msg1-RepetitionTimeOffsetROGroup is not provided

…

[TS 38.213 clause 8.2]

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

…

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

…

1>	else if the BWP selected for Random Access procedure is configured only with Random Access resources with msg1-Repetitions set to true:

2>	assume Msg1 repetition is applicable for the current Random Access procedure;

…

2>	else (none of rsrp-ThresholdMsg1-RepetitionNumX is configured):

3>	assume Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number that configured for this BWP.

[TS 38.321, clause 5.1.3]

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

7.1.1.1.22.3	Test description

7.1.1.1.22.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that Short_DCI condition is applied in NR Serving cell configuration.

7.1.1.1.22.3.2	Test procedure sequence

Table 7.1.1.1.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to reconfigure active BWP as a CE-only BWP. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a MAC PDU containing a PDCP SDU | <-- | MAC PDU | - | - |
| 4 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 5 | Check: Does the UE transmit PRACH preambles with preamble index k on 2 consecutive ROs starting from the RO #n in radio frame #m, where - 8 ≤ k < 16 for FR1, 4 ≤ k < 8 for FR2; - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2. | - | - | 1 | P |
| 6 | The SS transmits Random Access Response with RAPID corresponding to preamble index k and RA-RNTI associated with the RO #n+1 in radio frame #m, where k,n and m are same as step 5 | <-- | Random Access Response | - | - |

7.1.1.1.22.3.3	Specific message contents

Table 7.1.1.1.22.3.3-1: RRCReconfiguration (Table 7.1.1.1.22.3.2-1, step 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated |  | ServingCellConfig | Table 7.1.1.1.22.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.22.3.3-2: ServingCellConfig (Table 7.1.1.1.22.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  |  |
| BWP-Uplink[1] SEQUENCE { |  | entry 1 |  |
| bwp-Common SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| release | null | No legacy RACH resource is configured on BWP#1 |  |
| } |  |  |  |
| additionalRACH-ConfigList-r17 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17 { | 1 entry |  |  |
| AdditionalRACH-Config-r17[1] SEQUENCE { |  | entry 1 |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon |  |  |
| msgA-ConfigCommon-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalRACH-perPCI-ToAddModList-r18 | Not present |  |  |
| additionalRACH-perPCI-ToReleaseList-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum2-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum4-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum8-r18 | Not present |  |  |
| preambleTransMax-Msg1-Repetition-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.22.3.3-3: RACH-ConfigCommon (Table 7.1.1.1.22.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreambles-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17{ | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] | FeatureCombinationPreambles | entry 1 Table 7.1.1.1.22.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.22.3.3-4: FeatureCombinationPreambles

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| startPreambleForThisPartition-r17 | 0 |  |  |
| } |  |  |  |

<End of Change 1>