3GPP TSG-RAN5 Meeting #105	R5-246896

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4757 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 8.2.5.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.2.5.7.2 needed to be updated according to TS 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | SCGFailureInformation message send on SRB1 and need check IE failureType according to TS 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

8.2.5.7.2	Radio link failure / LBT Failure / NR-DC

8.2.5.7.2.1	Test Purpose (TP)

(1)

With { UE in NR RRC_CONNECTED state with NR-DC, and SharedSpectrum }

ensure that {

when { UE receives an IP Packet to loopback on SCG DRB and UE encounters consistent uplink LBT failures problem }

then { UE initiates the NR SCG failure information procedure to report SCGFailureInformation with failure type scg-lbtFailure }

}

8.2.5.7.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321 clause 5.21.1, TS 38.331 clauses 5.3.10.3 , 5.7.3.2 and 5.7.3.5.

[TS 38.321, clause 5.21.1]

The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed by lower layers if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication is sent to the MAC entity from lower layers.

[TS 38.331, clause 5.7.3.2]

A UE initiates the procedure to report SCG failures when neither MCG nor SCG transmission is suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with clause 5.3.10.3;

[TS 38.331, clause 5.7.3.5]

The UE shall set the contents of the SCGFailureInformation message as follows:

…

1>	else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures:

2>	set the failureType as other and set the failureType-v1610 as scg-lbtFailure;

…

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	upon consistent uplink LBT failure indication from SCG MAC:

...

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

...

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

8.2.5.7.2.3	Test description

8.2.5.7.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 10 is the PSCell.

-	NR Cell 10 is configured to operate in shared spectrum.

None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) with Bearers (MCG(s) and SCG), Test Mode (On) and Test Loop Function (On) and mode B according to TS 38.508-1 [4], Table 4.5.1-1.

8.2.5.7.2.3.2	Test procedure sequence

Table 8.2.5.7.2.3.2-0 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble.

Table 8.2.5.7.2.3.2-0: Cell configuration power level changes over time for conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -75 |
|  | Noc | dBm/SCS | - | -82 |

Table 8.2.5.7.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP Packet on SCG DRB | - | - | - | - |
| 1A | The SS adjusts the SS/PBCH EPRE and Noc levels according to row "T1" in table 8.2.5.7.2.3.2-0. | - | - | - | - |
| 2 | UE detects consistent LBT failures on NR PSCell while attempting to loop back the data received at step 1. | - | - | - | - |
| 3 | Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformation message with failureType-v1610 set to scg-lbtFailure-r16? | --> | SCGFailureInformation | 1 | P |

8.2.5.7.2.3.3	Specific message contents

Table 8.2.5.7.2.3.3-1: SCGFailureInformation (step 3, Table 8.2.5.7.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-24A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCGFailureInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| scgFailureInformation SEQUENCE { |  |  |  |
| failureReportSCG SEQUENCE { |  |  |  |
| failureType | other-r16 |  |  |
| measResultFreqList | Not checked |  |  |
| measResultSCG-Failure | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| failureType-v1610 | scg-lbtFailure-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>