3GPP TSG-RAN5 Meeting #105	R5-246917

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1859 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.3L.3.x for inter-band EN-DC with TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases 6.3L.3.x Output power dynamics for inter-band EN-DC with TxD are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test cases 6.3L.3.x for inter-band EN-DC with TxD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3L.3.1(new), 6.3L.3.2(new), 6.3L.3.3(new), 6.3L.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3L	Output power dynamics for DC with Tx Diversity

6.3L.1

6.3L.2

6.3L.3	Output power dynamics for inter-band EN-DC with Tx Diversity

6.3L.3.1	Minimum output power for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.1.1	Test purpose

Same test purpose as in clause 6.3G.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.1.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.1 Minimum output power for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test descriptions as in clause 6.3G.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3G.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.1.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.1.5	Test requirements

Same test requirement as in clause 6.3G.1.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.2	Transmit OFF Power for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.2.1	Test purpose

Same test purpose as in clause 6.3G.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.2.4	Test description

This test is covered by clause 6.3L.3.3 Transmit ON/OFF time mask for inter-band EN-DC with Tx Diversity.

6.3L.3.2.5	Test requirements

Same test requirement as in clause 6.3G.2.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3	Transmit ON/OFF time mask for Tx Diversity

6.3L.3.3.1	General ON/OFF time mask for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.3.1.1	Test purpose

Same test purpose as in clause 6.3G.3.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.3.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.3.1.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.3.1 General ON/OFF time mask for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test descriptions as in clause 6.3G.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3G.3.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.3.1.4 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3G.3.1.4 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.3.1.5	Test requirements

Same test requirement as in clause 6.3G.3.1.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3.2	PRACH time mask for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.3.2.1	Test purpose

Same test purpose as in clause 6.3G.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.3.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.3.2.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.3.2 PRACH time mask for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test descriptions as in clause 6.3G.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3G.3.2.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.3.2.4 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3G.3.2.4 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.3.2.5	Test requirements

Same test requirement as in clause 6.3G.3.2.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3.3	SRS time mask for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.3.3.1	Test purpose

Same test purpose as in clause 6.3G.3.3.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.3.3.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.3.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.3.3.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.3.3 SRS time mask for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test descriptions as in clause 6.3G.3.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3G.3.3.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.3.3.4 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3G.3.3.4 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.3.3.5	Test requirements

Same test requirement as in clause 6.3G.3.3.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4	Power control for inter-band EN-DC with Tx Diversity

6.3L.3.4.1	Absolute power tolerance for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.4.1.1	Test purpose

Same test purpose as in clause 6.3G.4.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4.1.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.4.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.4.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.4.1.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.4.1 Absolute power tolerance for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.3G.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3G.4.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.4.1.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.4.1.5	Test Requirement

Same test requirement as in clause 6.3G.4.1.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4.2	Relative power tolerance for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.4.2.1	Test purpose

Same test purpose as in clause 6.3G.4.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4.2.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.4.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.4.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.4.2.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.4.2 Relative power tolerance for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.3G.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3G.4.2.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.4.2.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.4.2.5	Test Requirement

Same test requirement as in clause 6.3G.4.2.5 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4.3	Aggregate power tolerance for inter-band EN-DC with Tx Diversity within FR1

6.3L.3.4.3.1	Test purpose

Same test purpose as in clause 6.3G.4.3.1 in TS 38.521-1 [8] for the NR carrier.

6.3L.3.4.3.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC with Tx Diversity.

6.3L.3.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.4.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3L.3.

6.3L.3.4.3.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.3G.4.3 Aggregate power tolerance for Tx Diversity in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.3G.4.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3G.4.3.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3G.4.3.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3L.3.4.3.5	Test Requirement

Same test requirement as in clause 6.3G.4.3.5 in TS 38.521-1 [8] for the NR carrier.

<End of Changes>