3GPP TSG-RAN5 Meeting #105	R5-247028

Orlando, United States, Nov 18-22, 2024

Title:	Discussion on updating NTN demodulation test case

Source:	Huawei, HiSilicon

Document for:	Endorsement

# Background

In TS 38.306, UE capability “ntn-ScenarioSupport-r17” is defined as following:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 Indicates whether the UE supports the NTN features in GSO scenario or NGSO scenario. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports the NTN features for both GSO and NGSO scenarios, and also supports mobility between GSO and NGSO scenarios. | UE | No | No | No |

In clause 8.2.1.1.2 of TS 38.101-5 (clause 8.2.1.1.2 of TS 38.521-5 at the same time), the applicability of NTN demodulation requirements is defined as following:

##### 8.2.1.1.2 Applicability of requirements for optional UE features The performance requirements in Table 8.2.1.1.2-1 shall apply for UEs which support optional UE features only. Table 8.2.1.1.2-1: Requirements applicability for optional UE features UE feature/capability [TBD]; Test type; Test list; Applicability notes. NR NTN access (nonTerrestrialNetwork-r17); FR1 FDD; PDSCH; Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4). NR NTN scenario support (ntn-ScenarioSupport-r17); FR1 FDD; PDSCH; Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4); The requirements apply only when ntn-ScenarioSupport-r17 is “ngso” or is not configured.. Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17); FR1 FDD; PDSCH; Clause 8.2.1.2.2.1 (Test 1-3). Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17); FR1 FDD; PDSCH; Clause 8.2.1.2.2.1 (Test 1-4). Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1.

In RAN4 WF R4-2214386, RAN4 group have agreement that NTN demodulation requirements are defined in LEO-600, and the K_offset value is set as 8 slots.

Issue 2-1: PDSCH requirements for GEO and LEO - Proposals: to confirm the following tentative agreements: - Option 1: Define PDSCH Demod requirements for LEO-600. Do not define separate set of requirements for GEO - Agreements: - Agree with Option 1 Issue 2-3: K-offset value - Proposals: Only specify the k_offset values for LEO-600. Encourage TE vendors to provide the input on the feasibility of supporting a large k_offset value referring to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases. - Option 1: Provide the specific values of k_offset for HARQ disabled and HARQ process of 32 - Agreement: - K_offfset = [8 slots] applied for all HARQ configurations

# Discussion

In TS 38.521-5 test case 8.2.1.2.2.1.1_1, the test procedure Step 2 are defined as below:

... 2 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508[12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1. …

Based on above mentioned RAN 4 agreements, following conclusions can be drawn:

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is configured as “ngso”, UE needs to test NTN demodulation requirements in TS 38.101-5, with LEO-600 scenario and the K_offset=8 slots.

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is not configured (it means UE support both “ngso” and “gso”), UE needs to test NTN demodulation requirements in TS 38.101-5, with LEO-600 scenario and the K_offset=8 slots.

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is configured as “gso”, UE does not need to test NTN demodulation requirements in TS 38.101-5.

However, based on the test procedure Step 2 in clause 8.2.1.2.2.1.1_1.3.2 of TS 38.521-5, we can obtain that:

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is configured as “ngso”, UE needs to test NTN demodulation requirements in TS 38.101-5, with LEO-1200 scenario and the K_offset=14 slots.

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is not configured (it means UE support both “ngso” and “gso”), UE needs to test NTN demodulation requirements in TS 38.101-5, with GSO scenario and the K_offset=258 slots.

- if UE support “nonTerrestrialNetwork-r17” and “ntn-ScenarioSupport-r17” is configured as “gso”, UE needs to test NTN demodulation requirements in TS 38.101-5, with GSO scenario and the K_offset=258 slots.

- Observation 1:	The test procedure in RAN5 test case 8.2.1.2.2.1.1_1 has conflict with RAN4 agreements.

In this test case, the SIB19 information in TS 38.508-1 clause 5.6.3.1 is applied which is pasted as below. As per the RAN4 agreed WF, the K_offset is supposed to be set to “8” for NGSO scenario and no need to test GSO scenario.

Table 5.6.3.1-1: SIB19 for RF tests Derivation Path: Table 4.6.2-18C. Information Element; Value/remark; Comment; Condition. SIB19-r17 ::= SEQUENCE {. ntn-Config-r17::= SEQUENCE {. epochTime-r17; Not present. ntn-UlSyncValidityDuration-r17; s240. cellSpecificKoffset-r17; 258; GSO. 14; NGSO. kmac-r17; Not present. ta-Info-r17 SEQUENCE {. ta-Common-r17; 0. ta-CommonDrift-r17; 0. ta-CommonDriftVariant-r17; 0. }. ntn-PolarizationDL-r17; Not present. ntn-PolarizationUL-r17; Not present. ephemerisInfo-r17; See cl 5.6.2. }. ta-Report-r17; Not present. }. }.

Observation 2: the value of cellSpecificKoffset-r17 in SIB19 should be updated specifically in RAN5 test case 8.2.1.2.2.1.1_1.

Based on Observations 1 and 2:

- Proposal 1:	Update RAN5 test case 8.2.1.2.2.1.1_1 in TS 38.521-5 to align with RAN4 agreement.

# Conclusion

- Observation 1:	The test procedure in RAN5 test case 8.2.1.2.2.1.1_1 has conflict with RAN4 agreements.

Observation 2: the value of cellSpecificKoffset-r17 in SIB19 should be updated specifically in RAN5 test case 8.2.1.2.2.1.1_1.

- Proposal 1:	Update RAN5 test case 8.2.1.2.2.1.1_1 in TS 38.521-5 to align with RAN4 agreement.

Proposal 1 is implemented in CR R5-247029 (CR 0080)