3GPP TSG-RAN5 Meeting #105	R5-247048

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0740 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for Further measurement gap enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Further measurement gap enhancements test cases, new PICS items are needed according to TS 38.306 and TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PICS was added for Further measurement gap enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of PICS statement for Further measurement gap enhancements TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| 2 | Support two independent measurement gap configurations for FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_independentGapConfig | No |  |  |
| … |  |  |  |  |  |  |  |
| 97 | Support SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreqMeasGap_r18 | No |  |  |
| xx1 | Support reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-16 | pc_nr_NeedForGap_Reporting_r16 | No |  |  |
| xx2 | Support reporting the interruption requirement information for SSB based measurement towards NR target without gap in the UE response to a network configuration RRC message | 38.306, 4.2.9 | Rel-18 | pc_nr_NeedForInterruptionReport_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGap-Reporting-r16 (pc_nr_NeedForGap_Reporting_r16). |
| xx3 | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS and PDCCH or PDSCH reception from the serving cell with a different numerology | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasCRS_InsideBWP_EUTRA_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18) or eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18). |
| xx4 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsNCSG_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGapNCSG-Reporting-r17 (pc_nr_NeedForGapNCSG_Reporting_r17) and concurrentMeasGap-r17. |
| xx5 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsPreMG_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGap-r17 and one of preconfiguredNW-ControlledMeasGap-r17 (pc_preconfiguredNW_ControlledMeasGap_r17) and preconfiguredUE-AutonomousMeasGap-r17 (pc_preconfiguredUE_AutonomousMeasGap_r17). |
| (()xx6 | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG | 38.306, 4.2.9 | Rel-18 | pc_dynamicCollision_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGapsPreMG-r18 (pc_concurrentMeasGapsPreMG_r18). |
| xx7 | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity | 38.306, 4.2.9 | Rel-18 | pc_eutra_MeasEMW_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18) or eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18). |
| xx8 | Support inter-RAT EUTRAN measurements without gap when CRS is completely contained within UE's active DL BWP | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementInsideBWP_r18 | No |  |  |
| xx9 | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementOutsideBWP_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NeedForGapNCSG-Reporting-r17 (pc_eutra_NeedForGapNCSG_Reporting_r17). |

[End of change]