Title: 	Discussion on test point selection for NR Occupied Bandwidth for NR V2X in FR1

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR Occupied Bandwidth for NR V2X in FR1.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

The test environment for basic NR FR1 test cases is selected as Normal. The same environment selection can be adopted for V2X.

Proposal 1: Define test environment for Occupied bandwidth for V2X as Normal.

  - Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest for Occupied bandwidth for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 2: Select Lowest SCS supported for UE for Occupied bandwidth for NR V2X in FR1.

  - Test Frequencies

Test frequencies are selected as Mid range for Occupied bandwidth for basic test cases in FR1 Type 1 frequency bands, i.e. bands with frequency range less than 200MHz. The same approach can be followed for NR V2X test cases on bands n38 and n47.

Proposal 3: Test Mid range frequencies for Occupied bandwidth for NR V2X in FR1.

  - Test Channel Bandwidth

All the channel bandwidths defined in a band are tested for Occupied channel bandwidth for basic test cases in FR1. The same approach can be followed for NR V2X test cases. All the channel bandwidths applicable to NR V2X need to be tested, i.e. 10MHz, 20MHz, 30MHz and 40MHz.

Proposal 4: Test 10MHz, 20MHz, 30MHz and 40MHz as test channel bandwidth for Occupied bandwidth for NR V2X in FR1.

  - Uplink configuration

The uplink configuration is selected as maximum transmission bandwidth configuration and CP-OFDM waveform to get the maximum spectrum utilization. The lowest modulation scheme is selected. The same approach can be followed for NR V2X test cases.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform and QPSK modulation to test Occupied bandwidth for NR V2X.

  - Exceptions for OBW UL-MIMO test

Extracted from Clause 6.5E.1 [6]

For NR V2X UE with two transmit antenna connectors, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.5.1-1. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

Observation 1: Same Occupied bandwidth requirements as specified for single transmission apply for UEs supporting SL-MIMO.

Proposal 7: Adopt the same test configuration of single carrier test case for SL-MIMO measurement in V2X FR1.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Occupied Bandwidth test case definition:

Proposal 1: Define test environment for Occupied bandwidth for V2X as Normal.

Proposal 2: Select Lowest SCS supported for UE for Occupied bandwidth for NR V2X in FR1.

Proposal 3: Test Mid range frequencies for Occupied bandwidth for NR V2X in FR1.

Proposal 4: Test 10MHz, 20MHz, 30MHz and 40MHz as test channel bandwidth for Occupied bandwidth for NR V2X in FR1.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform and QPSK modulation to test Occupied bandwidth for NR V2X.

Proposal 7: Adopt the same test configuration of single carrier test case for SL-MIMO measurement in V2X FR1.

References

[1] R5-180065 , “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN 5 Ad hoc, Munich, Jan. 2018.

[3] R4-1800402, “38.101-1 Implementation of agreed CRs after NR-AH-1801”, Qualcomm Incorporated, RAN4 NR-AH#1801, San Diego, CA, USA, Jan 2018.

[4] TR 38.817-01 v0.3.0, “General aspects for UE RF for NR”.

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] 3GPP TS 38.101-1 v16.20.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)