3GPP TSG-RAN5 Meeting #105	R5-247071

# Orlando, USA, 18 Nov – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3098 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.5E.4 NR V2X Transmit Intermodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X Transmit Intermodulation test cases are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completing the test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.4.0 (new), 6.5E.4.1 (new), 6.5E.4.1D (new), 6.5E.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0956 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5E.3.3.1.5	Test requirement

6.5E.3.3.1.5.1	Message contents exceptions (network signalling value "NS_33")

The transmitted spurious emission power derived in step 3 of subtest 1, step 4 of subtest 2 and step 3 of subtest 3 should be converted to EIRP value depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. The EIPR value should not exceed test requirements in Table 6.5E.3.3.1.5.1-1.

Table 6.5E.3.3.1.5.1-1: Test requirements for "NS_33"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 | - | 5950 | -30 | 1 |
| Frequency range | 5815 | - | 5855 | -30 | 1 |

6.5E.4	Transmit intermodulation for V2X

6.5E.4.0	Minimum conformance requirements

6.5E.4.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.4.3 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.3.

6.5E.4.0.2	Transmit intermodulation for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.5.4.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.4.0.1 shall apply for the sidelink in licensed band or Band n47.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.4

6.5E.4.1	Transmit intermodulation for V2X / non-concurrent operation

6.5E.4.1.1	Test purpose

Same test purpose as in 6.5.4.1.

6.5E.4.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

6.5E.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.4.0.

6.5E.4.1.4	Test description

6.5E.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2E.1-1 and table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.5E.4.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.4.1.4.1-1: Test Configuration Table for Transmit Intermodulation

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | QPSK |  | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.4.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.5E.4.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.5E.4.1.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the rectangular filtered mean power of the UE. The guard symbols need to be excluded for measuring the wanted signal and the intermodulation product.

3.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5E.4.1.5-1.

4.	Set the interference CW signal level according to table 6.5E.4.1.5-1.

5.	Search the intermodulation product signals below and above the UL carrier frequency, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 2.

6.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5E.4.1.5-1.

7.	Search the intermodulation product signals below and above the UL carrier frequency, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 2.

8.	Repeat the measurement using the second offset in table 6.5E.4.1.5-1.

6.5E.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10.

6.5E.4.1.5	Test requirement

The ratio derived in step 5 and 7, shall not exceed the described value in table 6.5E.4.1.5-1.

Table 6.5E.4.1.5-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.5-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |
| Note 1: The test requirements do not apply when the interfering signal overlaps with the channel bandwidth of the downlink signal. |  |  |

6.5E.4.1D	Transmit intermodulation for V2X / non-concurrent operation / SL-MIMO

6.5E.4.1D.1	Test purpose

Same test purpose as in 6.5E.4.1.

6.5E.4.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.4.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.4.0.

6.5E.4.1D.4	Test description

Same test description as in 6.5E.4.1.4 with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

For test procedures:

The steps 2~8 are excuted for each transmit antenna connector respectively.

6.5E.4.1D.5	Test requirement

The ratio derived in step 5 and 7 on each transmit antenna connector, shall not exceed the described value in table 6.5E.4.1D.5-1.

Table 6.5E.4.1D.5-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.5-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |
| Note 1: The test requirements do not apply when the interfering signal overlaps with the channel bandwidth of the downlink signal. |  |  |

6.5E.4.2	Transmit intermodulation for V2X con-current operation

6.5E.4.2.1	Test purpose

Same test purpose as in 6.5E.4.1.1 for concurrent operation.

6.5E.4.2.2	Test applicability

No test case details are specified. The minimum output power requirements for inter-band concurrent operation and intra-band concurrent operation apply and are tested in clauses 6.5.4 for NR Uu operation and 6.5E.4.1 for NR sidelink option.

6.5F	Output RF spectrum emissions for shared spectrum channel access

6.5F.1	Occupied bandwidth for shared spectrum channel access

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU and TT for > 5.925 GHz

- Message exception for NR-U

6.5F.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits.

< End of Changes >