Title: 	Discussion on test point selection for NR Transmit Intermodulation for NR V2X in FR1

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR Transmit Intermodulation for NR V2X concurrent operation in FR1.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

The test environment for basic NR FR1 test cases is selected as Normal. The same environment selection can be adopted for V2X.

Proposal 1: Define test environment for Transmit Intermodulation for V2X as Normal.

  - Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest and Highest for Transmit Intermodulation for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 2: Select Lowest and Highest SCS supported for UE for Transmit Intermodulation for V2X in FR1.

  - Test Frequencies

Test frequencies are selected as Low and High range for Transmit Intermodulation for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 3: Test Low and High range frequencies for Transmit Intermodulation for NR V2X in FR1.

  - Test Channel Bandwidth

Test channel bandwidth are selected as Mid and Highest for Transmit Intermodulation for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 4: Test Mid and Highest channel bandwidth for Transmit Intermodulation for NR V2X in FR1.

  - Uplink configuration

The uplink configuration is selected as no MPR applies. The same approach can be followed for NR V2X test cases.

Proposal 5: Select uplink configuration with the minimum MPR value to test Transmit Intermodulation for NR V2X.

  - Exceptions for Spurious emission UL-MIMO test

Extracted from Clause 6.5E.4 [6]

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

Observation 1: Same requirements as specified for single transmission apply for UEs supporting SL-MIMO.

Proposal 6: Adopt the same test configuration of single carrier test case for SL-MIMO measurement in V2X FR1.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Occupied Bandwidth test case definition:

Proposal 1: Define test environment for Transmit Intermodulation for V2X as Normal.

Proposal 2: Select Lowest and Highest SCS supported for UE for Transmit Intermodulation for V2X in FR1.

Proposal 3: Test Low and High range frequencies for Transmit Intermodulation for NR V2X in FR1.

Proposal 4: Test Mid and Highest channel bandwidth for Transmit Intermodulation for NR V2X in FR1.

Proposal 5: Select uplink configuration with the minimum MPR value to test Transmit Intermodulation for NR V2X.

Proposal 6: Adopt the same test configuration of single carrier test case for SL-MIMO measurement in V2X FR1.

References

[1] R5-180065 , “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN 5 Ad hoc, Munich, Jan. 2018.

[3] R4-1800402, “38.101-1 Implementation of agreed CRs after NR-AH-1801”, Qualcomm Incorporated, RAN4 NR-AH#1801, San Diego, CA, USA, Jan 2018.

[4] TR 38.817-01 v0.3.0, “General aspects for UE RF for NR”.

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] 3GPP TS 38.101-1 v16.20.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)