3GPP TSG-RAN5 Meeting #105	R5-247236

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0748 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for CSI reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no PICS for spatial or power domain CSI reporting functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PICS for spatial or power domain CSI reporting functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There won’t be PICS for spatial or power domain CSI reporting functionality and not testcases to verify functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0536 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CAPNOK11 and CAPNOK12 are used correspondingly in CNOK11 and CNOK12 in R5-247235. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------- Start of first change ---------------------------------------------------------

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-RedCap and non-eRedCap UEs and optional for RedCap and eRedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | N/A | Yes | Yes | pdsch-MappingTypeA is purely mandatory |
| 6 | Support receiving PDSCH using PDSCH mapping type B | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB | Yes |  |  |
| 7 | Support resource allocation Type 0 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH | No |  |  |
| 8 | Support scaling factor 0.75 is applied to the band in the max data rate calculation | 38.306, 4.2.7 | Rel-15 | pc_scalingFactor0dot75 |  |  |  |
| 9 | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell | 38.306, 4.2.7.10 | Rel-15 | pc_csi_RS_CFRA_ForHO | No |  |  |
| 10 | Support Type 1 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType1 | No |  |  |
| 11 | Support Type 2 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType2 | No |  |  |
| 12 | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_RepetitionMultiSlots | No |  |  |
| 13 | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) | 38.306, 4.2.7.7 | Rel-15 | pc_dynamicSwitch_SUL | No |  |  |
| 14 | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_CB_PUSCH | No |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value different from "oneLayer" |
| 15 | Void |  |  |  |  |  |  |
| 16 | Support receiving PDSCH with interleaved VRB-to-PRB mapping | 38.306, 4.2.7.10 | Rel-15 | pc_interleavingVRB_ToPRB_PDSCH | Yes |  |  |
| 17 | Support dynamic EN-DC power sharing for at least one EN-DC band combination_FR1 only | 38.306, 4.2.7.9 | Rel-15 | pc_dynamicPowerSharingENDC | Yes |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |
| 18 | Supports up to 10 search spaces in a SCell per BWP | 38.306, 4.2.7.10 | Rel-15 | pc_maxNumberSearchSpaces | No |  |  |
| 19 | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation | 38.306, 4.2.7.10 | Rel-15 | pc_spatialBundlingHARQ_ACK | Yes |  |  |
| 20 | Support alternative additional DMRS position for co-existence with LTE CRS | 38.306, 4.2.7.5 | Rel-15 | pc_additionalDMRS_DL_Alt | No |  |  |
| 21 | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_RepetitionMultiSlots | Yes |  |  |
| 22 | Support beam correspondence without UL beam sweeping | 38.306, 4.2.7.2 | Rel-15 | pc_beamCorrespondenceWithoutUL_BeamSweeping | Yes |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |
| 22A | Support beam correspondence based on SSB | 38.306, 4.2.7.2 | Rel-16 | pc_beamCorrespondence_SSBbased | No |  |  |
| 22B | Support beam correspondence based on CSI-RS | 38.306, 4.2.7.2 | Rel-16 | pc_beamCorrespondence_CSI_RSbased | No |  |  |
| 23 | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_eightLayers | CY |  | Set to false if Table A.4.3.2-1/23A or 23B set to true. |
| 23A | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 4 Layers. | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_fourLayers | CY |  | Set to false if Table A.4.3.2-1/23 or 23B set to true. |
| 23B | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 2 Layers. | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_twoLayers | CY |  | Set to false if Table A.4.3.2-1/23 or 23A set to true. |
| 24 | Supports DCI and timer based active BWP switching delay type1 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type1 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 24A | Supports DCI and timer based active BWP switching delay type2 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type2 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 24B | Supports incremental delay for DCI and timer based active BWP switching type 1 on multiple CCs simultaneously | 38.306, 4.2.7.10 | Rel-16 | pc_bwp_SwitchingMultiCCs_type1_r16 | No |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |
| 24C | Supports incremental delay for DCI and timer based active BWP switching type 2 on multiple CCs simultaneously | 38.306, 4.2.7.10 | Rel-16 | pc_bwp_SwitchingMultiCCs_type2_r16 | No |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |
| 25A | Support modified MPR behaviour bit 0 | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour_bit0 | No |  | Applicable to FR2 bands n257, n258, n260 and n261 |
| 25 | Support modified MPR behaviour | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour | No |  |  |
| 26 | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PDSCH | No |  |  |
| 27 | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PUSCH | No |  |  |
| 28 | Support almost contiguous UL CP-OFDM transmissions in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR1 | No |  |  |
| 29 | Support almost contiguous UL CP-OFDM transmissions in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR2 | No |  |  |
| 30 | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset | 38.306, 4.2.7.10 | Rel-16 | pc_crossSlotScheduling | No |  |  |
| 31 | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR1 | Yes |  |  |
| 31a | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR2 | Yes |  |  |
| 32 | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency | 38.306, 4.2.7.6 | Rel-16 | pc_multiDCI_MultiTRP_r16 | No |  |  |
| 33 | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS | 38.306, 4.2.7.2 | Rel-15 | pc_rateMatchingLTE_CRS | Yes |  |  |
| 34 | Support of BWP operation without bandwidth restriction | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_WithoutRestriction | No |  |  |
| 35 | Support of receiving SCell dormancy indication on SPCell using DCI format 2_6 outside the active time | 38.306, 4.2.7.4 | Rel-16 | pc_scellDormancyOutsideActiveTime_r16 | No |  |  |
| 36 | Supports pi/2-BPSK modulation scheme for power boosting in FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_powerBoosting_pi2BPSK | No |  |  |
| 37 | Support of dynamic 1Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-16 | pc_ULTxSwitchingBandPair | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37a | Support of 35us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_35us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37b | Support of 140us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_140us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37c | Support of 210us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_210us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37d | Support of DL interruption during 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-18 | pc_uplinkTxSwitching_DL_Interruption_r16 | No |  | Not allowed to be setted to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 38 | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 | 38.306 4.2.7.2 | Rel-16 | pc_mpr_PowerBoost_FR2 | No |  |  |
| 39 | Supports the alternative 64QAM MCS table for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dl_64qam_mcs_tableAlt | No |  |  |
| 40 | Supports the CQI table with target BLER of 10^-5 | 38.306, 4.2.7.10 | Rel-15 | pc_cqi_tableAlt | No |  |  |
| 41 | Supports of single DCI based spatial division multiplexing scheme | 38.306, 4.2.7.5 | Rel-16 | pc_singledci_sdm | No |  |  |
| 42 | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR1_FDD | No |  | FR1 FDD bands |
| 42a | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR1_TDD | No |  | FR1 TDD bands |
| 42b | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR2 bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR2 | No |  | FR2 bands |
| 43 | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR1_FDD | No |  | FR1 FDD bands |
| 43a | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR1_TDD | No |  | FR1 TDD bands |
| 43b | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR2 bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR2 | No |  | FR2 bands |
| 44 | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 FDD bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR1_FDD | No |  | FR1 FDD bands |
| 44a | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 TDD bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR1_TDD | No |  | FR1 TDD bands |
| 44b | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR2 bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR2 | No |  | FR2 bands |
| 45 | Support PUSCH repetition type B | 38.306, 4.2.7.7 | Rel-16 | pc_pusch_RepetitionTypeB_r16 | No |  |  |
| 46 | Support of 2-Step RACH | 38.306, 4.2.7.10 | Rel-16 | pc_twoStepRACH_r16 | No |  |  |
| 47 | Void |  |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |  |
| 49 | Void |  |  |  |  |  |  |
| 50 | Void |  |  |  |  |  |  |
| 51 | Void |  |  |  |  |  |  |
| 52 | Support monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling | 38.306, 4.2.7.10 | Rel-16 | pc_dci_Format1_2And0_2_r16 | No |  |  |
| 53 | Support of multi-DCI based multi-TRP | 38.306, 4.2.7.6 | Rel-16 | pc_multi_dci_multi_trp | No |  |  |
| 54 | Support of single DCI based FDMSchemeA | 38.306, 4.2.7.2 | Rel-16 | pc_single_dci_fdmschemeA | No |  |  |
| 55 | Support of single-DCI based inter-slot TDM | 38.306, 4.2.7.2 | Rel-16 | pc_single_dci_interslot_tdm | No |  |  |
| 56 | Support of maximum number of TRS resource sets per CC which the UE can track simultaneously is at least 2 | 38.306, 4.2.7.2 | Rel-16 | pc_simultaneous_TRS | No |  |  |
| 57 | Support of low PAPR DMRS | 38.306 4.2.7.2 | Rel-16 | pc_lowPAPR_DMRS_pusch_precoding | No |  |  |
| 58 | Support of UL full power transmission mode of full power | 38.306, 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode_r16 | No |  |  |
| 59 | Support of UL full power transmission mode of fullpowerMode1 | 38.306, 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode1_r16 | No |  |  |
| 60 | Support of UL full power transmission mode of fullpowerMode2 | 38.306, 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode2_r16 | No |  |  |
| 61 | Support of PDSCH processing capability 2 | 38.306, 4.2.7.5 | Rel-16 | pc_pdsch_processing_cap2 | No |  |  |
| 62 | Support Pre-Emption Indication | 38.306, 4.2.7.10 | Rel-15 | pc_preEmptIndication_DL | No |  |  |
| 63 | Support of SSB based BFD | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberSSB_BFD | CY |  |  |
| 64 | Support of CSI-RS based BFD | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberCSI_RS_BFD | CY |  |  |
| 65 | Support of SSB and/or CSI-RS based Link Recovery | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberCSI_RS_SSB_CBD | CY |  |  |
| 66 | Support of type II codebook | 38.306, 4.2.7.2 | Rel-15 | pc_typeIICodebook | No |  |  |
| 67 | Support of Enhanced Type II codebook with at least 16 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-16 | pc_enhanced_typeII_codebook | No |  |  |
| 68 | Support of TDD NR UL transmission with a 7.5 kHz shift to the LTE raster | 38.101-1, 5.4.2 | Rel-15 | pc_frequencyShift7p5khz_TDD | No |  | Mandatory since Rel-16 |
| 69 | Support of FDD NR UL transmission with a 7.5 kHz shift to the LTE raster | 38.101-1, 5.4.2 | Rel-15 | pc_frequencyShift7p5khz_FDD | Yes |  |  |
| 70 | Void |  |  |  |  |  |  |
| 71 | Support of density of CSI-RS for Channel Measurement Report | 38.306, 4.2.7.2 | Rel-16 | pc_supportedCSI_RS_Density_CMR | No |  |  |
| 72 | Support of SSB/CSI-RS for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_ssb_csirs_SINR_measurement | No |  |  |
| 73 | Support of SSB as CMR with dedicated CSI-IM for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_ssbWithCSI_IM | No |  |  |
| 74 | Support of SSB as CMR with dedicated NZP IMR for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_ssbWithNZP_IMR | No |  |  |
| 75 | Support of CSI-RS as CMR with dedicated NZP IMR configured for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_csirsWithNZP_IMR | No |  |  |
| 76 | Support of CSI-RS as CMR without dedicated IMR configured for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_csi_RSWithoutIMR | No |  |  |
| 77 | Support of SCell beam failure recovery | 38.306, 4.2.7.2 | Rel-16 | pc_scellBFR | No |  |  |
| 78 | Support of the maximum number of activated TCI states per BWP per CC is other than n1, including control and data | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberActiveTCI_PerBWP | No |  |  |
| 79 | Support enhanced UL performance for the transient period | 38.306, 4.2.7.2 | Rel-16 | pc_enhancedUL_TransientPeriod_r16 | No |  |  |
| 80 | Supports the priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP | 38.306, 4.2.7.10 | Rel-16 | pc_dci_DL_PriorityIndicator_r16 | No |  |  |
| 81 | Supports the priority indicator field configured in DCI formats 0_1 and 0_2 in a BWP when configured to monitor both DCI formats 0_1 and 0_2 in the BWP | 38.306, 4.2.7.10 | Rel-16 | pc_dci_UL_PriorityIndicator_r16 | No |  | A UE supporting this feature shall also support ul-IntraUE-Mux-r16 and dci-Format1-2And0-2-r16 |
| 82 | Supports restricting data transmission from a given LCH to a configured (sub-) set of dynamic grant priority levels | 38.306, 4.2.6 | Rel-16 | pc_lch_ToGrantPriorityRestriction_r16 | No |  |  |
| 83 | Supports two PUCCH group in CA with a same numerology across CCs for data and control channel. | 38.306 4.2.7.7 | Rel-15 | pc_twoPUCCH_group | No |  |  |
| 84 | Support of transparent Tx diversity requirements for at least one NR FR1 band for single band (non-CA) case | 38.306, 4.2.7.238.331, Annex C | Rel-16 | pc_txDiversity_r16 | No |  | FR1 only This capability has been introduced in Rel-16 and is early implementable from Rel-15 onwards. |
| 84A | Support of 2Tx Tx diversity for the band configured | 38.306, 4.2.7.7 | Rel-18 | pc_txDiversity2Tx_r18 | No |  |  |
| 85 | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | 38.306, 4.2.7.2 | Rel-17 | pc_pusch_RepetitionMsg3_r17 | No |  |  |
| 86 | Support of DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. | 38.306, 4.2.7.10 | Rel-15 | pc_dl_SchedulingOffset_PDSCH_TypeA | Yes |  |  |
| 87 | Support of CQI reporting with 4 bits per subband for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_cqi_4_BitsSubbandNTN_SharedSpectrumChAccess_r17 | No |  | UE supports CQI reporting with 4 bits per subband for NTN and shared spectrum channel access |
| 88 | Support of propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_ta_BasedPDC_NTN_SharedSpectrumChAccess_r17 | No |  | UE supports propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access |
| 89 | Support of 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n8 | No |  | UE supports 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |
| 90 | Support of 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n16 | No |  | UE supports 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |
| 91 | Support of NTN features in GSO scenario | 38.306, 4.2.2 | Rel-17 | pc_ntn_ScenarioSupport_r17_GSO | No |  | UE supports NTN features in GSO scenario |
| 92 | Support of NTN features in NGSO scenario | 38.306, 4.2.2 | Rel-17 | pc_ntn_ScenarioSupport_r17_NGSO | No |  | UE supports NTN features in NGSO scenario |
| 93 | Void |  |  |  |  |  |  |
| 94 | Supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 | 38.306, 4.2.7.11 | Rel-16 | pc_extendedBand_n77_r16 | No |  | Applicable for UE support band n77 and in the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. |
| 95 | Supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 | 38.306, 4.2.7.11 | Rel-17 | pc_extendedBand_n77_2_r17 | No |  | Applicable for UE support band n77 and in Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. |
| 96 | Void |  |  |  |  |  |  |
| 97 | Void |  |  |  |  |  |  |
| 98 | Void |  |  |  |  |  |  |
| 99 | Void |  |  |  |  |  |  |
| 100 | Void |  |  |  |  |  |  |
| 101 | Void |  |  |  |  |  |  |
| 102 | Void |  |  |  |  |  |  |
| 103 | Void |  |  |  |  |  |  |
| 104 | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_2MIMO_FR1_r17_FDD | No |  | FR1 FDD bands |
| 105 | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_2MIMO_FR1_r17_TDD | No |  | FR1 TDD bands |
| 106 | Support of 1024QAM modulation scheme for PDSCH for FR1_FDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_FR1_r17_FDD | No |  | FR1 FDD bands |
| 107 | Support of 1024QAM modulation scheme for PDSCH for FR1_TDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_FR1_r17_TDD | No |  | FR1 TDD bands |
| 108 | Support RTT-based propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS. | 38.306, 4.2.7.5 | Rel-17 | pc_rtt_BasedPDC_CSI_RS_ForTracking_r17 | No |  | A UE supporting this feature shall also indicate support of csi-RS-ForTracking and supportedSRS-Resources as specified in TS 38.331. |
| 109 | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS. | 38.306, 4.2.7.5 | Rel-17 | pc_rtt_BasedPDC_PRS_r17 | No |  | If UE provides parameter maxNumberPRS-Resource-r17 and optionally parameter maxNumberPRS-ResourceProcessedPerSlot-r17 as described in TS 38.331, consider this as supported, otherwise not supported. A UE supporting this feature shall also indicate support of supportedSRS-Resources as specified in TS 38.331. |
| 110 | Support propagation delay compensation based on legacy TA procedure for TN and non-shared spectrum channel access. | 38.306, 4.2.7.10 | Rel-17 | pc_ta_BasedPDC_TN_NSSChAccess_r17 | No |  |  |
| 111 | Support Type 1 PUSCH transmissions with configured grant | 38.306 4.2.7.10 | Rel-15 | pc_type1_PUSCH_RepetitionMultiSlots | No |  |  |
| 112 | Support Type 2 PUSCH transmissions with configured grant | 38.306 4.2.7.10 | Rel-15 | pc_type2_PUSCH_RepetitionMultiSlots | No |  |  |
| 113 | Support the dynamic indication of the number of repetitions for PUSCH transmission | 38.306 4.2.7.10 | Rel-16 | pc_pusch_RepetitionTypeA_r16 | No |  |  |
| 114 | Support repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication | 38.306 4.2.7.7 | Rel-17 | pc_pucch_Repetition_F0_1_2_3_4_DynamicIndication_r17 | No |  |  |
| 115 | Support the increased maximum number of PUSCH Type A repetitions up to 32 | 38.306 4.2.7.2 | Rel-17 | pc_maxNumberPUSCH_TypeA_Repetition_r17 | No |  |  |
| 116 | Support PUSCH repetitions based on available slots | 38.306 4.2.7.2 | Rel-17 | pc_puschTypeA_RepetitionsAvailSlot_r17 | No |  |  |
| 117 | Supports TB processing over multi-slot PUSCH without repetition | 38.306 4.2.7.2 | Rel-17 | pc_tb_ProcessingMultiSlotPUSCH_r17 | No |  |  |
| 118 | Supports repetition of TB processing over multi-slot PUSCH | 38.306 4.2.7.2 | Rel-17 | pc_tb_ProcessingRepMultiSlotPUSCH_r17 | No |  |  |
| 119 | Support SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH | 38.306, 4.2.7.5 | Rel-17 | pc_sfn_schemeA_r17 | No |  |  |
| 120 | Support SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH | 38.306, 4.2.7.5 | Rel-17 | pc_sfn_schemeB_r17 | No |  |  |
| 121 | Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group | 38.306, 4.2.7.2 | Rel-16 | pc_multi_sps_r16 | No |  | The UE can include this feature only if the UE indicates support of downlinkSPS. |
| 122 | Support transmission of two PUCCH formats in TDM in the same slot | 38.306, 4.2.7.10 | Rel-15 | pc_twoPUCCH_AnyOthersInSlot | No |  |  |
| 123 | Support of unified TCI state operation with joint DL/UL TCI update for intra-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedJointTCI_r17 | No |  |  |
| 124 | Support of unified TCI state operation with separate DL/UL TCI update for intra-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_r17 | No |  |  |
| 125 | Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info | 38.306, 4.2.7.2 | Rel-17 | pc_mTRP_InterCell_r17 | No |  |  |
| 126 | Support of unified TCI with separate DL/UL TCI update for inter-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_InterCell_r17 | No |  |  |
| 127 | Support of dynamic 2Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-17 | pc_2Tx_ULTxSwitchingBandPair_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127a | Support of R18 dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL | 38.306 4.2.7.1 | Rel-18 | pc_ULTxSwitchingBandPair_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127b | Support of R18 dynamic UL Tx switching with 35us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_35us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127c | Support of R18 dynamic UL Tx switching with 140us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_140us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127d | Support of R18 dynamic UL Tx switching with 210us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_210us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127e | Support of R18 dynamic UL Tx switching with DL interruption occurring during Tx switching | 38.306 4.2.7.1 | Rel-18 | pc_uplinkTxSwitching_DL_Interruption_r18 | No |  | Not allowed to be setted to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 128 | Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. | 38.306, 4.2.7.2 | Rel-17 | pc_aperiodicCSIRS_FastScellActivation_r17 | No |  | Includes maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 to indicate the number of RS configurations for fast SCell activation that can be indicated by the MAC CE |
| 129 | Indicates whether the UE supports conditional PSCell addition in EN-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in EN-DC. | 38.306, 4.2.7.9 | Rel-17 | pc_condPSCellAdditionENDC_r17 | No |  |  |
| 130 | Support of conditional PSCell addition in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_condPSCellAdditionNRDC_r17 | No |  |  |
| 131 | Support of activation (with or without RACH) and deactivation on SCG in EN-DC | 38.306, 4.2.7.9 | Rel-17 | pc_scg_ActivationDeactivationENDC_r17 | No |  |  |
| 132 | Support of activation (with or without RACH) and deactivation on SCG in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_scg_ActivationDeactivationNRDC_r17 | No |  |  |
| 133 | Support of TCI state update and activation by a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) And b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_multiMAC_CE_r17 | No |  | A UE supporting this feature shall also support unifiedSeparateTCI-r17. |
| 134 | Indicates whether UE supports partial frequency sounding for SRS with frequency hopping. | 38.306, 4.2.7.2 | Rel-17 | pc_srs_partialFequencySounding_r17 | No |  |  |
| 135 | Indicates whether the UE supports partial frequency sounding for SRS with frequency hopping and start RB location hopping. | 38.306, 4.2.7.2 | Rel-17 | pc_srs_startRB_locationHoppingPartial_r17 | No |  | A UE supporting this feature shall also indicate support of pc_srs_partialFequencySounding_r17 as specified in TS 38.306. |
| 136 | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot (pdcch-MonitoringAnyOccasions) | 38.306, 4.2.7.5 | Rel-15 | pc_pdcchMonitoringAnyOccasions | No |  |  |
| 137 | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot with minimum time separation between two consecutive transmissions of PDCCH with span (pdcch-MonitoringAnyOccasionsWithSpanGap) | 38.306, 4.2.7.5 | Rel-15 | pc_pdcchMonitoringAnyOccasions_SpanGap | No |  |  |
| 138 | Support of UE reporting of information related to TA pre-compensation | 38.306, 4.2.7.2 | Rel-17 | pc_uplink_TA_Reporting_r17 | No |  | UE supports UE reporting of information related to TA pre-compensation. |
| 139 | Indicates whether the UE supports mTRP BFR based on two BFD-RS sets. | 38.306, 4.2.7.5 | Rel-17 | pc_mTRP_BFR_twoBFD_RS_Set_r17 | No |  |  |
| 140 | Indicates the support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs. | 38.306, 4.2.7.5 | Rel-17 | pc_mTRP_PDCCH_Repetition_r17 | No |  |  |
| 141 | Support of reception of UE-specific K_offset | 38.306, 4.2.7.2 | Rel-17 | pc_ue_specific_K_Offset_r17 | No |  |  |
| 142 | Support of at least 2 TAGs | 38.306, 4.2.7.4 | Rel-15 | pc_supportedNumberTAG_n2_n3_n4 | No |  |  |
| 143 | Indicates whether UE supports multi-TRP PUSCH repetition based on codebook with PUSCH repetition type A. | 38.306, 4.2.7.7 | Rel-17 | pc_mtrpPuschTypeACb_r17 | No |  |  |
| 144 | Indicates whether UE supports multi-TRP PUSCH repetition based on codebook with PUSCH repetition type B. | 38.306, 4.2.7.8 | Rel-17 | pc_mtrpPuschTypeBCb_r17 | No |  |  |
| 145 | Indicates whether UE supports cyclic mapping when the number of repetitions is larger than 2 with repetition type. | 38.306, 4.2.7.2 | Rel-17 | pc_mtrpPuschCyclicMapping_r17 | No |  |  |
| 146 | Indicates whether UE supports PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs). | 38.306, 4.2.7.2 | Rel-17 | pc_mtrpPuschTwoPhrReporting_r17 | No |  |  |
| 147 | Indicates whether UE supports DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUCCH_Rep_r17 | No |  | UE supports DMRS bundling over PUCCH |
| 148 | Indicates whether UE supports DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_multiSlot_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 149 | Indicates whether the UE supports DM-RS bundling for PUSCH repetition type A over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_RepTypeA_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 150 | Indicates whether the UE supports DM-RS bundling for PUSCH repetition type B over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_RepTypeB_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 151 | Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | 38.306, 4.2.7.2 | Rel-18 | pc_support3MHz_ChannelBW_Symmetric_r18 | No |  | FR1 FDD bands |
| 152 | Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | 38.306, 4.2.7.2 | Rel-18 | pc_support12PRB_CORESET0_r18 | No |  | FR1 FDD bands |
| 153 | Support of 35us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_35us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 154 | Support of 140us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_140us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 155 | Support of 210us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_210us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 156 | Support of DL interruption during 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_DL_Interruption_r17 | No |  | Not allowed to be set to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 157 | Indicates the interruption time on DL/UL reception within a NR band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS | 38.306, 4.2.7.1 | Rel-16 | pc_srs_SwitchingTimeNR | No |  |  |
| 158 | Indicates the interruption time on DL/UL reception within a EUTRA band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS | 38.306, 4.2.7.1 | Rel-16 | pc_srs_SwitchingTimeEUTRA | No |  |  |
| 159 | Indicates whether the UE supports multi-PUSCHs for configured grant by indicating whether the UE supports the determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG | 38.306, 4.2.7.2 | Rel-18 | pc_multiPUSCH_CG_r18 | No |  | A UE supporting this feature shall also indicate support of at least one of configuredUL-GrantType1, configuredUL-GrantType1-v1650, configuredUL-GrantType2, and configuredUL-GrantType2-v1650. |
| 160 | Supports {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. | 38.306 4.2.7.2 | Rel-18 | pc_prach_CoverageEnh_r18 | No |  |  |
| 161 | UE is 2Rx XR UE | 38.306, 4.2.7.2 | Rel-18 | pc_supportOf2RxXR_r18 | No |  | A UE reporting this parameter shall not indicate support of pc_supportOfRedCap_r17 or pc_supportOfERedCap_r18. |
| 162 | Indicates whether the UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)' field. | 38.306, 4.2.7.4 | Rel-18 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | No |  |  |
| 163 | Indicates whether the UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)', 'Precoding information and number of layers' and 'SRS resource indicator' fields. | 38.306, 4.2.7.4 | Rel-18 | pc_multiCell_PUSCH_DCI_0_3_SameSCS_r18 | No |  |  |
| 164 | Indicates whether the UE supports co-scheduled cell indication scheme based on FDRA field of DCI format 1_3. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3 | No |  |  |
| 165 | Indicates whether the UE supports co-scheduled cell indication scheme based on Scheduled cell set indicator field of DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3 | No |  |  |
| 166 | Indicates whether the UE supports co-scheduled cell indication scheme based on FDRA field of DCI format 0_3. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3 | No |  |  |
| 167 | Indicates whether the UE supports co-scheduled cell indication scheme based on Scheduled cell set indicator field of DCI format 0_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3 | No |  |  |
| 168 | Indicate whether the UE supports search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_supportOfSearchSpace_r18_DCI_1_3 | No |  |  |
| 169 | Indicate whether the UE supports search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_supportOfSearchSpace_r18_DCI_0_3 | No |  |  |
| 170 | Indicates whether the UE supports harq feedback type1 for DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_harqFeedbackType1_DCI_1_3 | No |  |  |
| 171 | Indicates whether the UE supports harq feedback type2 for DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_harqFeedbackType2_DCI_1_3 | No |  |  |
| 172 | Indicates whether the UE supports basic feature of Rel-18 enhanced DMRS ports for PDSCH for scheduling of mapping type A. | 38.306 4.2.7.5 | Rel-18 | pc_pdsch_TypeA_DMRS_r18 | No |  |  |
| 173 | Indicates whether the UE supports unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC. TCI state indication for update and activation includes: - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) with DL assignment; - MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2) without DL assignment. The capability signalling comprises the following parameters: - maxNumActiveDL-TCI-AcrossCC-r18 indicates the maximum number of activated DL TCI states across all CCs, - maxNumActiveUL-TCI-AcrossCC-r18 indicates the maximum number of activated UL TCI states across all CCs. | 38.306, 4.2.7.2 | Rel-18 | pc_tci_SeparateTCI_UpdateMultiActiveTCI_PerCC_r18 | No |  | FR2 |
| 174 | Indicates whether the UE supports two TA enhancement for multi-DCI based intra-cell Multi-TRP operation. A UE supporting this feature shall also indicate support of multiDCI-MultiTRP-r16. | 38.306, 4.2.7.6 | Rel-18 | pc_multiDCI_IntraCellMultiTRP_TwoTA_r18 | No |  |  |
| 175 | Support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ltm_BeamIndicationJointTCI_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-r18 and ltm-SCG-r18. |
| 176 | Support unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ltm_BeamIndicationSeparateTCI_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-r18 and ltm-SCG-r18. |
| 177 | Support fast processing of LTM candidate cell RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_ltm_FastProcessingConfig_r18 | No |  | A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. |
| 178 | Support MAC-CE activated joint LTM TCI states | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MAC_CE_JointTCI_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. |
| 179 | Support MAC-CE activated DL/UL LTM TCI states | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MAC_CE_SeparateTCI_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. |
| 180 | Support RACH-based early TA acquisition | 38.306 4.2.7.2 | Rel-18 | pc_rach_EarlyTA_Measurement_r18 | No |  | A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-r18 and ltm-SCG-r18. |
| 181 | Support TA indication in cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ta_IndicationCellSwitch_r18 | No |  | A UE supporting this feature shall also indicate support of at least one of ltm-RACHLessCG-r18 and ltm-RACHLessDG-r18 and support of ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18 for the same band. |
| 182 | Support UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for | 38.306 4.2.7.2 | Rel-18 | pc_ue_TA_Measurement_r18 | No |  | A UE supporting this feature shall also indicate the support of at least one of ltm-RACHLessCG-r18 and ltm-RACHLessDG-r18 and support of ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18 for the same band. |
| 183 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_AffectedBands_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. |
| 184 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_SwitchingTimeList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. |
| 185 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_PrepTime_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. |
| 186 | Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) | 38.306 4.2.7.7 | Rel-18 | pc_rach_EarlyTA_BandList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. |
| 187 | Indicates whether the UE supports cell DTX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDTXonly' or 'both' and UE shall also indicate support of longDRX-Cycle |
| 188 | Indicates whether the UE supports cell DRX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDRX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDRXonly' or 'both' |
| 189 | Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9 | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_DRX_DCI2_9_r18 | No |  | A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18 |
| 190 | Indicates whether the UE supports R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, when co-scheduled UE(s)' modulation order is explicitly signalled by DCI. | 38.306 4.2.7.10 | Rel-18 | pc_advReceiver_MU_MIMO_r18 | No |  |  |
| CAPNOK11 | Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. | 38.306 4.2.7.2 | Rel-18 | pc_spatialAdaptation_CSI_Feedback_r18 | No |  | A UE supporting of this feature shall also indicate support of csi-ReportFramework and spatialAdaptation-CSI-FeedbackPerBC-r18. |
| CAPNOK12 | Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. | 38.306 4.2.7.2 | Rel-18 | pc_powerAdaptation_CSI_Feedback_r18 | No |  | A UE supporting of this feature shall also indicate support of csi-ReportFramework and powerAdaptation-CSI-FeedbackPerBC-r18. |

--------------------------------------------------------- End of first change ---------------------------------------------------------