3GPP TSG-RAN5 Meeting #105	R5-247239

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0749 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Half-duplex FDD PICS. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RedCap capabilities for half-duplex and half-duplex overlapping MFBI band have been updated to support also eRedCap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating half-duplex and half-duplex overlapping MFBI band to support also eRedCap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eRedCap half-duplex and half-duplex overlapping MFBI band functionality is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0538 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------- Start of change ---------------------------------------------------------

### A.4.3.12	RedCap Capabilities

According to TS 38.306 [17] clause 4.2.21:

CA, MR-DC, DAPS, CPAC and IAB (i.e the RedCap IE is not expected to act as IAB mode) related UE features and corresponding capabilities are not supported by the RedCap UEs.

-	PICS associated to the following features are as below:

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

-	UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branches or more than 1 UL MIMO layers are not supported by RedCap UE.

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported.

-	For FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported.

Table A.4.3.12-1: RedCap UE Capabilities

| Item | UE Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOf16DRB_RedCap_r17 | No |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOfRedCap_r17 | No |  | This PICS shall always be true for RedCap UE. |
| 3 | Void |  |  |  |  |  |  |
| 4 | Void |  |  |  |  |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap or eRedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_r17 | No |  | FDD FR1 only |
| 6 | Support of relaxed RRM measurements in RRC_CONNECTED for RedCap UE. | 38.306 4.2.21.5 | Rel-17 | pc_rrm_RelaxationRRC_ConnectedRedCap_r17 | No |  |  |
| 7 | Support of initiating UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. | TS 38.331 5.7.4.2 | Rel-17 | pc_UAI_rrm_RelaxationRRC_ConnectedRedCap | No |  | The UE will initiate UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. It is only applicable for RedCap UE. |
| 8 | Support of Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_Relaxed_Measurement_RedCap_r17 | No |  | It is optional for RedCap UE to support Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE |
| 9 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for overlap MFBI band undertest for RedCap or eRedCap UE | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_MFBI | No |  | FDD FR1 only It is only applicable when pc_halfDuplexFDD_TypeA_RedCap_r17=True |

--------------------------------------------------------- End of change ---------------------------------------------------------