Title:	Discussion on test point selection for additional spurious emission with NS_62 in FR1

Document for:	Discussion and approval

Source:	China Telecom

# Background

The intention with NS_17 is to protect 1541 – 1625 MHz as indicated in Table 1 below.

Table1: Additional requirements for "NS_62"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1625 | -70+ 66/15 (f-1610) | 1MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using 0 dBi antenna. |  |  |  |

In general for certain NS value both A-MPR requirements and emissions minimum requirements such as SEM and/or Spurious are defined. The test point analysis of emission requirements would follow that of A-MPR. But for NS_62 only additional spurious emission requirements apply and no A-MPR is allowed, therefore the test point analysis of general spurious emission can be reused as much as possible, which reflects the most critical test point configuration for spurious emission measurement.

## Test Environment

The test environment for NR general spurious emission measurement for FR1 can be reused.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for additional spurious emission for NS_62 in FR1.

## Test Frequencies

UL test carrier centre frequencies should be located to part closest to the band to be protected 1541 ≤ f ≤ 1625.

PROPOSAL 2: Select 1672.5MHz for 5MHz channel bandwidth as the test frequencies for NS_62 in FR1.

## Test Channel Bandwidth

Minimum requirements for NS_62 apply for channel bandwidths 5MHz, which shall be selected to test additional spurious emission.

PROPOSAL 3: 5MHz need to be selected to test additional spurious emission for NS_62 in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for general spurious emission, i.e. lowest supported subcarrier spacing.

PROPOSAL 4: Select Lowest supported subcarrier spacing for NS_62 in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for general spurious emission, i.e. CP-OFDM QPSK.

PROPOSAL 5: Test CP-OFDM QPSK for additional spurious emission for NS_62 in FR1.

## Uplink RB allocation

Since there is no additional MPR for NS_62, the selection of UL RB allocation can be based on the agreed proposals for general spurious emission.

for NS_62 has some region-specific requirements in the lower part of operating band n54. Edge_1RB_left and Outer_Full can be considered as a worst-case scenario. No need to test with Edge_1RB_Right as it is not close to the lower part of the operating band.

PROPOSAL 6: Edge_1RB_left and Outer_Full are tested for additional spurious emission for NS_62 in FR1.

2.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_62:

| Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- |
| 1 (NC) | 1 X 1 X 2 X 1 | 1 | 2 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_62 A-SE test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for additional spurious emission for NS_62 in FR1.

PROPOSAL 2: Select 1672.5MHz for 5MHz channel bandwidth as the test frequencies for NS_62 in FR1.

PROPOSAL 3: 5MHz need to be selected to test additional spurious emission for NS_62 in FR1.

PROPOSAL 4: Select Lowest supported subcarrier spacing for NS_62 in FR1.

PROPOSAL 5: Test CP-OFDM QPSK for additional spurious emission for NS_62 in FR1.

PROPOSAL 6: Edge_1RB_left and Outer_Full are tested for additional spurious emission for NS_62 in FR1.

# References

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)