3GPP TSG-RAN5 Meeting #105	R5-247346

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3404 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on message content for FR2 CQI and RI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For FR2 CQI and RI test cases with aperiodic reporting, it is required to transmit HARQ feedback and CSI feedback in PUSCH in the same UL slot and UL symbols. However, according to TS 38.213, section 9, a UE does not expect to detect a DCI format scheduling a PDSCH reception and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission. So, when the UCI for HARQ feedback is sent in PUSCH, all DCI format 1-1 used to schedule the PDSCH transmission have to be sent before sending the DCI format 0-1 to configure the PUSCH transmission in which the UCI is expected. In order to transmit all the DL slots defined in reference measurement channel, it is needed to avoid the transmission of the HARQ feedback and the CSI reporting multiplexed in the same UL slots and UL symbols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update condition in Table 5.4.2.4-17 to include FR2 Aperiodic CQI test cases. Include new table in section 5.4.2.6 to modify PUSCH-TimeDomainResourceAllocationList to decrease number of symbols for PUSCH to 12 for CSI reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Defined PDSCH reference measurement channel will not be transmitted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4, 5.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

PUSCH-TimeDomainResourceAllocationList

Table 5.4.2.4-17: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: Table 5.4.2.0-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  | APERIODIC_CQI |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC_CQI | For aperiodic CQI reporting test cases |

## <<< Skip unchanged sections >>>

PUSCH-TimeDomainResourceAllocationList

Table 5.4.2.6-20: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: Table 5.4.2.0-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  | FR2 |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| startSymbolAndLength | 55 | Start symbol(S)=0, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

## <<< END OF CHANGES >>>