3GPP TSG-RAN5 Meeting #105	R5-246274r1

# Orlando, US, 18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3002 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of four antenna port Reference sensitivity requirements for bands n13 and n85 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_NR_bands_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing four antenna port Referense sensitivity requirements for bands n13 and n85 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 4RX Referense sensitivity requirements for bands n13 and n85 in: - Table 7.3.2.3-2: Four antenna port reference sensitivity allowance - Table 7.3.2.5-2a: Four antenna port Reference sensitivity test requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4RX Referense sensitivity requirements for bands n13 and n85 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.3, 7.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-246272 (CR 3000) correct the order of 3 MHz and 5 MHz CBWs colums in Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add China Telecom as co-source in the cover sheet |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 7.3.2	Reference sensitivity power level

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that don’t support RedCap.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d and Table 7.3.2.3-2.

-------- Skipped unchanged clauses --------

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2.3-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2.3-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n8, n13, n28, n71, n85 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79 | -2.2 |
| NOTE 1: When 4Rx operation is supported by FWA form factor. NOTE 2: When 4Rx operation is supported by handheld UE. |  |

The reference sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d and Table 7.3.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3.

-------- Skipped unchanged clauses --------

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3.

-------- Skipped unchanged clauses --------

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands for PC3

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 3 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -102.7 +TT |  | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 | -100.7 +TT |  | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 | -99.7 +TT |  | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n56 | 15 |  | -100.4 +TT | -97.2 +TT | -95.4 +TT | -89.2 +TT | -87.2 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.5 +TT | -95.5 +TT | -91 +TT | -87.3 +TT |  |  |  |  |  |  |
| n71 | 15 | -100.7 +TT |  | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -92.6 +TT |  | -91.3 +TT |  | -84.2 +TT | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -92.7 +TT |  | -91.4 +TT |  | -84.2 +TT |  |
|  | 60 |  |  | -98.2 +TT | -97.1 +TT | -94.9 +TT | -93.7 +TT | -92.8 +TT |  | -91.6 +TT |  | -84.2 +TT |  |
| n85 | 15 | -99.7 +TT |  | -96.5 +TT | -94.1 +TT | -88.5 +TT |  |  | -81.1 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT | -94.4 +TT | -89.9 +TT |  |  | -81.2 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n86 | 15 |  | -99.4 +TT | -96.2 +TT | -93.8 +TT | -88.2 +TT |  |  | -80.8 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -94.1 +TT | -89.6 +TT |  |  | -80.9 +TT |  |  |  |  |
| n13 | 15 | -99.7 +TT |  | -96.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT |  |  |  |  |  |  |  |  |  |
| n206 | 15 |  | -99.4 +TT | -96.2 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -93.5 +TT | -92.4 +TT |  |  |  |  |  |  |  |
| n266 | 15 | -102.1 +TT | -99.9 +TT | -96.97 +TT | -95.17 +TT | -90 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.2 +TT | -95.1 +TT | -90.1 +TT |  |  |  |  |  |  |  |
| n286 | 15 | -102.6 +TT | -100.9 +TT | -97.9+ TT | -95.9 +TT | -93.2 +TT |  | -80.9 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -98 +TT | -96 +TT | -93.4 +TT |  | -81 +TT |  |  |  |  |  |
| n25 | 15 | -99.2 +TT |  | -96.0 +TT | -94.2 +TT | -93.0 +TT | -92.0 +TT | -84.9 +TT | -84.4 +TT | -82.2 +TT | -80.3+TT |  | FDD |
|  | 30 |  |  | -96.3 +TT | -94.3 +TT | -93.2 +TT | -92.1 +TT | -85.0 +TT | -84.5 +TT | -82.3 +TT | -80.4 +TT |  |  |
|  | 60 |  |  | -96.7 +TT | -94.6 +TT | -93.4 +TT | -92.3 +TT | -85.1 +TT | -84.6 +TT | -82.4 +TT | -80.5 +TT |  |  |
| n30 | 15 | -101.7 +TT |  | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -102.2 +TT |  | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 | -102.7 +TT |  | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| n716 | 15 |  | -99.6 +TT | -96.4 +TT | -94 +TT | -88.4 +TT | -86.68 +TT -77.39 +TT | -85.08 +TT -69.69 +TT | -83.28 +TT -66.59 +TT |  |  |  | FDD |
|  | 30 |  | -96.7 +TT | -94.3 +TT | -89.8 +TT | -86.7 +TT | -85.18 +TT -77.49 +TT | -83.38 +TT -69.79 +TT | -96.78 +TT -66.69 +TT |  |  |  |  |
| n85 | 15 | -99.7 +TT | -101.9 +TT | -96.5 +TT | -86.7 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT | -86.8 +TT |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 5: When 4Rx operation is supported by FWA form factor. NOTE 6: When 4Rx operation is supported by handheld UE. NOTE 7: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------