3GPP TSG-RAN5 Meeting #105	R5-247047r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3387 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RRC IEs for Further measurement gap enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Further measurement gap enhancements test cases, new RRC IEs needs to be defined according to TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC IEs were clarified according to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of RRC IEs for Further measurement gap enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: aligned MeasWindowConfig with the title in Table 4.6.3-80C updated the list of entries in Table 4.6.3-82H |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.6.3	Radio resource control information elements

–	AdditionalSpectrumEmission

[Skip unchanged text]

–	MeasConfig

Table 4.6.3-69: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | MeasObjectToAddModList |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | ReportConfigToAddModList |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | MeasIdToAddModList |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | QuantityConfig |  |  |
| measGapConfig | Not present |  |  |
| measGapSharingConfig | Not present |  |  |
| interFrequencyConfig-NoGap-r16 | Not present |  |  |
| effectiveMeasWindowConfig-r18 | Not present |  |  |
| effectiveMeasWindowConfig-r18 CHOICE { |  |  |  |
| setup | MeasWindowConfig |  | EMW_Setup |
| release | null |  | EMW_Release |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EMW_Setup | Setup effective measurement window in NR for E-UTRA measurements |
| EMW_Release | Release effective measurement window in NR for E-UTRA measurements |

[Skip unchanged text]

–	MeasWindowConfig

Table 4.6.3-80C: MeasWindowConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasWindowConfig-r18 ::= SEQUENCE { |  |  |  |
| windowOffsetPeriodicity CHOICE { |  |  |  |
| periodicityMs40 | 0 |  | MS40 |
| periodicityMs80 | 0 |  | MS80 |
| } |  |  |  |
| windowDuration | ms5 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MS40 | 40 ms for measurement window periodicity |
| MS80 | 80 ms for measurement window periodicity |

[Skip unchanged text]

–	NeedForInterruptionInfoNR

Table 4.6.3-82H: NeedForInterruptionInfoNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForInterruptionInfoNR-r18 ::= SEQUENCE { |  |  |  |
| intraFreq-needForInterruption-r18 SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18 { | 1 entry |  |  |
| NeedForInterruptionNR-r18[1] | NeedForInterruptionNR with condition NGNI | entry 1 |  |
| } |  |  |  |
| interFreq-needForInterruption-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18b | 1 entry |  |  |
| NeedForInterruptionNR-r18[1] | NeedForInterruptionNR with condition NGWI | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

–	NeedForInterruptionNR

Table 4.6.3-82I: NeedForInterruptionNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForInterruptionNR-r18 ::= SEQUENCE { |  |  |  |
| interruptionIndication-r18 | no-gap-with-interruption |  | NGWI |
|  | no-gap-no-interruption |  | NGNI |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NGWI | interruption is needed to perform SSB based measurements without measurement gap |
| NGNI | interruption is not needed to perform SSB based measurements without measurement gap |

[End of change]