3GPP TSG-RAN5 Meeting #105	R5-247142r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0065 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | min test time update for nb-ntn npdcch and npdsch demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest, TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The “Min Gap between consecutive NPDCCH taking into account RTT (ms)” field originally considered K_offset to be 256ms. But 36.508 config assumes 258ms. So the calculation needs to be updated accordingly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Min Gap between consecutive NPDCCH taking into account RTT(ms) value to consider K_offste of 258ms Updated the min test time for TC 8.3.1.1.3 as per the test time determined via simulation in R5-247141 Due to the addition of SNR uncertainty due to finite test time (as per R5-247141), Annex F MU and TT value is updated. The SNR value in test requirements table is also accordingly adjusted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex G.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on discussion paper R5-247141 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.1.1.3.5	Test requirement

Table 8.3.1.1.3.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.12.1.1 for each throughput test shall meet or exceed the specified value in Table 8.3.1.1.3.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.3.1.1.3.5-1: Test requirements under standalone with 1 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 FDD | 1 | EPA5 | 1 | 70% | 10.3 | NB2 |

<<several sections skipped>>

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 PDSCH in standalone mode for UE category M1 under NTN fading conditions | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.3.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions | ± 0.8 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty2 + Fading profile power uncertainty2 + (0.25 x AWGN flatness and signal flatness)2 + variation due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.15 dB for Test 1 and ±0.2 dB for Test 2. |
| 8.3.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 | Same as clause 8.3.1.1.1. | Same as clause 8.3.1.1.1 with following exception, Result variation due to finite test time ±0.35 dB for Test 1 and ±0.3 dB for Test 2. |
| 8.3.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | ± 0.9 dB | Same as clause 8.3.1.1.1 with following exception, Result variation due to finite test time ±0.5 dB for Test 1 |
| 8.3.1.2.1 Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | Same as clause 8.12.2.1.1 in TS 36.521-1 [14]. |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

<<several sections skipped>>

## F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 8.2.1.1.1 PDSCH in standalone mode for UE category M1 under NTN fading conditions | SNRs as specified in clause 8.2.1.1.1 | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions | SNRs as specified in clause 8.3.1.1.1 | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 | SNRs as specified in clause 8.3.1.1.2 | Same as clause 8.3.1.1.1 | Same as clause 8.3.1.1.1 |
| 8.3.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | SNRs as specified in clause 8.3.1.1.3 | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.2.1 Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | SNRs as specified in clause 8.12.2.1.1 in TS 36.521-1 [14] | Same as clause 8.12.2.1.1 in TS 36.521-1 [14] | Same as clause 8.12.2.1.1 in TS 36.521-1 [14] |

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 881 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 489 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1982 | 2304 | 1013760 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

Table G.3.5-2: Minimum Test time for PDSCH for UE Category NB2

| Test No | Demodulation scenario (info only) |  | MNAS (Simulation) |  | Min Gap between consecutive NPDCCH taking into account RTT (ms) |  | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.1.3 Test 1 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 |  | 600 |  | 283 |  | 288 | 599904 |  |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |  |  |  |  |

<<several section skipped>>

## G.4.6	Minimum Test time

Table G.4.6-1: Minimum Test time for NPDCCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | HD-FDD |
| 8.3.1.2.1: Test 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 36166 | 15430827 | 15431000 |
| 8.3.1.2.1: Test 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 36166 | 55550976 | 55551000 |

<<End of changes>>