3GPP TSG-RAN5 Meeting #105	R5-247296r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3607 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to spec implementation, including editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some of the agreed CRs in RAN5#104 have not been properly implemented in the specification TS 38.533 18.4.0 has some editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial issues have been corrected Correction of implementation in the specification according to agreed CRs from RAN5#104: R5-245952, R5-245953 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec will remain wrongly implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 6.7, 14, 16.7, 17, 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: further editorial corrections included for 4.7.1.1.1, 4.7.1.1.2, 17.4.1.1, 17.5.5.2.1, 18.3.1.5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 4.7.1	SS-RSRP

#### 4.7.1.0	Minimum conformance requirements

4.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

The intra-frequency SS-RSRP absolute accuracy requirements are defined for the SS-RSRP measured from a cell on the same frequency as that of the PCell or PSCell in FR1.

The accuracy requirements in Table 4.7.1.0.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 [2] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band  for each relevant SSB.

Table 4.7.1.0.1-1: SS-RSRP intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in Section 3A.4.1 |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

Table 4.7.1.0.1-2: SS-RSRP and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value(L3 SS-RSRP) | Measured quantity value(L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156≤ SS-RSRP<-155 | Not valid | dBm |
| RSRP_2 | -155≤ SS-RSRP<-154 | Not valid | dBm |
| RSRP_3 | -154≤ SS-RSRP<-153 | Not valid | dBm |
| RSRP_4 | -153≤ SS-RSRP<-152 | Not valid | dBm |
| RSRP_5 | -152≤ SS-RSRP<-151 | Not valid | dBm |
| RSRP_6 | -151≤ SS-RSRP<-150 | Not valid | dBm |
| RSRP_7 | -150≤ SS-RSRP<-149 | Not valid | dBm |
| RSRP_8 | -149≤ SS-RSRP<-148 | Not valid | dBm |
| RSRP_9 | -148≤ SS-RSRP<-147 | Not valid | dBm |
| RSRP_10 | -147≤ SS-RSRP<-146 | Not valid | dBm |
| RSRP_11 | -146≤ SS-RSRP<-145 | Not valid | dBm |
| RSRP_12 | -145≤ SS-RSRP<-144 | Not valid | dBm |
| RSRP_13 | -144≤ SS-RSRP<-143 | Not valid | dBm |
| RSRP_14 | -143≤ SS-RSRP<-142 | Not valid | dBm |
| RSRP_15 | -142≤ SS-RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141≤ SS-RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140≤ SS-RSRP<-139 | -140≤ RSRP<-139 | dBm |
| RSRP_18 | -139≤ SS-RSRP<-138 | -139≤ RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | -46≤ RSRP<-45 | dBm |
| RSRP_112 | -45≤ SS-RSRP<-44 | -45≤ RSRP<-44 | dBm |
| RSRP_113 | -44≤ SS-RSRP<-43 | -44≤ RSRP | dBm |
| RSRP_114 | -43≤ SS-RSRP<-42 | Not valid | dBm |
| RSRP_115 | -42≤ SS-RSRP<-41 | Not valid | dBm |
| RSRP_116 | -41≤ SS-RSRP<-40 | Not valid | dBm |
| RSRP_117 | -40≤ SS-RSRP<-39 | Not valid | dBm |
| RSRP_118 | -39≤ SS-RSRP<-38 | Not valid | dBm |
| RSRP_119 | -38≤ SS-RSRP<-37 | Not valid | dBm |
| RSRP_120 | -37≤ SS-RSRP<-36 | Not valid | dBm |
| RSRP_121 | -36≤ SS-RSRP<-35 | Not valid | dBm |
| RSRP_122 | -35≤ SS-RSRP<-34 | Not valid | dBm |
| RSRP_123 | -34≤ SS-RSRP<-33 | Not valid | dBm |
| RSRP_124 | -33≤ SS-RSRP<-32 | Not valid | dBm |
| RSRP_125 | -32≤ SS-RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31≤ SS-RSRP | Not valid | dBm |
| RSRP_1271 | Infinity | Infinity | dBm |
| NOTE: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [13], but not for the purpose of measurement reporting. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.6.

4.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

The intra-frequency relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency in FR1.

The accuracy requirements in Table 4.7.1.0.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 [2] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band  for each relevant SSB.

Table 4.7.1.0.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in Section 3A.4.1. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.2.1.2 and 10.1.6.

4.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

The inter-frequency SS-RSRP absolute accuracy requirements in this clause are defined for the SS-RSRP measured from a cell on a different frequency as that of the PCell or PSCell in FR1.

The accuracy requirements in Table 4.7.1.0.3-1 are valid under the following conditions:

-	Conditions defined in TS 38.101-1 [2] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to clause B.2.3 for a corresponding Band  for each relevant SSB.

Table 4.7.1.0.3-1: SS-RSRP inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | 6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | 6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void. NOTE 3: NR operating band groups in FR1 are as defined in Section 3A.4.1. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.4.1.1 and 10.1.6.

4.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

The inter-frequency SS-RSRP relative accuracy requirements in this clause are defined for the SS-RSRP measured from one cell on a frequency in FR1compared to the SS-RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 4.7.1.0.4-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 [2] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band  for each relevant SSB,

-

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 4.7.1.0.4-1: SS-RSRP inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | 6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in Section 3A.4.1. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.4.1.2 and 10.1.6.

#### 4.7.1.1	Intra-frequency measurements

##### 4.7.1.1.1	EN-DC FR1 SS-RSRP absolute measurement accuracy

4.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

4.7.1.1.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards.

4.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.1.1.

4.7.1.1.1.4	Test description

4.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.1.1.1.4.1-1.

Table 4.7.1.1.1.4.1-1: EN-DC FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.7.1.1.1-1 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1.1-2 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1.1-3 | LTE FDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.7.1.1.1-4 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1.1-5 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1.1-6 | LTE TDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 4.7.1.1.1.4.1-2.

Table 4.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.7.1.1.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in the same frequency. Cell 2 is the PSCell and Cell 3 is the target cell for SS-RSRP measurements. The connection setup is done according to the settings in clause C.1.1.

4.7.1.1.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.7.1.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 3 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 4.7.1.1.1.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 4.7.1.1.1.5-1 as appropriate and repeat steps 5-7.

4.7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.7.1.1.1.4.3-1: Common Exception messages for EN-DC FR1 SS-RSRP
absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 |
| Specific message contents exceptions for Test Configuration 4.7.1.1.1.-1 and 4.7.1.1.1-4 | Table H.3.1-3 with Condition SSB.1 FR1 and Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 4.7.1.1.1-2 and 4.7.1.1.1-5 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 4.7.1.1.1-3 and 4.7.1.1.1-6 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 4.7.1.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for EN-DC FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

4.7.1.1.1.5	Test requirement

Table 4.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 4.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 4.7.1.1.1.5-2 for test configurations 1, 2, 4 and 5, and the corresponding absolute accuracy requirements in Table 4.7.1.1.1.5-3 for test configurations 3 and 6.

Table 4.7.1.1.1.5-1: EN-DC FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Physical cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2,5 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3,6 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3,6 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control Channel RMC |  | Config 1,4 |  | CCR.1.1 FD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3,6 |  | CR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1,4 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 2,5 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 3,6 |  | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 |
| Time offset with Cell 2 |  | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3,5,6 | s | - | 3 | - | 3 | - | 3 |
| SMTC Configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3,6 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15KhZ | -107.5 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | N/ANote 6 |  | -94 |  | -115 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -107.4 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3,6 | Depending on band group |  | N/ANote 6 |  | -91 |  | -112 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -5.57 | 1.88 | -5.57 | 0.09 | -5.56 |
|  |  |  | dB | 6 | 1.4 | 6 | 1.4 | 3 | -0.8 |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -101.5 | -106.1 | -82 | -86.6 | -111 + ΔBG_offset | -114.8 + ΔBG_offset |
|  | Config 3,6 | Depending on band group |  | N/ANote 6 | N/ANote 6 | -85 | -89.6 | -109+ ΔBG_offset | -112.8+ ΔBG_offset |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -80.39+ ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  | -51.91 |  | -75.12 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |  |  |  |

Table 4.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 3) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 36 |
|  |  |  | Bands NR_FDD_FR1_B | 36 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 38 |
|  |  |  | Bands NR_FDD_FR1_G | 39 |
|  |  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 3) | 56 | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  |  | Bands NR_FDD_FR1_B | 48 |
|  |  |  | Bands NR_TDD_FR1_C | 49 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  |  | Bands NR_FDD_FR1_G | 51 |
|  |  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 3) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  |  | Bands NR_FDD_FR1_B | 32 |
|  |  |  | Bands NR_TDD_FR1_C | 32 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 33 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  |  | Bands NR_FDD_FR1_G | 34 |
|  |  |  | Bands NR_FDD_FR1_H | 35 |
| Highest reported value (Cell 3) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  |  | Bands NR_FDD_FR1_B | 53 |
|  |  |  | Bands NR_TDD_FR1_C | 53 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  |  | Bands NR_FDD_FR1_G | 55 |
|  |  |  | Bands NR_FDD_FR1_H | 56 |
| NOTE: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |

Table 4.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for
the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 3) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 38 |
|  |  |  | Bands NR_FDD_FR1_B | 38 |
|  |  |  | Bands NR_TDD_FR1_C | 39 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 39 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 41 |
|  |  |  | Bands NR_FDD_FR1_H | 41 |
| Highest reported value (Cell 3) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 50 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 53 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 3) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  |  | Bands NR_FDD_FR1_G | 36 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 3) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 54 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 55 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 56 |
|  |  |  | Bands NR_FDD_FR1_G | 57 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| NOTE: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 4.7.1.1.2	EN-DC FR1 SS-RSRP relative measurement accuracy

4.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

4.7.1.1.2.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards.

4.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.1.1.

4.7.1.1.2.4	Test description

4.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.1.1.2.4.1-1.

Table 4.7.1.1.2.4.1-1: EN-DC FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.7.1.1.2-1 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1.2-2 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1.2-3 | LTE FDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.7.1.1.2-4 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1.2-5 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1.2-6 | LTE TDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 4.7.1.1.2.4.1-2.

Table 4.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.7.1.1.2.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in the same frequency. Cell 2 is the PSCell and Cell 3 is the target cell for SS-RSRP measurements. The connection setup is done according to the settings in clause C.1.1.

4.7.1.1.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.7.1.1.2.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values of Cell 2 and Cell 3 in the periodic MeasurementReport. The SS-RSRP value of Cell 3 reported by the UE is compared to the reported SS-RSRP of Cell 2. If the resulting value is outside the limits in Table 4.7.1.1.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 4.7.1.1.2.5-1 as appropriate and repeat steps 5-7.

4.7.1.1.2.4.3	Message contents

Message contents are same as in clause 4.7.1.1.1.4.3.

4.7.1.1.2.5	Test requirement

Table 4.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 4.7.1.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 4.7.1.1.2.5-2.

Table 4.7.1.1.2.5-1: Same as Table 4.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | N/A |  | -94 |  | -114 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | Same as Noc/15kHz |  | Same as Noc/15kHz |  | Same as Noc/15kHz |  |
|  | Config 3,6 | Depending on band group |  | N/A |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -4.97 | 1.88 | -4.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 2 | 6 | 2 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -100 | -104 | -82 | -86 | -111 + ΔBG_offset | -114 + ΔBG_offset |
|  | Config 3,6 | Depending on band group |  | N/A | N/A | -85 | -89 | -108 + ΔBG_offset | -111 + ΔBG_offset |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -70.05 |  | -52.05 |  | -80.20+ ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | N/A |  | -51.77 |  | -73.93 + ΔBG_offset |  |

Table 4.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 3) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 3) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# < Unchanged sections omitted >

### 6.7.1	SS-RSRP

#### 6.7.1.0	Minimum conformance requirements

##### 6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.1.

##### 6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.2.

##### 6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.3.

##### 6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.4.

#### 6.7.1.1	Intra-frequency measurements

##### 6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy

6.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.1.4	Test description

6.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.1.4.1-1.

Table 6.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.1.4.1-2.

Table 6.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 6.7.1.1.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.1.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.1.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.1.1.5	Test requirement

Table 6.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-3 for test configuration 3.

Table 6.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Physical cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 |
| Time offset with Cell 2 |  | Config 1 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -107.5 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | -94 |  | -115 + ΔBG_offset |  |
| Note2 | Config 1,2 |  | dBm/SCS | -107.4 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | -91 |  | -112 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -5.57 | 1.88 | -5.57 | 0.09 | 0.09 |
|  |  |  | dB | 6 | 1.4 | 6 | 1.4 | 3 | 3 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -101.5 | -106.1 | -111 + ΔBG_offset | -111 + ΔBG_offset | -113 + ΔBG_offset | -116.8 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 | N/ANote 6 | -109+ ΔBG_offset | -109+ ΔBG_offset | -110+ ΔBG_offset | -113.8+ ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -80.39+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  | -51.91 |  | -75.12 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 1 is not used when testing with 30kHz SSB SCS. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 36 |
|  |  |  | Bands NR_FDD_FR1_B | 36 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 38 |
|  |  |  | Bands NR_FDD_FR1_G | 39 |
|  |  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  |  | NR_FDD_FR1_B | 48 |
|  |  |  | NR_TDD_FR1_C | 49 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  |  | NR_FDD_FR1_G | 51 |
|  |  |  | NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  |  | Bands NR_FDD_FR1_B | 32 |
|  |  |  | Bands NR_TDD_FR1_C | 32 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 33 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 33 |
|  |  |  | Bands NR_FDD_FR1_G | 34 |
|  |  |  | Bands NR_FDD_FR1_H | 35 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  |  | Bands NR_FDD_FR1_B | 53 |
|  |  |  | Bands NR_TDD_FR1_C | 53 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 54 |
|  |  |  | Bands NR_FDD_FR1_G | 55 |
|  |  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 6.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 38 |
|  |  |  | Bands NR_FDD_FR1_B | 38 |
|  |  |  | Bands NR_TDD_FR1_C | 39 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 39 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 41 |
|  |  |  | Bands NR_FDD_FR1_H | 41 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 50 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 53 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 35 |
|  |  |  | Bands NR_FDD_FR1_G | 36 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 54 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 55 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 56 |
|  |  |  | Bands NR_FDD_FR1_G | 57 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy

6.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.2.4	Test description

6.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.2.4.1-1.

Table 6.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.2.4.1-2.

Table 6.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values of Cell 1 and Cell 2 in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 6.7.1.1.2.5-2 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.2.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.2.4.3	Message contents

Message contents are same as in clause 6.7.1.1.1.4.3.

6.7.1.1.2.5	Test requirement

Table 6.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.2.5-2.

Table 6.7.1.1.2.5-1: Same as Table 6.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | N/A |  | -94 |  | -114+ ΔBG_offset |  |
| Note2 | Config 1,2 |  | dBm/SCS | Same as Noc/15kHz |  | Same as Noc/15kHz |  | Same as Noc/15kHz |  |
|  | Config 3,6 | Depending on band group |  | N/A |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -4.97 | 1.88 | -4.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 2 | 6 | 2 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -100 | -104 | -82 | -86 | -111 + ΔBG_offset | - 114 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/A | N/A | -85 | -89 | -108 + ΔBG_offset | -111 + ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -70.05 |  | -52.05 |  | -80.20+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/A |  | -51.77 |  | -73.93 + ΔBG_offset |  |

Table 6.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# < Unchanged sections omitted >

# 14	NR standalone tests for Satellite access

## 14.0	General

# < Unchanged sections omitted >

### 16.7.1	SS-RSRP for RedCap

# < Unchanged sections omitted >

#### 16.7.1.2	Intra-frequency measurements for RedCap for 2 Rx UE

##### 16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE

16.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.1.4	Test description

16.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.1.4.1-1.

Table 16.7.1.2.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.1.4.1-2.

Table 16.7.1.2.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.1.4.2 with following exceptions:

-	Table 6.7.1.1.1.5-1 is replaced by Table 16.7.1.2.1.5-1.

-	Table 6.7.1.1.1.5-2 is replaced by Table 16.7.1.2.1.5-2.

16.7.1.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.1.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.1.1-1 applies to test configuration 16.7.1.2.1-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.1.2.1-3

16.7.1.2.1.5	Test requirement

Table 16.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-3 for test configuration 3.

Table 16.7.1.2.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1,4 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -107.50 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -115.20 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -107.50 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -112.20 + ΔBG_offset |  |
|  |  |  | dB | 2.23 | -5.57 | 2.23 | -5.57 | 0.37 | -5.56 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -101.50 | -106.10 | -82 | -86.60 | -111.00 + ΔBG_offset | -114.80+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -89.60 | -109.20 + ΔBG_offset | -112.20+ ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -80.39 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.91 |  | -75.13 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 16.7.1.2.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 36 |
|  |  |  | Bands NR_FDD_FR1_B | 36 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 38 |
|  |  |  | Bands NR_FDD_FR1_G | 39 |
|  |  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  |  | NR_FDD_FR1_B | 48 |
|  |  |  | NR_TDD_FR1_C | 49 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  |  | NR_FDD_FR1_G | 51 |
|  |  |  | NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  |  | Bands NR_FDD_FR1_B | 32 |
|  |  |  | Bands NR_TDD_FR1_C | 32 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 33 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 33 |
|  |  |  | Bands NR_FDD_FR1_G | 34 |
|  |  |  | Bands NR_FDD_FR1_H | 35 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  |  | Bands NR_FDD_FR1_B | 53 |
|  |  |  | Bands NR_TDD_FR1_C | 53 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 54 |
|  |  |  | Bands NR_FDD_FR1_G | 55 |
|  |  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 16.7.1.2.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 38 |
|  |  |  | Bands NR_FDD_FR1_B | 39 |
|  |  |  | Bands NR_TDD_FR1_C | 39 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 40 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 41 |
|  |  |  | Bands NR_FDD_FR1_H | 42 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 51 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 54 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 35 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 36 |
|  |  |  | Bands NR_FDD_FR1_G | 37 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 55 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 56 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 57 |
|  |  |  | Bands NR_FDD_FR1_G | 58 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE

16.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.2.4	Test description

16.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.2.4.1-1.

Table 16.7.1.2.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.2.4.1-2.

Table 16.7.1.2.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.1.1.2.5-1 is replaced by Table 16.7.1.2.2.5-1.

-	Table 6.7.1.1.2.5-2 is replaced by Table 16.7.1.2.2.5-2.

16.7.1.2.2.4.3	Message contents

Message contents are same as in clause 16.7.1.2.1.4.3.

16.7.1.2.2.5	Test requirement

Table 16.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.2.2.5-2.

Table 16.7.1.2.2.5-1: Same as Table 16.7.1.2.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | N/A |  | -94 |  | -114+ ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | Same as Noc/15kHz |  | Same as Noc/15kHz |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | N/A |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -4.97 | 1.88 | -4.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -100 | -104 | -82 | -86 | -111.00 + ΔBG_offset | -114.00 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/A | N/A | -85 | -89 | -108.00 + ΔBG_offset | -111.00 + ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -70.05 |  | -52.05 |  | -80.20 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/A |  | -58.05 |  | -80.20 + ΔBG_offset |  |

Table 16.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x – 8 | RSRP_x – 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x – 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x – 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# < Unchanged sections omitted >

### 17.4.1	UE transmit timing for RedCap

#### 17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2

Editor’s Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz.

-	UE PC3.

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

17.4.1.1.1	Test purpose

To verify that the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 7.1A.2.

# < Unchanged sections omitted >

##### 17.5.5.2.1	NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS

Editor’s Note: This test case is incomplete in following aspects:

-	Message contents are still TBD.

TT analysis is available for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

17.5.5.2.1.1	Test purpose

To verify fulfillment of the uplink spatial relation switch delay requirement defined in 38.133 [6] clause 8.12A.5 by a UE capable of beam correspondence without the need for UL beam sweeping.

# < Unchanged sections omitted >

#### 18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used

Editor’s Notes: This test case is incomplete:

-	E-UTRA – NR FR2 testability issue is not cleared.

-	The TT analysis is complete for test frequencies f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz

18.3.1.5.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

# < Unchanged sections omitted >

< End of changes >