3GPP TSG-SA WG6 Meeting #63	S6-244703

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244350, S6-244197)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0687 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Service provisioning information retrieval request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Service provisioning information retrieval request, - The application group profile contains Geo Location and table Table 8.17.3.17-1 also contains location information within (Connectivity information and UE location). It is confusing to decide which location information to be used when all 3 infomration is present. - “AC Profile” IE within “Application information” should be mandatory as specified in Table 8.3.3.3.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added clarification to use “Expected Group Geographical Service Area” when Application group profile is present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will remain unclear and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.3.2, 8.17.3.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e., GPSI) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| List of UE IDs | O | List of UE IDs in an Application Group, applicable for S-EAS or S-EES triggered EAS discovery request. |
| Serving MNO information (NOTE 2) | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| Target DNAI (NOTE 1) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| EAS Instantiation Triggering Suppress | O | Indicates to the EES that EAS instantiation triggering should not be performed for the current request, and Instantiable EAS Information (e.g. instantiated, instantiable but not be instantiated yet) is to be provided in response. |
| EAS selection request indicator | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| Indication of service continuity planning | O | Indicates that this EAS discovery request is triggered for service continuity planning. |
| Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. This IE is used by EES as analytics input to get edge load analytics information from ADAES service as described in clause 8.8 of TS 23.436 [27]. |
| Tunnel information | O | It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server associated with application(s). |
| NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| > Application group profile (NOTE 6) | O | Application group profile associated with the AC Profile, as defined in Table 8.2.11-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > Application Group ID | O | Application group identifier as defined in 7.2.11. |
| > EAS content synchronization support | O | Indicates if the EAS content synchronization support is required or not. |
| > Bundle ID (NOTE 5) | O | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > List of EASIDs (NOTE 5) | O | A list of EASIDs specific to a particular EAS bundle. |
| > Bundle type (NOTE 4) | O | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements (NOTE 4) | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X, UAV, application enabler) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area (NOTE 6) | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area (NOTE 6) | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. NOTE 4: When EAS discovery request is sent by the EEC, this IE shall not be included. NOTE 5: “Bundle ID" and "List of EASIDs" shall not both be present. NOTE 6: If application group profile IE is present, the expected group geographic service area IE present in the application group profile is preferentially used. If there is no EAS satisfied the expected group geographic service area or EAS does not provide better E2E response time, then the UE location and other service area IEs are considered. |  |  |

* * * Next Change * * * *

#### 8.17.3.17	Service provisioning information retrieval request

Table 8.17.3.17-1 describes the information elements for Service provisioning information retrieval request from the ECS to a partner ECS.

Table 8.17.3.17-1: Service provisioning information retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | Endpoint information of ECS (e.g. URI, FQDN, IP address) |
| Security credentials | M | Security credentials of the ECS. |
| Federation information | O | List of information for different federation agreements related to the ECS |
| > Federation identifier | O | Identifier of the federation |
| > ECSP identifiers | O | The list of ECSPs preferred by the ECS. This information is used by the ECS-ER to filter the discovered partner ECS information. |
| Application information | O | List of information about services the EEC wants to connect to, including the option to provide application group profile information. |
| > AC Profile | M | Filter information about required services as described in Table 8.2.2-1. |
| > Application group profile (NOTE) | O | Application group profile associated with the AC Profile, as defined in Table 8.2.11-1. |
| Connectivity information | O | Connectivity information such as serving PLMN information where the services are required. |
| UE location (NOTE) | O | Location of the UE for which the services are required. |
| NOTE: If application group profile IE is present, the expected group geographic service area IE present in the application group profile is preferentially used. If there is no ECS satisfied the expected group geographic service area, then the UE location is considered. |  |  |

* * * End of Change * * * *