3GPP TSG-SA WG6 Meeting #63	S6-244692

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-244611)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0325 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | APIs for location history request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The history location querying function has been approved last meeting. And the procedure and information flows for location tracing configuration request/response, location history request/response also have been approved last meeting. However, the APIs for such two procedures/information flows have not been implemented, so this CR mainly provides corresponding APIs for the two procedures/information flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide corresponding APIs for the history location querying function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The history location querying function will not be realized for vertial location services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1,9.4.x(new), 9.4.x.1(new), 9.4.x.2(new), 9.4.x.3(new), 9.4.x.4(new), 9.4.x.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.4	SEAL APIs for location management

9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_LocationHistoryInfoEvent | Create_Location_Tracing_Configuration | VAL server | Request/Response |
|  | Update_Location_Tracing_Configuration | VAL server |  |
|  | Delete_Location_Tracing_Configuration | VAL server |  |
|  | Obtain_history_Location_Info | VAL server |  |

* * * Next Change * * * *

9.4.x	SS_LocationHistoryInfoEvent API

9.4.x.1	General

API description: This API enables the VAL server to configure a list of location tracing parameters to the SEAL LM server and obtain the UE history location information over LM-S.

9.4.x.2	Create_Location_Tracing_Configuration operation

API operation name: Create_Location_Tracing_Configuration

Description: Creates the location tracing configuration.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.49

Outputs: Refer subclause 9.3.2.50

See subclause 9.3.21.1 for the details of usage of this API operation.

* * * Next Change * * * *

9.4.x.3	Update_Location_Tracing_Configuration operation

API operation name: Update_Location_Tracing_Configuration

Description: Updates the location tracing configuration.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.49

Outputs: Refer subclause 9.3.2.50

See subclause 9.3.21.1 for the details of usage of this API operation.

* * * Next Change * * * *

9.4.x.4	Cancel_Location_Tracing_Configuration operation

API operation name: Cancel_Location_Tracing_Configuration

Description: Cancels the location tracing configuration.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.53

Outputs: Refer subclause 9.3.2.54

See subclause 9.3.21.3 for the details of usage of this API operation.

* * * Next Change * * * *

9.4.x.5	Obtain_history_Location_Info operation

API operation name: Obtain_history_Location_Info

Description: Request UEs history location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.51

Outputs: Refer subclause 9.3.2.52

See subclause 9.3.21.2 for the details of usage of this API operation.

* * * End of Changes * * * *