3GPP TSG-SA WG6 Meeting #62	S6-243681

Maastricht, Netherlands, 19th – 23rd August 2024           (revision of S6-243483,243054)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0191 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Interconnection service API discover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, CAPIF |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Table 8.25.2.3-1, in description of Information element Query information, “service API category” is duplicated. 2. Since there are many criterias in Information element Query information (e.g. service API category, interfaces, protocols, service API category, service KPIs), in table 8.25.2.4-1, the description of Information element CAPIF core function identity information should not just limit to service API category query criteria. 3. 8.25.3.2, “service API category” is no need in procedures. 4. 8.25.3.2, Note add “without detail service API information” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Same as reason. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent for Interconnection API discover information flow and procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.3, 8.25.2.4, 8.25.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

8.25.2.3	Interconnection service API discover request

Table 8.25.2.3-1 describes the information flow interconnection service API discover request from one CAPIF core function to another CAPIF core function.

Table 8.25.2.3-1: Interconnection service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | Identity information of the CAPIF core function discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs or CAPIF core function (e.g. service API category, Serving Area Information (optional), UE IP address (optional), preferred AEF location (optional), required API provider name (optional), interfaces, protocols, Service KPIs (optional)) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

8.25.2.4	Interconnection service API discover response

Table 8.25.2.4-1 describes the information flow interconnection service API discover response from one CAPIF core function to another CAPIF core function.

Table 8.25.2.4-1: Interconnection service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information | O (see NOTE) | List of service APIs corresponding to the request, including service API information as specified in Table 8.25.2.1-1. |
| CAPIF core function identity information | O (see NOTE) | Indicates the CAPIF core function matching the query criteria |
| NOTE: The service API information or the CAPIF core function identity information or both shall be present, if the Result information element indicates that the interconnection service API discover operation is successful. Otherwise, both shall not be present. |  |  |

8.25.3.2	Service API discovery involving multiple CCFs

This subclause describes a procedure for service API discovery involving multiple CCFs

Pre-condition:

1.	CCF-A and CCF-B connect to each other, and either belong to the single trust domain of the same CAPIF provider or trust domains of different CAPIF providers.

2.	When CCF-A and CCF-B belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing.

Figure 8.25.3.2-1: Service API discovery involving multiple CCFs

1.	The API invoker sends a service API discover request to the CCF-A. It includes the API invoker identity and query information.

2.	The CCF-A verifies the identity of the API invoker and retrieves the stored service API(s) information. The information of CCF-B matching the discovery criteria is returned to API invoker in the service API discover response.

NOTE:	The remaining steps are only applied when the service API category without detail service API information is included in the interconnection API publish request as described in subclause 8.25.2.1.

3.	The API invoker sends an service API discover request to the CCF-B. The identity of API invoker is included. The query information is also provided.

4.	Upon receiving the service API discover request, the CCF-B verifies the identity of the API invoker. The CCF-B retrieves the stored service API(s) information as per the query information in the service API discover request. Further, the CCF-B applies the discovery policy and performs filtering of service APIs information which matches the discovery criteria.

5.	The CCF-B sends an service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.