3GPP TSG SA WG2#164	            S2-2408704

Maastricht, 19-23 August 2024 	(revision of S2-2408573)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4986 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on AF request related to Edge Computing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If PDU session is subject to HR-SBO, V-SMF subscription to V-NEF is needed (step 0b in clause 4.3.6.5.2). However, in current description given in 4.3.6.1, this subscription is omitted. The procedure defined in clause 4.3.6.5.2 is for AF request including HPLMN DNN, HPLMN S-NSSAI. As part of this procedure, the request can target an individual UE or any UE. However, the current wording reads that the procedure is only for the case of “any UE”. In order to capture the case of “individual UE”, step 1 should also mention clause 4.3.6.4 where AF request targeting an individual UE is defined. Clause 5.2.6.38.1 should be removed as it is no more a NEF but a SMF service that is being used as shown in clause 6.2.3.2.7 of TS 23.548. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting procedure defined for the case if PDU session is subject to HR-SBO. - Adding a note to step 2 of Figure 4.3.6.5.2 to clarify “individual UE” case for AF request. - Removal of clause 5.2.6.38.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current descriptions are not fully correct or misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1, 4.3.6.5.2, 5.2.6.38 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.3.6.1	General

Clause 4.3.6 describes the procedures between an Application Function and the SMF to maintain an efficient user plane path and/or to provide N6-LAN service function chaining for Application Functions that require it.

As described in clauses 5.6.7 and 5.6.16 of TS 23.501 [2], an Application Function may send requests to influence SMF routeing decisions for User Plane traffic of PDU Sessions. The AF requests may influence UPF (re)selection and allow routeing of user traffic to a local access (identified by a DNAI) to a Data Network and/or influence the steering of user traffic to service function chain(s) identified by SFC identifier(s). The AF may also provide in its request subscriptions to SMF events. A V-SMF supporting HR-SBO provides, SUPI, an indication of support for HR-SBO, and HPLMN DNN and S-NSSAI related to the PDU session to the L-PSA UPF at PDU session establishment.

NOTE 1:	NEF uses the SUPI to derive PLMN ID of the UE.

The following cases can be distinguished:

NOTE 2:	Such requests target an on-going PDU Session.

When receiving an AF request on Application Function influence on traffic routing targeting an individual UE IP address the NEF needs to determine whether the target PDU Session is working in HR-SBO mode. If the target PDU Session is NOT working in HR-SBO mode (non roaming or LBO PDU Session) then the NEF contacts the PCF of the PDU Session as further defined in the clause 4.3.6.4. If the target PDU Session is working in HR-SBO mode, the NEF does not contact the PCF of the PDU session but needs to store the AF request in UDR as defined in clause 4.3.6.5.

The NEF determines whether the PDU Session that the AF requests to influence is working in HR-SBO mode or not and in the former case determines the HPLMN of the UE, the DNN and S-NSSAI of the PDU Session as follows:

If the AF has determined based on procedure in clause 4.3.6.5.2 that the PDU session is home routed, the AF provides IP address of the UE (assigned by HPLMN) and HPLMN ID, HPLMN DNN and S-NSSAI, and the NEF determines if the PDU session is subject to HR-SBO using the provided information and proceeds according to clause 4.3.6.5.2.

If the AF has provided an IP address of the UE known to the AF (IP address not assigned by HPLMN), and DNN and S-NSSAI, the NEF determines if the PDU session is subject to HR-SBO based on local configuration. If PDU session is not subject to HR-SBO, the NEF proceeds according to clause 4.3.6.4. If PDU session is subject to HR-SBO the NEF determines the HPLMN and proceeds from according to clause 4.3.6.5.2, excluding step 0a.

NOTE 3:	The DNN can be unique for a PLMN (see TS 23.003 [33]) and can be mapped to a HPLMN, or the IP address can be within a range that can be mapped to a HPLMN.

If the AF has not provided the (H)PLMN ID, DNN and S-NSSAI:

-	If the IP address of the UE in the AF request is a private IP address, the NEF determines if the request relates to HR-SBO or not based on local configuration, or based on interactions with UPF, since the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. If the request relates to HR-SBO, the NEF proceeds according to clause 4.3.6.5.3, otherwise it proceeds according to clause 4.3.6.4.

NOTE 4:	Whether or not NEF interact with UPF to determine if HR-SBO applies is based on configuration. If NEF does not interact with UPF, the 5GC NF (e.g. SMF/UPF, NEF) in PLMN can be configured to have separate private IP address ranges for HR-SBO PDU Sessions. This would allow NEF to distinguish AF request for influence on traffic routing for HR-SBO PDU Sessions from non-roaming and LBO PDU Sessions.

-	If the IP address of the UE in the AF request is a public IP address:

-	If this Public IP address belongs to an IP range not owned by the PLMN of the NEF, then the target PDU Session is working in HR-SBO mode and the NEF proceeds according to clause 4.3.6.5.4.

-	If the UE IP Address in the AF request is an IP address NATed by the PLMN that the NEF belongs to, the NEF invokes steps 3 to 6 of clause 4.15.10 to get the corresponding private UE IP address. If HR-SBO applies, the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. Thus, the NEF can also receive these parameters and determines if the AF request relates to HR-SBO PDU Session, the NEF proceeds according to clause 4.3.6.5.3. Otherwise the NEF proceeds according to clause 4.3.6.4.

NOTE 5:	In this Release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

NOTE 6:	It is assumed that the NEF is configured with the NATed IP range of its own PLMN. It is assumed that the NEF is configured based on HR-SBO roaming agreements for the Public IP address ranges with an HPLMN ID.

NOTE 7:	Whether the AF needs to use the NEF or not is according to local deployment. If the AF request is expected to possibly address PDU Sessions in HR-SBO mode, then the AF sends its requests via the NEF.

When AF requests are routed (by the AF or by the NEF) to an individual PCF this may use the BSF. This case applies to both AF influence on traffic routing as well as AF influence on Service Function Chaining. This is described in clause 4.3.6.4.

-	AF requests described in clause 5.6.7 of TS 23.501 [2] or clause 5.6.16 of TS 23.501 [2] targeting a group of UE(s), or any UE accessing a combination of DNN and S-NSSAI and optionally PLMN ID of the DNN and S-NSSAI, or targeting individual UE(s) by one or more GPSI(s) or targeting UEs with External Subscriber Category(s) which can be combined with External Group ID(s) or any UE as described in table 5.6.7-1. These AF requests may also affect UE(s) with an established PDU session. For such requests the AF shall contact the NEF and the NEF stores the AF request information in the UDR. For non-roaming and LBO cases, the PCF(s) receive a corresponding notification if they had subscribed to the create/update/delete operations of the AF request information corresponding to UDR Data Keys / Data Sub-Keys. This is defined in clause 6.3.7.2 of TS 23.501 [2] and further described in clause 4.3.6.2. For HR-SBO case, V-SMF(s) receive a corresponding notification if it has subscribed to the create/update/delete operation of the AF request information corresponding to UDR Data Keys /Data Sub-Keys, this is described in clause 4.3.6.5. For a single UE, NEF determines if HR-SBO applies for the PDU session based on the local configuration using the received GPSI. For "any UE", both PCF(s) and SMF(s) (via NEF) may need to subscribe to creates/updates/deletes of AF influence on traffic routing data in UDR depending on if a specific PLMN is targeted.

NOTE 8:	Such requests can target on-going or future PDU Sessions.

If the AF request targets any UE the procedure described in clause 4.3.6.5 may also be performed. In that case if HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have been provided (see clause 4.3.6.5.2), if the HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have not been provided by the AF, the NEF determines the HPLMN DNN and HPLMN S-NSSAI as described in clause 4.3.6.5.5.

This Release of the specification does not support AF influence on traffic routing for HR-SBO PDU session, if the AF request targets an external group ID, or UE(s) identified by SUPI.

If the AF interacts with 5GC via the NEF, the NEF performs the following mappings or determinations where needed:

-	Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

-	Map the AF-Service-Identifier into a list of DNAI(s) and Routing Profile ID(s) determined by local configuration.

The NEF can only provide this mapping when the DNAI(s) being used by the applications are statically defined. When the DNAI(s) where applications are instantiated may vary dynamically, the AF should provide the target DNAI(s) in its request together with either Routing Profile ID(s) or with N6 traffic routing information.

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into Internal Group Identifier, according to information received from UDM.

-	Map the External Subscriber Category(s) and any UE, or External Subscriber Category and External Group ID(s) to, Internal Group ID(s) or Internal Group ID(s) and Subscriber Category(s).

-	Map the geographical area in Spatial Validity Condition into areas of validity, determined by local configuration.

-	Determine whether an AF request targeting an UE IP address corresponds to HR-SBO and if yes determine the HPLMN of UE, the DNN/S-NSSAI of the PDU Session based on the description given above.

NOTE 9:	As a user can be associated with multiple Subscriber Category(s), some values of Subscriber Category(s) can correspond to an SLA between an application provider represented by an AF and the 5GC operator. In the NEF API, the combination of application identifier and External Subscriber Category can also be used to refer to this SLA.

Second CHANGE

##### 4.3.6.5.2	AF traffic influence request includes HPLMN DNN, HPLMN S-NSSAI

Figure 4.3.6.5.2-1: Processing an AF request to influence traffic routing for HR-SBO session when AF provides HPLMN DNN and HPLMN S-NSSAI

0a.	AF subscribes to H-SMF regarding the change of serving PLMN change event and is notified after V-SMF insertion/inter-PLMN V-SMF change as described in clause 6.7.2.6 of TS 23.548 [74]. The notification takes place as soon as the H-SMF has received an indication of Handover Complete.

0b.	V-SMF supporting HR-SBO subscribes to notification of AF request by invoking Nnef_TrafficInfluenceData_Subscribe service from V-NEF (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and HPLMN DNN and HPLMN ID and UE IP address) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and HPLMN DNN and HPLMN ID and "any UE" indication) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and HPLMN DNN and SUPI).

NOTE:	Using SUPI as a Data Key is for the case when AF request targeting GPSI.

-	For any UE, the V-SMF supporting HR-SBO performs the above subscription using HPLMN ID, S-NSSAI and DNN and "any UE" indication as Data Key.

-	For individual UE, when a PDU Session is authorized for HR-SBO as described in clause 6.7 of TS 23.548 [74], the V-SMF serving the PDU Session performs the above subscription using HPLMN ID, HPLMN S-NSSAI and DNN and UE IP address; and HPLMN S-NSSAI and HPLMN DNN and SUPI as Data Key.

V-NEF subscribes to notification of AF request by invoking Nudr_DM_Subscribe service from V-UDR using the same Data Set and Data Key as V-SMF.

1.	As it is step 1 of Figure 4.3.6.2-1.

2.	The AF sends its traffic influence request to V-NEF with HPLMN DNN and S-NSSAI of the UE. The AF request also includes HPLMN ID and UE IP address, or GPSI, or HPLMN ID and "any UE" indication as described in clause 4.3.6.1.

NOTE: To target an individual UE address, step 1 of Figure 4.3.6.4-1 is used with the above details.

3.	Based on the provided HPLMN ID, HPLMN DNN and S-NSSAI as part of traffic influence request, V-NEF identifies that the AF request is targeting a HR-SBO PDU session via VPLMN as described in clause 4.3.6.1.

4a.	V-NEF creates/updates/delete the information at V-UDR as follows:

-	(in the case of Nnef_TrafficInfluence_Create or Update): The V-NEF stores the AF request information in the V-UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID, HPLMN ID (not used if SUPI is used), HPLMN S-NSSAI and DNN and either, SUPI or UE IP address or "any UE" indication). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing, as described in Table 5.2.12.2.1-1.

-	(in the case of Nnef_TrafficInfluence_delete): The V-NEF deletes the AF requirements in the V-UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing.

4b.	The V-NEF responds to the AF.

5.	The V-UDR notifies the subscribed V-NEF of the AF traffic influence request information.

6.	The V-NEF notifies the subscribed V-SMF of the AF traffic influence request information.

7.	As similar to step 6 of Figure 4.3.6.2-1, when V-SMF receives notification from V-NEF, the V-SMF may take appropriate actions to reconfigure the User plane of the PDU Session; and as step 7 of Figure 4.3.6.2-1, V-SMF sends Nsmf_PDUSession_SMContextStatusNotify to AMF.

Third CHANGE

#### 5.2.6.38	Void

End of CHANGES