TSG SA Meeting SA#106	SP-241953

10 - 13 December, 2024, Madrid, Spain                                                   (was SP-241860)

# Title:				pCR on TR 22.850 - updates related to SA6 AIML_App

# Source:				Lenovo, Ericsson, KPN, TNO

# Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL/ Rel-19

# 1. Introduction

This paper provides updates related to activities of AIML_App in SA6. In particular:

- In clause 5.2.1.3, the AI/ML related definitions are updated based on the agreed TS 23.482

- In 5.2.2.13 and 5.2.2.14 the activity summary for the AIML_App SID and WID with respect to ML lifecycle was added.

- In 6.2 there were minor updates referring to the agreed TS 23.482 instead of TR 23.700-82.

# 2. Proposal

Incorporate the following change in TR 22.850.

*** First Change ***

#### 5.2.1.3	TSG SA WG6

The following definitions are provided in clause 3 of TS 23.482 [34]:

-	ML model: According to TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

-	ML model lifecycle: The lifecycle of an ML model aka ML model operational workflow consists of a sequence of ML operations for a given ML task / job (such job can be an analytics task or a VAL automation task). This definition is aligned with the 3GPP definition on ML model lifecycle according to 3GPP TS 28.105 [9].

-	ML model training: According to TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

-	ML model inference: According to TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

-	AI/ML intermediate model: For federated learning, members need to train models for multiple rounds, intermediate models indicate the model which do not meet the required training rounds and/or meet the requirements of the federated training.

-	AIMLE service: An AIMLE service is an AIMLE capability which aims assisting in performing or enabling one or more AIML operations.

-	AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g. to provide ML model training and inference locally e.g. at the VAL UE side.

-	AIMLE client set identifier: an identifier of the set of selected AIMLE clients.

-	AI/ML server: an application layer entity which is an AI/ML endpoint and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enabler server.

-	FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process.

-	FL client: An FL member which locally trains the ML model as requested by the FL server. Such FL client functionality can be at the network (e.g. AIMLE server with FL client capability) or at the device side (e.g. AIMLE client with FL client capability).

-	FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

-	Split AI/ML operation pipeline: A Split AI/ML operation pipeline is a workflow for ML model inference in which AI/ML endpoints are organized and collaborate to process ML models in sequential stages, where processing at each stage involves ML model inference on the output of the previous stage.

*** Second Change ***

#### 5.2.2.13	Rel-19 SA WG6 SID - Application layer support for AI/ML services (FS_AIMLAPP)

##### 5.2.2.13.1	Description

The objective of this study is to enable support for AI/ML services at the application enablement layer. This includes the following:

-	Analysis of Rel-18 and Rel-19 Requirements: Analyse the requirements in [6] related to AI/ML model distribution, transfer, and training. Identify key issues and develop corresponding architectural requirements at the application enablement layer, along with potential enhancements to the application layer architecture.

-	Architectural and Functional Implications: Study the architectural and functional implications on existing SA WG6 application enablers (e.g. ADAES, other SEAL services, EDGEAPP) for supporting AI/ML lifecycle operations. This includes operations such as data collection, data preparation, training, inference, and federated learning for ML models used in ADAE layer analytics.

-	Potential Solutions and APIs: Identify potential solutions, including information flows and developer-friendly application enablement APIs, to satisfy the architectural requirements and enhancements identified in the previous points.

-	Impact on Deployments and Business Models: Investigate the possible impacts of application layer support for AI/ML services on different deployments and business models.

##### 5.2.2.13.2	Activities summary

#### In this study, 3GPP TR 23.700-82 [7] described the AI/ML enablement capabilities for supporting vertical use cases. The agreed AIMLE activities which were progressed in normative phase are described in clause 5.2.2.14.2.

#### 5.2.2.14	Rel-19 SA WG6 WID - Application enablement for AI/ML services (AIML_App)

##### 5.2.2.14.1	Description

The objectives of this work include the following:

Develop Stage 2 normative technical specification for AIML enablement service as a new SEAL service, based on the key issues, architecture, solutions, and conclusions captured in [7]. The Stage 2 normative technical specification will include the following aspects:

-	Architecture requirements, deployment models and application architecture for AIML service enablement over 3GPP networks.

-	Procedures, information flows and APIs supporting concluded solutions related to AIML enablement capabilities for AI/ML, FL (e.g. Vertical FL among VAL UEs, Horizontal FL), Transfer Learning. Such capabilities include:

-	Support AIML client management (e.g. registration, discovery) and selection.

-	Support AIML service lifecycle management aspects (e.g. training, inference, data management).

-	Support AIML operation split and ML model distribution operations.

-	Support AIML operations in edge / distributed deployments.

-	Procedures information flows and APIs supporting concluded solutions for application layer support capabilities related to new ADAE analytics services. Such new analytics services include:

-	DN Energy Analytics.

-	Analytics for supporting FL.

Identify potential enhancements to other enablement frameworks (e.g. SEAL, EGDEAPP and CAPIF) based on the specified solutions for the above objectives.

##### 5.2.2.14.2	Activities summary

In AIML_App, the following logical entities have been introduced within SEAL frmaework:

- AIMLE server includes of a common set of services for exposure of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIMLE services are offered to the vertical application layer (VAL) and include:

  - Support for application-layer ML model related aspects, including model retrieval, model training, model monitoring, model selection, model distribution, model update and model storage / discovery.

  - Assistance in AI/ML task transfer and split AI/ML operations.

  - Support HFL/VFL operations, including FL member registration, FL grouping and FL-related events notification, VFL feature alignment, HFL training, ML model training capability evaluation for FL (HFL/VFL).

  - Support for AIMLE client registration, discovery, participation and selection.

- AIMLE client functional entity acts as the application client supporting AIMLE services.

- ML repository is a logical entity that serves both as a registry for AI/ML members or FL members and as a repository for application layer ML model related information. It can be accessed by the AIMLE server.

###### 5.2.2.14.2.1	Model Lifecycle enablement for AI/ML

Some AIMLE capabilities are applicable to ML model lifecycle enablement which provides assistance for use cases where an ASP/VAL layer wants to find and use other application entities to perform some ML operations (e.g. ML model inference) and AIMLE server as a mediator to accomplish that.

An example including some capabilities is illustrated in Figure 5.2.2.14.2.3-1 as depicted also in Annex C.4 of [34]. In this figure, the support capabilities are based on AIMLE capabilities identified in this specification. In particular, AIMLE is undertaking:

- ML model related support capabilities such as model retrieval, discovery and storage (as covered in procedures in clauses 8.2 and 8.11 of [34] )

- ML operation related support capabilities such as VFL/ HFL and TL enablement, Split AI/ML Operation support, Data management assistance, AI/ML task transfer, FL assistance in member grouping, registration and event notification (as covered in procedures in clauses 8.4, 8.6, 8.12, 8.14, 8.15-8.18 of [34]).

- AIMLE client related support capabilities, including AIMLE client registration, discovery, participation, monitoring, selection (as covered in procedures in clauses 8.7-8.10, 8.13 of [34]).

Figure 5.2.2.14.2.1-1: ML model Lifecycle Enablement

###### 5.2.2.14.2.2	Data Collection/Storage/Exposure activites.

Analysis of data collection activities as part of AIML_App work.

Data Collection in TS 23.482 refers to application data collection from the UE. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531.

ML model performance degradation can be detected in the AI/ML enablement by leveraging ADAES, e.g., based on information collected from analytics consumer. In AIML_App, one possible use of AI/ML enablement is for supporting ML-enabled ADAES analytics services (as specified in TS 23.436). For Data Collection and Storage related to ADAES analytics:

- Application layer - Data Collection and Coordination Function (A-DCCF) coordinates the collection and distribution of data requested by the consumer (ADAE server). Data Collection Coordination is supported by a A-DCCF. ADAE server can send requests for data to the A-DCCF rather than directly to the Data Sources. A-DCCF may also perform data processing/abstraction and data preparation based on the VAL server requirements.

- Application layer – Analytics and Data Repository Function (A-ADRF) stores historical data and/or analytics, i.e., data and/or analytics related to past time period that has been obtained by the consumer (e.g. ADAE server). After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an A-ADRF. Whether the consumer directly contacts the A-ADRF or goes via the A-DCCF is based on configuration.

###### 5.2.2.14.2.3	ML-related information storage and discovery for AI/ML

Analysis of ML model storage and exposure activities as part of AIML_App work.

In AIML_App, ML repository has been defined as

- a registry for AI/ML members or FL members (application layer entities participating in an AI/ML operation) and

- as a repository for application layer ML model related information.

AIMLE server stores the ML model to the ML repository along with the ML model information (e.g. ML model ID). AIMLE server can also discover the ML models under certain filtering criteria (e.g. applicable to an ADAES analytics ID).

AIMLE server also registers and stores information on VAL servers, AIMLE servers or AIMLE clients which are expected to serve as AI/ML members or FL members in a model lifecycle operation (e.g. ML training, FL, TL). AIMLE clients or other VAL servers can discover the availability and capabilities of registered AI/ML members or FL members for a given ML model ID. Such discovery allows e.g., the VAL server identifying the candidate FL members to be considered for an FL process.

###### 5.2.2.14.2.4	Model training/delivery/ (de)-activation/inference emulation activities

In AIML_App, AIMLE server or the AIMLE client (at VAL UE side) can also be used for training an application layer ML model e.g. for given analytics service. Such ML model training can be used to support ADAES analytics services (as provided in TS 23.436). Based on the VAL request to provide ML-enabled analytics, ADAES may consume AIMLE services (e.g., for ML model training for a given analytics ID) to derive application layer data analytics.

The trained ML model can be delivered to VAL server or ADAES via the ML model training notification API.

3GPP SA6 has not defined any procedures for model (de)-activation and inference emulation.

###### 5.2.2.14.2.5	ML model inference and delivery support for AI/ML

Analysis of ML model inference activities as part of AIML_App work.

3GPP SA6 has not defined dedicated procedures for supporting ML model inference; however, it provides assistance for registering and discovering AIMLE clients serving as ML model inference entities for a given analytics ID or model ID or split operation pipeline.

###### 5.2.2.14.2.6	Performance evaluation and accuracy monitoring activities

Analysis of ML model performance evaluation and monitoring activities as part of AIML_App work.

AIMLE server based on VAL request provides a capability for monitoring and detecting a degradation related to an ML operation / analytics operation and translating to an ML model performance degradation (expected or predicted) and performing a trigger action to alleviate this issue (new model training or re-training). Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model.

AIML_App has provided the basic capability for performance monitoring activity, which is expected to be further worked in further release.

*** Third Change ***

## 6.2	AI/ML related terminology

### 6.2.1	Analysis on terminology consistency

This clause identifies any potential misalignments and inconsistencies for AI/ML terminology across 3GPP, based on clause 5.

#### 6.2.1.1	Analysis on ML model

The term 'ML model' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.1-1.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.1-1: Definition of ML model as defined across 3GPP WGs

| TSG (TS/TR) | ML model |
| --- | --- |
| SA WG5 TS 28.105 [9] | A manageable representation of an ML model algorithm. NOTE 1: An ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output. NOTE 2: An ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification. NOTE 3: An ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information. |
| RAN WG1 TR 38.843 [3] | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |

#### 6.2.1.2	Analysis on ML model training

The term 'ML model training' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.2-1. RAN WG3 follows the definition of SA WG5.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.2-1: Definition of ML model training as defined across 3GPP WGs

| TSG (TS/TR) | ML model training |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. |
| RAN WG1 TR 38.843 [3] | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |

#### 6.2.1.3	Analysis on ML model re-training

The term 'ML model re-training' has been defined differently by SA WG5 and RAN WG1, as illustrated in Table 6.2.1.3-1. RAN WG1 introduces two new terms, i.e. ML model parameter update and ML model update, which is nothing but ML model re-training.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.3-1: Definition of ML model re-training / ML model parameter update as defined across 3GPP WGs

| TSG (TS/TR) | ML model re-training / ML model parameter update / ML model update |
| --- | --- |
| SA WG5 TS 28.105 [9] | ML model re-training: A process of training a previous version of an ML model and generate a new version. |
| RAN WG1 TR 38.843 [3] | ML model parameter update: A process of updating the model parameters of a model. Model update: A process of updating the model parameters and/or model structure of a model |
| SA WG6 TS 23.482 [34] | ML model update: A process of training a new version of a ML model and updating its parameters. |

#### 6.2.1.4	Analysis on ML model testing

The term 'ML model testing' has been defined differently by SA WG5 and RAN WG1, as illustrated in Table 6.2.1.4-1.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.4-1: Definition of ML model testing as defined across 3GPP WGs

| TSG (TS/TR) | ML model testing |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process of testing an ML model using testing data. |
| RAN WG1 TR 38.843 [3] | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |

#### 6.2.1.5	Analysis on ML model inference

The term 'ML model inference' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.5-1. RAN WG3 follows the definition of SA WG5.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.5-1: Definition of ML model inference as defined across 3GPP WGs

| TSG (TS/TR) | ML model inference |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process of running a set of input data through a trained ML model to produce set of output data, such as predictions. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference. |
| RAN WG1 TR 38.843 [3] | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |

#### 6.2.1.6	Analysis on ML model activation & ML model de-activation

The term 'ML model activation' and 'ML model deactivation' have been defined by RAN WG1, as illustrated in Table 6.2.1.6-1. SA WG5 mentions the terms ML activation and ML deactivation several times in TS 28.105 [9] but does not provide a definition.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.6-1: Definition of ML model inference as defined across 3GPP WGs

| TSG (TS/TR) | ML model inference |
| --- | --- |
| RAN WG1 TR 38.843 [3] | ML Model activation: enable an AI/ML model for a specific AI/ML-enabled feature. ML Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature. |

#### 6.2.1.7	Analysis on ML model lifecycle

The term 'ML model lifecycle' has been defined by SA WG6, as illustrated in Table 6.2.1.7-1. However, SA WG2 TS 23.288 [8] SA WG2 TR 23.700-84 [4], SA WG4 TR 26.927 [12], SA WG5 TS 28.105 [9], SA WG6 TR 23.700-82 [7], RAN WG1 TR 38.843 [3] and RAN WG3 also mentions one or more phases of ML model life cycle without providing a clear definition of ML model lifecycle.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.1.7-1: Definition of ML model lifecycle as defined across 3GPP WGs

| TSG (TS/TR) | ML model lifecycle |
| --- | --- |
| SA WG6 TS 23.482 [34] | The lifecycle of an ML model aka ML model operational workflow consists of a sequence of ML operations for a given ML task / job (such job can be an analytics task or a VAL automation task). This definition is aligned with the 3GPP definition on ML model lifecycle according to 3GPP TS 28.105 [9]. |

#### 6.2.1.8	Analysis on ML model lifecycle management

SA WG5 describes the ML model lifecycle in clause 4a.0 of TS 28.105 [9], and ML model lifecycle management capabilities for ML model training, ML model testing, ML inference emulation, ML model deployment and ML inference in clause 6.1 of TS 28.105 [9].

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.