3GPP TSG-SA3 Meeting #119 	S3-245051-r1

- Orlando, USA, 11 - 15 November 2024

- Source:	IIT Bombay

- Title:	New solution for security aspects on user consent for energy information exposure/retrieval

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to approve this pCR.

# 2	References

[1]	3GPP TR 33.766 Study on security aspects of energy savings in 5G

# 3	Rationale

It is proposed to approve the new solution for Key Issue 2 of TR 33.766 [1] related to security aspects on user consent for energy information exposure/retrieval.

# 4	Detailed proposal

*** Start of Change ***

### 6.Y	Solution #Y: User consent check for energy information exposure/retrieval

### 6.Y.1	Introduction

The solution addresses Key Issue 2 (i.e., to confirm user consent for energy information exposure/retrieval). Generic security requirements and procedures to support user consent enforcement are provided in Annex V of TS 33.501.

### 6.Y.2	Solution details

#### 6.Y.2.1 General

This solution is based on the following conclusion for KI#1 clause 8.1 in TR 23.700-66:

“A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator's policy:

-	If the authorized consumer is AF, the granularities include: per UE, per UE per application, per PDU session.

-	If the authorized consumer is 5GC NFs, the granularities include: per application, per UE, per-UE-per-QoS flow, per PDU session.”

In the context of the above conclusion, this solution confirms user consent for user-specific energy-related information exposure/retrieval.

Note:	This solution applies when the regional regulations require a legal basis to process.

#### Editor’s note: Clarifications on the new impact/aspects of this solution with respect to the framework of Annex V in TS 33.501 [3] are FFS.

#### 6.Y.2.2 User consent check procedure for energy information exposure/retrieval

gNB

AMF

SMF

PCF

UDM

EIF

NEF

AF

1. Energy_exposure subscription request

2.Energy_exposure subscription request

3.Energy_exposure subscription request

(Requesting user consent)

4.Energy_exposure subscription response

(Provisioning user consent parameters)

5.Energy_exposure subscription response

6.Energy_exposure subscription response

Energy information collection

Receiving information from different NFs

Energy related information

7.Energy exposure

8.Energy exposure

Figure 6.Y.2-1: User consent check procedure for energy information exposure/retrieval

A subscription (for energy-related information exposure) and authorization procedure for an AF are the same as provided in the existing Solution # 3 in [1]. It is based on token-based authorization, in which the token claim contains all relevant parameters, such as the application identifier, NSSAI, UE identifier, and service identifier. However, the highlight of this solution is to confirm user consent for energy information exposure to AF.

- The AF sends “Energy_exposure subscription request” to NEF which contains token with token claim parameters (as detailed with Solution # 3 [1]). This subscription is for receiving energy information at granularity including user level energy information.

- The NEF authorizes the AF for Energy_exposure subscription and forwards “Energy_exposure subscription request” to EIF upon successful authorization for this subscription.

- Before sending “Energy_exposure response” to the NEF, the EIF sends this request to UDM for user consent check.

- The EIF receives user consent parameters from UDM on the provisioning energy information exposure to the AF.

- Upon receiving user consent confirmation, the EIF responds to the NEF.

- The NEF sends “Energy_exposure subscription response” to the AF.

- The EIF collects energy information from other network functions and reports it to NEF.

- The NEF sends this energy information to the AF as per the requirements of Energy_exposure service.

NOTE: Energy information exposure update can be periodic based on service level agreements with user consent.

### 6.Y.3	Evaluation

The solution relies on the UDM storing user consent parameters related to energy information while considering the existing security requirements and procedures to support user consent enforcement based on Annex V of TS 33.501 [3].

Note: Requirements and procedure related to user consent for energy information in this solution are applicable only when it is required by regional regulations or operator’s local policy, not otherwise.

*** End of Change ***