3GPP TSG-SA3 Meeting #119	S3-244681

Orlando, US, 11 - 15 November 2024

# Source:	Cisco Systems

# Title:	Conclusion for KI#7: Supporting all 5G SBA certificate types

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval of proposed conclusion for KI#7 of TR 33.776

# 2	References

[1]	3GPP TR 33.776 (V0.5.0), "Study of Automated Certificate Management Environment (ACME) for the Service Based Architecture (SBA)".

[2]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF) ".

# 3	Rationale

This contribution proposes a conclusion for KI#7 in TR 33.776 [1].

# 4	Detailed proposal

* * * Start of change - all new text * * *

## 7.X		Key issue #7: Supporting all 5G SBA certificate types

### 7.X.1		Analysis

This key issue identifies the need to support all 5G SBA certificate profiles.

Solution #8 addresses this key issue. TS 33.310, clause 6.1.3c [3], defines the certificate profiles for 5GC SBA. X.509 version 3 certificates are used for all entities in 5GC SBA. ACME supports X.509 version 3 certificates and the necessary extensions.

### 7.X.2		Conclusion

The normative phase can begin based on Solution #8.

* * * End of change * * * *